3GPP TSG-RAN WG4 Meeting #107	R4-2308897

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on SAN demodulation requirements for LTE NTN IOT

Source: 	Huawei, HiSilicon

Agenda item:	9.7.7.2

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] on IoT NTN SAN demodulation requirements was approved. In this contribution, we share our views about NB-IoT/eMTC SAN demodulation performance requirements for NTN.

Discussion

Channel model

Way forward: Updated simulation assumptions for PUSCH CE mode A (for alignment purpose) - Assume TxDuration is same as the number of repetitions Parameters; CE Mode A. Max number of HARQ transmissions; 4. RV sequences; 0, 2, 3, 1, 0, 2, 3, 1. Number of PUSCH repetitions; 8. Frequency hopping; OFF. Duplex mode; FDD only. Number of Tx antennas; 1. Number of Rx antennas; 1 and 2. Propagation channel; Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5. FRC; TS36.104 A3-2. System BW; 1.4MHz only. Fraction of maximum throughput; 70%. Way forward: Simulation assumption for PUSCH CE mode B (for alignment purpose) - Assume TxDuration is same as the number of repetitions Parameters; CE Mode B. Max number of HARQ transmissions; 2. RV sequences; 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1. Number of PUSCH repetitions; 256. Frequency hopping; OFF. Duplex mode; FDD only. Number of Tx antennas; 1. Number of Rx antennas; 1 and 2. Propagation channel; Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5. FRC; TS36.104 A3-1. System BW; 1.4MHz only. Fraction of maximum throughput; 70%. Way forward: Simulation assumptions for NPUSCH format 1 requirements (for alignment purpose) Number of TX antennas; Number of RX antennas; Subcarrier spacing; Number of allocated subcarriers; Propagation conditions; FRC; Repetition number; Fraction of maximum throughput. 1; 1 and 2; 3.75KHz; 1; Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1; TS36.104 A16-1; 4; 70%. 1; 1 and 2; 15KHz; 12; Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1; TS36.104 A16-5; 16; 70%.

For the channel model, both LoS and NLoS propagation condition can be observed in the real NTN scenario, so we prefer to consider both NTN-TDLA and NTN-TDLC for PUSCH CE mode A, PUSCH CE mode B and NPUSCH format 1 requirements.

Consider both NTN-TDLA and NTN-TDLC for PUSCH CE mode A, PUSCH CE mode B and NPUSCH format 1 requirements.

Proposals

In this contribution, we discuss on NB-IoT/eMTC SAN demodulation performance requirements for NTN. Our observations and proposals are:

- Consider both NTN-TDLA and NTN-TDLC for PUSCH CE mode A, PUSCH CE mode B and NPUSCH format 1 requirements.

Reference

R4-2305965, WF on IoT NTN SAN demodulation requirements, RAN4#106-bis-e, Ericsson