3GPP TSG-RAN WG4 Meeting #107	R4-2308921

Incheon, Republic of Korea, 22-26 May, 2023

Source:	Ericsson

Title:	On timing requirements for MIMO

Agenda item:	8.30.3.2

Document for:	Approval

# 1	Introduction

In WF [1] topic #2 lists Timing requirements for UL multi-DCI multi-TRP with two Tas agreements and open issues:

… # Topic#2 Timing requirements for UL multi-DCI multi-TRP with two TAs Issue 2-1-1: What is the assumption on M1/M2 for MTTD for UE not capable of supporting RTD>CP? - Proposals: - MTTD for UE not capable of supporting RTD > CP - Option 1: (MediaTek) - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=0 and M2=0 - Option 2: (Nokia, Samsung, Xiaomi, ZTE, Huawei, QC, Ericsson, vivo, Apple) - If UE supports sTxMP - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=1.6us and M2=0.5 us - If UE doesn’t support STxMP - Wait for RAN1 further progress for gap/scheduling restriction - No MTTD requirements for this case. Issue 2-1-2: MTTD requirements applicability Agreement: - RAN4 to discuss the MTTD requirement with two TAGs only for mDCI. Issue 2-1-3: Reference timing - Proposals: - Option 1: (Nokia, Ericsson) - The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission. - Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking. - Option 2: (Apple, Nokia, MediaTek, ZTE, vivo, Huawei, Samsung) - In UL timing requirements, some clarification needs to be added to accommodate - Two DL reference timings are supported where each DL reference timing is associated with one TAG. - Two TAGs associated with different UL/joint TCI state. - Option 3: (MediaTek) - Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing - FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs. - Option 4: (Huawei) - For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission. Note: FFS on whether single reference shall be allowed or not. More RAN1 input is expected for further RAN4 discussion. Issue 2-1-4: TDM and overlapping UL transmissions for multi-TRP with 2 TAs - Proposals: - Option 1: (Apple, MediaTek) - For FR2, RAN4 shall start from assumption that UE is only able to perform TX from one panel at a time. - It is proposed to wait for more RAN1 input before RAN4 further discussion. - Option 2: - When the UE does not support UL STxMP transmission: (Nokia, Huawei, ZTE, Samsung, vivo) - Postpone discussion and wait for further progress from RAN1. - When the UE support UL STxMP transmission (FR2 only) (Nokia, Huawei, ZTE, Samsung) - no restrictions from RRM perspective - Option 3: If UL transmissions associated with different TAs overlap, the earlier slot is reduced in duration relative to the later slot. (Ericsson) - Option 4: (Xiaomi) - Scheduling restriction is needed in specific scenarios for TDM UL two TA cases. - Wait to see the switching time discussion in the RF section of the switching time. Issue 2-1-5: TAG management for multi-TRP with 2 TAs - Proposals: - Option 1: (Apple) - RAN4 can do some study on TAG management for multi-TRP with 2 TAs. - For example: Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon. - Option 2: use LTE CA requirements as baseline (Ericsson, Huawei, ZTE) - Option 3: FFS (vivo, MediaTek, Samsung) Issue 2-1-6: TA adjustment accuracy Agreement: - RAN4 not to consider TA adjustment relaxation when 2 TA commands are used. …

In this tdoc we discuss some open issues.

# 2	Discussion

## 2.1	M1/M2 for MTTD for UE not capable of supporting RTD>CP?

RAN4 sent LS response [2] to RAN1 LS (R1-2205593). Content of the response is captured below.

RAN4 thanks RAN1 for the LS on maximum uplink timing difference between the two TAs for multi-DCI multi-TRP with two TAs. After RAN4 further discussion, following values are agreed as MTTD values.

For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs.

For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.

The issue of capable UE is settled. The issue of UE not capable of supporting RTD>CP remaons.

For each TAG or each UL transmission, maximum error possible at the gNB is

TBSE = Te + TACE + TAAE                                                                                                            (1)

Where:

- Te = UE transmit timing error (section 7.1.2, TS 38.133)

- TACE = TA command resolution error (section 4.2, TS 38.213)

- TAAE = TA adjustment accuracy (section 7.3.2, TS 38.133)

For FR1, for SCS of 15 kHz, TBSE corresponds to 24Ts (12Ts + 4Ts+ 8Ts). That means TBSE is 0.786µs.

TBSE can also be thought as maximum transmission error that is allowed at the UE per one UL transmission corresponding to one DL reference timing. When two TAGs are configured, maximum error possible is 2*TBSE=1.6µs. The transmission error is w.r.t DL timing reference and therefore MTTD can be represented as MRTD + 2*TBSE = MRTD +1.6µs.

From the last meeting agreement M1 is 1.6µs

For FR2, TBSE corresponds to 7Ts. That means TBSE is 0.228µs.

For FR2, MTTD = MRTD + 2*TBSE and from the last meeting agreement M2 is 0.456µs or 0.5µs

Proposal 1: 	For dual reference timing M1 is 1.6 µs and M2 is 0.5 µs, for MTTD for UE not capable of supporting RTD>CP.

## Reference timing

As we state in option 1, “The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission”. We believe this is similar or  at least compatible with  to the position of option 3, “Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing” and option 4, “For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission”.

Proposal 2: 	The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission.

Observation 1: 	We believe Proposal 2 is similar or  at least compatible with  to the position of option 3, “Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing” and option 4, “For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission”.

## TAG management for multi-TRP with 2 TAs

The idea in option is fine for us, “ Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon”.

We believe that CA can be used as a model for TAG management.

Proposal 3: 	The idea in option is fine for us, “ Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon”.

Proposal 4: 		CA can be used as a model for TAG management.

# 3	Conclusion

Proposal 1: 	For dual reference timing M1 is 1.6 µs and M2 is 0.5 µs, for MTTD for UE not capable of supporting RTD>CP.

Proposal 2: 	The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission.

Observation 1: 	We believe Proposal 2 is similar or  at least compatible with  to the position of option 3, “Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing” and option 4, “For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission”.

Proposal 3: 	The idea in option is fine for us, “ Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon”.

Proposal 4: 	CA can be used as a model for TAG management.

# References

[1] R4-2306362, WF on R18 NR MIMO RRM requirements, Samsung.

[2]	R4-2217279m Reply LS on MTTD for multi-DCI multi-TRP with two TAs (Response to:	R1-2205593), Ericsson