3GPP TSG-RAN WG4 Meeting #107	R4-2308924

Incheon, Republic of Korea, 22-26 May, 2023

Source:	Ericsson

Title:	On timing requirements for non-colocated intra band

Agenda item:	8.12.2

Document for:	Approval

# 1	Introduction

In WF from UE RF [1], the following issues are open:

… < Issue 2-1-1: Whether UE supports to switch between Type 1 and Type 2 on a request from BS > Agreement: - Type 1 and Type 2 means different UE capability. - Type 1 is mandatory and Type 2 is optional. < Issue 2-1-2: Whether Type 1 shall be default when no request comes from BS > Way Forward: - Type 1 is default as baseline to continue the following discussions. - How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2. - Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution. - FFS about the definition of Type1 in the next meeting. < Issue 2-1-2-1: How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2 > Way forward: - Encourage companies to provide their views on how UE behave in case of UE is configured change from default (Type 1 UE) to Type 2. - Continue further discussions in the next meeting. < Issue 2-1-2-2: Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution > Way forward: - Continue further discussions in the next meeting. …

In this contribution we discuss the open topics.

# 2	Discussion

The definition of Type 1 can be found in R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e [2].

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

Table 1: UE Types

In Table 1, a UE Type 1 has 4 antennas, shared LNA for 4 RX with limited dynamic range for up to rank-4 MIMO operation.

Observation 1: Type 1 UE is defined in R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e, shared LNA for 4 RX with limited dynamic range for up to rank-4 MIMO operation.

If UE does not indicate Type 2 and capability is absent then type 1 is default for ENDC.

Capability for CA is being defined; however, it is very likely that it will behave in a similar way i.e., absent capability means type 1.

Observation 2: If UE support both type 1 and type 2 for a given band combination, then BS must configure either.

Proposal 1: A Type 2 UE with Type 1 fallback is a new capability.

# 3	Summary

Observation 1: Type 1 UE is defined in R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e, shared LNA for 4 RX with limited dynamic range for up to rank-4 MIMO operation.

Observation 2: If UE supp both type 1 and type 2 for a given band comp, then BS must configure either.

Proposal 1: A Type 2 UE with Type 1 fallback is a new capability.

# References

[1]	R4-2306605, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI.

[2]	R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e

[2]	R4-2306335, WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, Huawei, HiSilicon