3GPP TSG RAN WG4 Meeting #107		 R4-2308928

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.12.2

Source: 	MediaTek Inc.

Title: 	Continue discussion on intra-band non-collocated UE

Document for:	Discussion

1. Introduction

In RAN4 #106-bis-e, WF [1] has been agreed to postpone Type 3a/3b/4a/4b UEs to future release. It was also agreed Type 1 UE is mandatory as baseline and further discuss how UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2.

2. Discussion

In previous RAN4 meeting, the WF[1] was agreed:

Way Forward:

- Type 1 is default as baseline to continue the following discussions.

  - How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2.

  - Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution.

- FFS about the definition of Type1 in the next meeting.

It is our understanding that UE would not be aware whether the Cells of two carriers are collocated or non-collocated but base station does. It would require an indication form gNB to inform UE whether it is configured as type 1 UE or type 2 UE.

Observation 1: UE have no idea whether the Cells of carriers are collocated or non-collocated. It would require an indication from gNB to inform UE on its downlink receiver operation type.

As we checked existing UE capability signalling for inter-band EN DC with downlink overlap non-collocated deployment, interBandMRDC-WithOverlapDL-Bands-r16, UE report the optional capability but there is no signalling to indicate UE to be configured as type 1 or type 2 UE configuration. Once UE reports supporting of the IE, it shall always be configured as type 2 UE (2-layer) behaviour. This is because if the default UE configuration is type 1 UE when UE reports its capability supporting on type 2, UE would not be able to handle large MRTD due to non-collocation of the cells since there is no existing signalling indication can change UE configuration to type 2 UE.

Observation 2: There is no existing signalling indication can change UE configuration switched between type 1 and type 2 configuration.

Observation 3: Once an EN-DC UE reports supporting of the IE, interBandMRDC-WithOverlapDL-Bands-r16, it will  always be configured as type 2 UE (2-layer) behaviour. Even an PCell or SCell may be changed to collocated one, there’s no signalling to indicate UE such change.

Although it may be possible to distinguish collocation or non-collocation from the configured layer number, more details on operation scenarios need to be further discussed. Here we provide some discussion on possible scenarios.

For UE indicating supporting on type 2 capability:

- Default UE configuration shall be type 2

- If SCell is attached and configured 2-layer, UE shall meet type 2 UE requriements

- If SCell is attached and configured 4-layer,

  - UE may treat it as error case or

  - UE may change its configuration to type 1 UE, but there is no legal time gap reserved for UE to handle such configuration change as well as protocol change including different timing alignment configurations.

The UE behaviour is unclear that is not beneficial to network scheduling. And it may further create ambiguity issue once type 3 and type 4 UE starts to be discussed in future release. Even UE can interpret the 4-layer configuration implies UE change from type 1 to type 2, this is customization change that UE need to dig time gap by itself that may bring unstable factor on protocol layer. Whether UE can have enough time gap is not guaranteed due to this is additional actions other than cell attach/release. In our understanding, UE cannot do such customization.

Observation 4: UE cannot do customization action form interpretation on scheduled DL layer number other than normal cell attach/release procedure.

Observation 5: It may bring ambiguity issue once type 3 and type 4 UE starts to be discussed in future release if some UE configurations are based on interpretation of scheduled DL layer number

As for whether to extend the existing IE for NR CA. In our understanding, the IE interBandMRDC-WithOverlapDL-Bands-r16 was defined for MRDC. So it may have non-backward compatible issue if the IE is reused for NR CA operation. It would be better to specify a new IE for this purpose. With these understanding, we agree with the proposal 1 in [3].

Proposal 1: Introduce an optional new IE for NR CA, [intraBandNonColocatedCA-r18] for UE supporting type 2 UE capability on NR CA operation.

- Default UE configuration is type 1 UE if the new IE is not signaled or supported.

- Default UE configuration shall be type 2 UE if capable to support the new IE for NRCA.

Proposal 2: Introduce an optional new signaling indication for both NR CA and MRDC for gNB to configure UE switching between type 1 and type 2 UE. Requirements and more details on the new signaling indication can be further discussed by RAN2 and RAN4 RRM session.

3. Conclusion

Observation 1: UE have no idea whether the Cells of carriers are collocated or non-collocated. It would require an indication from gNB to inform UE on its downlink receiver operation type.

Observation 2: There is no existing signalling indication can change UE configuration switched between type 1 and type 2 configuration.

Observation 3: Once an EN-DC UE reports supporting of the IE, interBandMRDC-WithOverlapDL-Bands-r16, it will  always be configured as type 2 UE (2-layer) behaviour. Even an PCell or SCell may be changed to collocated one, there’s no signalling to indicate UE such change.

Observation 4: UE cannot do customization action form interpretation on scheduled DL layer number other than normal cell attach/release procedure.

Observation 5: It may bring ambiguity issue once type 3 and type 4 UE starts to be discussed in future release if some UE configurations are based on interpretation of scheduled DL layer number

Proposal 1: Introduce an optional new IE for NR CA, [intraBandNonColocatedCA-r18] for UE supporting type 2 UE capability on NR CA operation.

- Default UE configuration is type 1 UE if the new IE is not signaled or supported.

- Default UE configuration shall be type 2 UE if capable to support the new IE for NRCA.

Proposal 2: Introduce an optional new signaling indication for both NR CA and MRDC for gNB to configure UE switching between type 1 and type 2 UE. Requirements and more details on the new signaling indication can be further discussed by RAN2 and RAN4 RRM session.

Reference:

- R4-2306605 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, RAN4#106-bis-e

- R4-2304307 LS on signaling support for intra-band non-collocated CA, RAN4#106-bis-e

- R4-2304308 On type 3a/3b UE for intra-band non-collocated CA/EN-DC