3GPP TSG-RAN WG4 Meeting # 107	R4-2308940

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.3.1.3

Source:	Ericsson

Title:	Remaining issues on SDR requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #106bis-e, new progress has been achieved in this WI. In this meeting, we go through the remaining issues that still need to be addressed as stated in [1]. Below, we summarize the remaining open issues on the SDR demodulation requirements’ definition.

Issue 3-1: PDCCH configuration - Use 2Tx to transmit PDCCH Issue 3-2: SDR test cases for 6 MIMO layers for 256QAM - Do not introduce 6 layers SDR requirements for 8Rx UE Issue 3-3: Maximum MCS and MIMO layers for 64QAM - 2 and 4 MIMO layers resus the existing MCS value defined in Table 5.5A-5 of TS 38.101-4 - 8 layers: MCS26 (Table 1) Issue 3-4: Maximum MCS and MIMO layers for 256QAM - 2 and 4 layers: reuse the requirements defined in Table 5.5A-5 of TS 38.101-4 - Candidate options for maximum achievable MCS for 8 layers - Option 1: MCS22 - Option 2: MCS24 Issue 3-5: Maximum MCS and MIMO layers for 1024QAM - Introduce SDR requirements for 1024QAM at least for 2 MIMO layers - Candidate options - Option 1: Only 2 layers: - Option 1a: MCS23 - Option 1b: MCS24 - Option 2: Both 2 and 4 layers: - Option 2a: MCS23 - Option 2b: MCS24

# 2	SDR demodulation requirements

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4 [2], the existing MCS index table has already covered the configuration applicable for UEs up to rank 4 for QPSK/16QAM/64QAM/256QAM, while up to rank 2 for 1024QAM.

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 8 PDSCH MIMO layers using 8 Rx are considered.

According to Observation 1, and using Table 2-1 and Table 2-2 below, we carried out simulations on SDR demodulation to define new requirements for 8 Rx UE in FR1.

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | Wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

Table 2-2: Additional test parameters for TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to FR1.30-1 |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

Based on the simulation results in [3], we provide the following Proposals.

Proposal 1: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS index for maximum modulation format 8 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Furthermore, our view is that SDR requirements for 256QAM can also be defined with 8 PDSCH MIMO layers. Proposal 2 summarizes the needed updates to Table 5.5A-5 in TS 38.101-4 [2].

Proposal 2: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 22 |
| 8 | 8 | 0.75 | 21 |
| 8 | 8 | 0.4 | 12 |
| Note 1: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Moreover, our simulation results in [3] showed that SDR requirements can be defined for 1024QAM modulation when using 8 Rx UE using 8 PDSCH MIMO layers. Proposal 3 summarizes the needed updates to Table 5.5A-6 in TS 38.101-4 [2].

Proposal 3: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 4.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 4 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
|  | 2 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 8 PDSCH MIMO layers using 8 Rx are considered.

Proposal 1: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 2: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 22 |
| 8 | 8 | 0.75 | 21 |
| 8 | 8 | 0.4 | 12 |
| Note 1: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 3: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 4.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 4 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
|  | 2 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

# References

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.

- R4-2308941, “Simulation results for 8 Rx SDR in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	SDR Tables

Table 5.5A-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| .2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Table 5.5A-6: 1024QAM MCS indexes for indicated UE capabilities

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2RX | 1 | 10 | 1 | 23 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
| 4RX | 1 | 10 | 1 | 24 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
|  | 2 | 10 | 1 | 23 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |