3GPP TSG-RAN WG4 Meeting # 107	R4-2308942

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.3.1.4

Source:	Ericsson

Title:	Remaining issues on CQI reporting for 8Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #106bis-e, some progress has been achieved in this WI. In this meeting, we go through the remaining issues that still need to be addressed as stated in [1]. Below, we summarize the remaining open issues on the CQI reporting definition.

Issue 4-1: Report quantity configuration - Use the legacy configuration, report quantity 'cri-RI-PMI-CQI ' - Use two-one-TypeI-SinglePanel-Restriction = 00000001 - Following options for i2 will be discussed for next RAN4 meeting: - Option 1: TE sets random i2 (1 out of 2 possibilities) during the test - Option 2: TE uses fixed value i2 = 0 or 1 during the test - Option 3: Leave TE implementation to use random i2 or fixed i2 = 0 or 1 Issue 4-2: The highest modulation order for 8Rx CQI test - Up to 64QAM Issue 4-3: Tx EVM assumption for derivation of SNR values for CQI test - No need to discuss if simulation results are well aligned Issue 4-4: SNR points - Option 1: [4,5] dB and [10,11] dB - Option 2: [7,8] dB for 64QAM, [1,2] dB for 16QAM - Other options are not precluded Issue 4-5: Whether to define PMI and RI requirements No need to define PMI and RI requirements

It is worth mentioning that we carried out simulations in [2] to draw our proposals.

# 2	CQI reporting requirements

In WF [1], we agreed to go back to the legacy configuration for 8 Rx UE CQI reporting requirements. Moreover, we already agreed to consider Rank 4. Both agreements can be reminded in Observation 1.

Observation 1: For 8 Rx UE CQI reporting, it has been agreed to

- Consider the legacy configuration report quantity 'cri-RI-PMI-CQI '

- Use two-one-TypeI-SinglePanel-Restriction = 00000001

Observation 2: Based on Table 5.2.2.2.1-8 and Table 5.2.2.2.1-4 in TS 38.214, i1,2=0 and i1,3=0. The index i1,1 should be one out of eight values, {0,…, 7}, while i2 should be one of 2 values, {0, 1}.

In the last meeting, the discussions have been whether to select the codebook index i2 or to leave it random. Our view is to not choose i2 and leave it random, where one out of two values will be selected.

Proposal 1: For 8 Rx UE CQI reporting, define requirements considring

- Legacy configuration report quantity 'cri-RI-PMI-CQI '

- two-one-TypeI-SinglePanel-Restriction = 00000001

- TE sets random i2 (1 out of 2 possibilities) during the test

Hereafter, we summarized the simulation parameters that have been agreed so far, in the following Table.

Table 2-1 Simulation parameters for CQI reporting.

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| andwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,0 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | N/.A |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| hMaximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

Based on the simulation results provided in [2], Fig. 1 presents the median CQI as function of SNR under static conditions in FR1.

Fig. 1 – Median CQI as function of SNR. CQI reporting under static condition for 8 Rx UE in FR1.

Proposal 2: Define the CQI reporting requirements for 8Rx UE under static conditions while considering Rank 4, 64QAM as the highest modulation order and based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4, with test points

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

Furthermore, and based on our simulation results in [2], it is worth noting that EVM values does not impact the CQI reporting requirements. However, and since we are using Table 2 for CQI reporting requirements, we can use Tx EVM = 3%.

Proposal 3: Based on our simulation results, EVM values did not impact CQI reporting requirements. However, we can always keep EVM = 3% since we are using Table 2 for CQI reporting.

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: For 8 Rx UE CQI reporting, it has been agreed to

- Consider the legacy configuration report quantity 'cri-RI-PMI-CQI '

- Use two-one-TypeI-SinglePanel-Restriction = 00000001

Observation 2: Based on Table 5.2.2.2.1-8 and Table 5.2.2.2.1-4 in TS 38.214, i1,2=0 and i1,3=0. The index i1,1 should be one out of eight values, {0,…, 7}, while i2 should be one of 2 values, {0, 1}.

Proposal 1: For 8 Rx UE CQI reporting, define requirements considring

- Legacy configuration report quantity 'cri-RI-PMI-CQI '

- two-one-TypeI-SinglePanel-Restriction = 00000001

- TE sets random i2 (1 out of 2 possibilities) during the test

Proposal 2: Define the CQI reporting requirements for 8Rx UE under static conditions while considering Rank 4, 64QAM as the highest modulation order and based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4, with test points

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

Proposal 3: Based on our simulation results, EVM values did not impact CQI reporting requirements. However, we can always keep EVM = 3% since we are using Table 2 for CQI reporting.

# References

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei.

- R4-2308943, “Simulation results for CQI reporting for 8Rx in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	CQI reporting in static condition

### We use the following tables in TS 38.101-4

Table A.4-3: Mapping of CQI Index to Information Bit payload (CQI table 2, Rank 3 and Rank 4)

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 [12] |  |  |  |  |  |  |  |  |  |