3GPP TSG-RAN WG4 Meeting #107                                      R4-2308949

Incheon, Korea, May 22 – 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On further enhancements to reduce MPR&PAR

Agenda Item:	8.28.2

Document for:	Approval

Introduction

In previous RAN4 meeting, following agreements have been achieved in WF [1]. RAN4 will continue simulation results collection and further analysis based on the results are expected.

Solutions to consider for MPR/PAR reduction Agreement: - RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107. - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107. Capturing of Simulation Results Agreement: - RAN4 agrees that companies shall fill both the ‘OBO’ and ‘10% BLER SINR’ columns in the RAN4 simulation templates for MPR/PAR result collection. Net gain calculations Agreement: - Use the following equation as the baseline Net Gain dB=PoutFilter i-PoutRef+SNR10%Ref-SNR10%Filter i - Where PoutFilter i is the achieved (TX) output power of the filtered waveform being compared against the reference, PoutRef is the output power of the reference, SNR10%Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%Filter i is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference. - Use the following equation when providing the results for net gain Net Gain dB=-DOPR_M0 +SNR10%Ref-SNR10%Filter i Spectrum extension factor (α) Agreement: - For evaluations, RAN4 should consider the spectrum extension factor (α) as: 0 (reference), 0.25, 0.111 (1/9), 0.125 and 0.375. BS Rx assumption Agreement: When expressing the net gain companies shall indicate the BS receiver type used. ACLR requirement Agreement: RAN4 shall further study whether the ACLR of a PC2 UE can be applied to a PC3 UE when the output power is boosted to the same level of a PC2 UE.

In this contribution, we would like to update our simulation results of solutions for Rel-18 further enhancements to reduce MPR&PAR and provide necessary analysis.

Discussion

Further analysis on MPR&PAR reduction schemes

Net gain (delta Pout minus BLER loss)

As what has been agreed previously, the power boost gain shall be reported comparing to the known baseline, i.e., “QPSK DFT-s-OFDM without any filter”. Besides, the net gain is defined as the achieved output power minus SNR loss under 10% BLER condition. The simulation results are provided towards the following cases.

Table 1: Simulation cases as per the agreement in RAN1#112

| No spectrum extension |  |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Case | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 1 | 8 | 0 [only QPSK] | 6 | 8 | 1 [only QPSK] | 1/4 |
| 2 | 8 | 6 | 6 | 8 | 8 | 1/4 |
| 3 | 40 | 2 | 30 | 40 | 3 | 1/4 |
| 4 | 40 | 6 | 30 | 40 | 8 | 1/4 |
| 5 | [6 | 3 | 4 | 6 | 5 | 1/3] |
| 6 | [36 | 7 | 32 | 36 | 8 | 1/9] |

Case 1/2                            Case 3/4

Case 5                              Case 6

Figure 1. MPR performance for target cases

Judging from the above figure, the power boost gain from FDSS is relatively small while spectrum extension can introduce considerable gain on top of it.

Case 1                             Case 2

Case 3                             Case 4

Case 5                             Case 6

Figure 2. Net gain for target cases

As depicted in Figure 2, the total gain of FDSS with spectrum extension under QPSK could be varied for different MCS. In principle, we think ~1dB total gain can be considered for inner region.

Observation 1: Based on the simulation results towards the cases listed in Table 1, the net gain can be around 1dB for inner region for QPSK under FDSS with spectrum extension, while the net gain from FDSS without spectrum extension is negligible.

Spectrum extension ratio

(a) MCS=0                    (b) MCS=4                    (c) MCS=7

Figure 3. An example on the total gain comparison between spectrum extension ratio 1/9 and 1/4

In Figure 3, we present the comparison of total gain between different spectrum extension ratios. If we observe the gain for edge/outer region, the extension ratio with better total gain is 1/4, while for inner region, the better extension ratio for lower MCS order is 1/9.

Observation 2: The best extension ratio for different combination of MCS and RB allocation is different:

- For edge RB allocation, the best extension ratio maximizing the total gain is 1/4.

- For inner RB allocation, when MCS index exceeds 4, extension ratio 1/9 shows better performance comparing to extension ratio 1/4.

Moreover, extension ratio 1/9 satisfies the DFT size restriction and it can achieve better spectrum efficiency than 1/4 under the same allocation obviously. Thus, extension ratios 1/4 and 1/9 should be supported to achieve the best performance for different MCS index and RB allocations.

Proposal 1: For spectrum extension ratio, both 1/4 and 1/9 should be specified.

Other spectrum extension scheme

According to RAN1 discussion, three extension methods have been agreed for studying:

•	Option 1: Symmetric extension

•	Option 2: Cyclic extension

•	Option 3: Cyclic shift plus symmetric extension

Option 3 can be denoted as the following general formulation:

| Xsek=Xk+L mod Ndata, k∈0,1,…Nsc-1, |  |
| --- | --- |

This formulation uses a general shift parameter L encompassing the other options which correspond to specific values of L:

- Option 1 is obtained by selecting L=Ndata-Ne/2. The SE becomes:

| X(se)=XNdata-Ne2,…,XNdata-1, X0,…,XNdata-1, X0, …,XNe2-1 | (1) |
| --- | --- |

- Option 2 is obtained by selecting L=0. The SE is then specifically

| X(se)=X0,…,XNdata-1, X0,…,XNe-1 | (2) |
| --- | --- |

Observation 3: Option 1 and Option 2 are special cases of Option 3 with specific cyclic shift L.

For any value of L, the original sequence of Fourier coefficient X0,…, XNdata-1 as shown in Figure 4 is always included in the in-band spectrum up to a cyclic-shift by L+Ne2 symbols; and the left-side excess-band symbols are always the repetition of symbols of the right-side in-band edge; and similarly for the right-side excess band.

For all options, which corresponds to different shift values L, the definition of SE needs to be known at the receiver. Beside this, it does not change the requirement of the receiver, except of performing the corresponding inverse cyclic shift.

Observation 4: The choice of L does not change the set of symbols in the in-band and thus does not change the receiver requirement.

Figure 4: Illustration of spectrum-extended data sequence as a function of the shift parameter L with Ndata=10 symbols and Ne=4.

In fact, any option can be equivalently be implemented as a cyclic extension or symmetric extension:

- Cyclic extension implementation of (1): cyclically shifted X[k] by L then perform cylic extension.

- Symmetric extension implementation of (1): cyclically shifted X[k] by L+Ne2 symbols and then perform symmetrical extension.

Observation 5: All options can either be implemented as a cyclic extension or symmetric extension.

While the definition of SE, i.e. the value of L, does not change the receiver requirement, it does change the PAPR of the received signal. This is because it changes the phase differences among the DFT-s-OFDM pulses. Indeed, it can be shown that with DFT-s-OFDM using FDSS-SE, the constellation symbols are multiplexed by the set of pulses gmnm=0Ndata-1 where

| gmn=hn-NfftNdatame-j2πLNdatam | (3) |
| --- | --- |

with the pulse-shaping filter hn=k=0Nsc-1Wkej2πkNfftn

Using Lemma 1 of [8] the phase difference between two neighbouring pulses can be shown to be

| gm[n]gm+1[n]=-2πNdataL+Ne-12 (mod π) | (4) |
| --- | --- |

This phase difference influence thee coherent combining of pulses and thus the overall fluctuation of the signal’s envelope.

With Option 1 (L=Ndata-Ne/2) and Option 2 (L=0) , the phase differences become -2πNe-1Ndata and gm[n]gm+1[n]=-πNe-1Ndata , respectively. This means that with these options the phase difference between neighbouring pulses depends of the extension size and is uncontrolled.

With Option 3 and L= Ndata8-Ne-12, we instead have gm[n]gm+1[n]≈-π4. In this case the phase difference between neighboring pulses is maintained constant. As a result, consecutive QPSK symbols will always be transmitted with an absolute phase difference of at least π4, and the signal’s envelope fluctuation would be minimized.

For practical performance comparison between option 2 and option 3, as shown in Figure 5 and 6, cyclic shift plus symmetric extension (Option 3) has about MPR gain of 0.1-0.2dB compared to symmetric extension (Option 2.

Figure 5. MPR performance comparison between Option 2 and Option 3 using TRRC (0.5, 0.1667) and 1/4 extension

Figure 6. MPR comparison between Option 2 and Option 3 using TRRC (0.5, 0.1667) and 1/3 extension

Moreover, the Option 1, 2, and 3 all have to copy a portion of initial DFT coefficients with the same length to obtain extended DFT coefficients, except that the location of copied portion is different. Thus, the implementation complexities of Option 1, 2, and 3 are similar and do not have significant difference.

Observation 6: Cyclic shift on with symmetric extension can provide 0.1~0.2 MPR gain compared to no cyclic shift for inner RB allocation without any additional impact to the receiver.

Proposal 2: Define RF requirements to support FDSS with spectrum extension for further coverage enhancement towards QPSK with DFT-s-OFDM waveform.

Miscellaneous

During previous meetings, there were concerns about the complexity of FDSS+SE. From transmitter perspective, we understand the complexity should be the same as FDSS because spectrum extension is merely mapping between data and RB resources, while for receiver it is up to implementation about whether to use the extended part or not, but it is definitely not the concern from transmitter aspect.

Observation 7: There is no identified implementation complexity increment for spectrum extension comparing to the specified FDSS scheme.

As for the transparent schemes other than FDSS that have been mentioned in the last meeting, more details on how to achieve power boosting without any performance degradation should be clarified first.

Proposal 3: Regarding the transparent schemes other than FDSS, more clarification on the details should be provided in order to get convinced evaluation of the gain.

Conclusion

In this contribution we discussed on the candidate solutions for further coverage enhancement, we have the following observations and proposals:

Observation 1: Based on the simulation results towards the cases listed in Table 1, the net gain can be around 1dB for inner region for QPSK under FDSS with spectrum extension, while the net gain from FDSS without spectrum extension is negligible.

Observation 2: The best extension ratio for different combination of MCS and RB allocation is different:

- For edge RB allocation, the best extension ratio maximizing the total gain is 1/4.

- For inner RB allocation, when MCS index exceeds 4, extension ratio 1/9 shows better performance comparing to extension ratio 1/4.

Observation 3: Option 1 and Option 2 are special cases of Option 3 with specific cyclic shift L.

Observation 4: The choice of L does not change the set of symbols in the in-band and thus does not change the receiver requirement.

Observation 5: All options can either be implemented as a cyclic extension or symmetric extension.

Observation 6: Cyclic shift on with symmetric extension can provide 0.1~0.2 MPR gain compared to no cyclic shift for inner RB allocation without any additional impact to the receiver.

Observation 7: There is no identified implementation complexity increment for spectrum extension comparing to the specified FDSS scheme.

Proposal 1: For spectrum extension ratio, both 1/4 and 1/9 should be specified.

Proposal 2: Define RF requirements to support FDSS with spectrum extension for further coverage enhancement towards QPSK with DFT-s-OFDM waveform.

Proposal 3: Regarding the transparent schemes other than FDSS, more clarification on the details should be provided in order to get convinced evaluation of the gain.

- References

- R4-2306627, “WF on remaining open issues for MPR/PAR reduction for coverage enhancement”, Nokia, 3GPP TSG-RAN4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.