3GPP TSG-RAN WG4 Meeting #107	R4-2308972
Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.28.1

Source:	OPPO

Title:	R18 Coverage enhancement related to SAR mitigation

Document for:	Approval

- Introduction

The coverage enhancement WF [1] was approved in last meeting, and the open issues are as below. This paper will discuss further on these open issues.

Issue 1: Whether to introduce PHR reporting for the carrier that is configured for DL but no UL < Agreement >: FFS whether to introduce PHR reporting for the carrier that is configured for DL but no UL. - The target scenario and details like how to calculate the PHR are encouraged to provide by proponent in the next meeting. Issue 2: Proposals based on power class related information reporting for FR1 carrier < Agreement >: FFS on the following power class related information reporting solutions for FR1 carrier: - Option 1: Power class being used by the UE instead of ΔPPowerClass. - Option 2: Report power-class fallback ΔPPowerClass in the PHR per serving cell using spare bits in the existing PHR MAC-CE; reporting by UEs implementing SAR mitigation by P-MPR and duty cycle averaging (the ‘P-MPR method’) with finer granularity than the ΔPPowerClass is supported using the said spare bits of the PHR in combination with the P-bit. - Power-capability change, power-class fallback or return to declared power class, can trigger an aperiodic report. - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, also trigger an aperiodic PHR. - Option 3: Report both power-class fallback ΔPPowerClass and the full power transmission capability for the current fall-back power class. Issue 3: Whether to introduce P-MPR reporting for FR1 carrier < Agreement >: FFS whether to introduce P-MPR reporting for FR1 carrier. Issue 4: Proposals based on ‘duration’ information reporting for FR1 carrier <Agreement>: FFS whether to consider the following duration related information reporting solutions for FR1 carrier. - Option 1: On top of the power class related information reporting, also consider report evaluation period, the starting time and estimated time for return to higher power class. - Option 2: Consider UE reporting on the following information as a ‘forecast’. - For current UEs, information about how long the gNB should not expect better performance: - For UEs that fall-back in power class: how long a duty-cycle related power class fallback is expected to persist. - For UEs that use P-MPR: how long the reported range of P-MPR is expected to persist. - For all enhanced UEs, information about how long the gNB can expect un-degraded performance: - How long a UE can execute UL grants based on duty-cycle capability and last reported Pcmax. - Alternatively, how long a duty cycle can be sustained without triggering additional P-MPR. - Option 3: Sustainable duty cycle.

- PHR for the carrier that is configured for DL but no UL

The proposal of PHR reporting for carriers with DL but no UL has been proposed in previous meetings, however, it is unclear what the targeting scenario or reason of this kind of reporting. In last meeting online discussion, it was clarified that this is for the case that in one band there is antenna blocked by human body, then the Tx power will be low, and UE can change it to another band for signal transmission. However, it was also commented that this is not something new, UE usually implement with serval antennas to solve this issue. Therefore, from UE implementation perspective, there seems no problem and necessity for such PHR reporting.

Observation 1:   The target scenario for PHR reporting for carrier with DL but no UL seems is to solve the antenna blocking, however, UE usually implement with serval antennas which makes this enhancement seems not necessary.

Proposal 1:         No PHR reporting for carrier with DL but no UL.

- Power class related reporting for FR1 carrier

The FR1 power class related reporting currently are mainly caused by the maxUplinkdutycycle related behaviours, i.e. currently in the spec when scheduled uplink duty cycle is higher than the maxUplinkdutycycle capability then the lower power class requirements apply. However, this description actually caused different understandings in RAN4, one understanding is that UE will change its power class in connected mode, however, the other understanding is that UE will never change its power class in connected mode since there is no signalling update approach currently. For this issue, it might be better to align the understanding in RAN4 since it will impact not only this discussion but also many other ongoing WIs.

Observation 2:   There are different understanding in UE behavior when the scheduled UL duty cycle exceeds maxUplinkdutycycle UE capability, i.e. whether UE power class will be changed.

Proposal 2:         Clarify that Option2 is the correct UE behavior instead of Opiton1 when scheduled UL duty cycle exceeds maxUplinkdutycycle:

Option 1: UE power class will fall back from PC2 to PC3

Option 2: UE power class will keep PC2 unchanged, but its achievable max transmit power will be restricted to 23dBm and the PC3 requirements will be applied.

With above understanding that UE power class will not be changed, then the “power class being used by UE” in connected mode actually is same as the power class reported in initial access.

Regarding ΔPPowerClass which is defined for several cases like duty cycle caused power reduction, Pemax configuration, SRS antenna switching.

- The duty cycle caused power reduction is not happened in reality up to now since the maxUplinkdutycycle capability in implementation has difficulty to align time window between UE and NW, and NW also has difficulty to handle UEs different time capability.

- The Pemax configuration is static in the field that NW knows.

- And SRS antenna switching is as NW configuration based.

So there seems the factors caused ΔPPowerClass changes will either not happened in NW or known by NW which makes there is no strong necessity to report this ΔPPowerClass.

Observation 3:   There factors that impact ΔPPowerClass including maxUplinkdutycycle caused power reduction, Pemax configuration, and SRS antenna switching. And all these factors are either not happening in the real field or already known by NW.

Proposal 3:         ΔPPowerClass doesn’t need to be reported.

- P-MPR reporting for FR1

It is unclear how P-MPR would be a problem for FR1. Similar proposal has been discussed already when FR2 P-MPR was introduced and conclusion is the PMPR reporting only for FR2. We didn’t notice the RLF of FR1 due to PMPR, if this happens then people may consider the PC1.5 is no longer useful in the NW.

Observation 4:   Radio link failure caused by high power UE is not observed in FR1 which is different from FR2.

Proposal 4:         P-MPR reporting for FR1 is not considered.

- ‘duration’ information report for FR1

The idea of duration information reporting is still about the SAR related discussion which means that UE may need to restrict it’s transmitting time to meet the SAR regulation restrictions. And the candidate reporting parameters include

- Option 1: “report evaluation period, the starting time and estimated time for return to higher power class”,

- Option 2: “How long a UE can execute UL grants based on duty-cycle capability and last reported Pcmax”,

- Option 3: “how long a duty cycle can be sustained without triggering additional P-MPR”,

- Option 4: “Sustainable duty cycle”.

For Option 1, the evaluation period reporting actually is intended to enhance the maxUplinkdutycycle approaches to make it possible in the NW since this is one of the problems that refrain UE and NW from implementing it. However, considering the difficulties in reality for NW to monitoring all UEs in the cell its starting time and stop time and duty cycle length, and also potential concerns from power consumption in UE, probably this option can be dropped.

For Option 2, Option 3, and Option 4, they are similar and are prediction time duration reporting. For this approach, more details maybe needed to further evaluate the complexity and possibility in real NW. And all of them need first UE and NW to do UL Tx time duration calculation and statistics. This cannot be done by current UEs considering the power consumption is sensitive to UEs.

Observation 5:   Time duration related reporting is difficult in real field which may impact UE power consumption, and also NW scheduling complexity with many UEs in NW.

Proposal 5:         ‘Duration’ information report for FR1 is not considered.

- Conclusions

In this contribution, the Tx switching is analysed. Based on the above analysis, we got the following observations and proposals.

Observation 1:   The target scenario for PHR reporting for carrier with DL but no UL seems is to solve the antenna blocking, however, UE usually implement with serval antennas which makes this enhancement seems not necessary.

Proposal 1:         No PHR reporting for carrier with DL but no UL.

Observation 2:   There are different understanding in UE behavior when the scheduled UL duty cycle exceeds maxUplinkdutycycle UE capability, i.e. whether UE power class will be changed.

Proposal 2:         Clarify that Option2 is the correct UE behavior instead of Opiton1 when scheduled UL duty cycle exceeds maxUplinkdutycycle:

Option 1: UE power class will fall back from PC2 to PC3

Option 2: UE power class will keep PC2 unchanged, but its achievable max transmit power will be restricted to 23dBm and the PC3 requirements will be applied.

Observation 3:   There factors that impact ΔPPowerClass including maxUplinkdutycycle caused power reduction, Pemax configuration, and SRS antenna switching. And all these factors are either not happening in the real field or already known by NW.

Proposal 3:         ΔPPowerClass doesn’t need to be reported.

Observation 4:   Radio link failure caused by high power UE is not observed in FR1 which is different from FR2.

Proposal 4:         P-MPR reporting for FR1 is not considered.

Observation 5:   Time duration related reporting is difficult in real field which may impact UE power consumption, and also NW scheduling complexity with many UEs in NW.

Proposal 5:        ‘Duration’ information report for FR1 is not considered.

- References

- R4-2306625, WF on NR further coverage enhancement part1, Huawei, RAN4#106bis-e, April 2023