3GPP TSG-RAN WG4 Meeting # 107										R4-2309006

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.872 to introduce CA_n5-n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the study results of CA_n5-n105 into TR 38.872 based on operators’ request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the spectrum allocation of CA_n5-n105 in section 5.4.1. - Add the potential UE RF architecture to support CA_n5-n105 in section 5.4.2. - Add the RF requirement study for CA_n5-n105 with 1UL configuration in section 5.4.3. - Add the RF requirement study for CA_n5-n105 with 2UL configuration in section 5.4.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study results of CA_n5-n105 would be missing in TR 38.872. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2, 5.4.3, 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1>>

## 5.4	CA_n5-n105

### 5.4.1	General

In this configuration the uplink is also CA_n5A-n105A, but there are two fall back options:

- DL_n5A-n105A_UL_n5A_BCS0

- UL_n5A-n105A_UL_n105A_BCS0

The aggregated spectrum allocation for CA_n5A-n105A is illustrated in Figure 5.4.1-1.

Figure 5.4.1-1: Aggregated spectrum allocation for CA_n5A-n105A

## << End of change 1>>

## << Start of change 2>>

### 5.4.2	UE RF architecture assumption

The UE RF architectures with 2 antenna and 3 antenna are assumed in the feasibility study for CA_n5-n105. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

#### 5.4.2.1	UE RF architecture with two antennas

For CA_n5-n105 with 2-antenna implementation, there are two potential UE RF architectures as shown in Figure 5.4.2.1-1.

#### Figure 5.4.2.1-1a: Potential UE architecture with 2 antenna to support CA_n5-n105

#### Figure 5.4.2.1-1b: Potential UE architecture with 2 antenna to support CA_n5-n105

The topology for 2-antenna architecture illustrated in Figure 5.4.2.1-1a is listed as below.

-	1) n105UL+n105DL+n5UL+n5DL quadplexer on antenna 1

-	2) n5DL+n105DL duplexer on antenna 2

The topology for 2-antenna architecture illustrated in Figure 5.4.2.1-1b is listed as below.

-	1) n5UL+n5DL+n105DL triplexer on antenna 1

-	2) n105UL+n105DL+n5DL triplexer on antenna 2

#### 5.4.2.2	UE RF architecture with three antennas

Figure 5.4.2.2-1 illustrates the potential UE RF architecture for CA_n5-n105 with 3-antenna implementation.

Figure 5.4.2.2-1: Potential UE architecture with 3 antenna to support CA_n5-n105

The topology for 3-antenna architecture illustrated in Figure 5.4.2.2-1 is listed as below:

1) n105UL+n105DL duplexer on antenna 1

2) n5UL+n5DL duplexer on antenna 2

3) n5DL+n105DL duplexer on antenna 3

## << End of change 2>>

## << Start of change 3>>

### 5.4.3	Common for 1 band UL and 2 bands UL of CA_n5-n105

#### 5.4.3.1	Operating bands for CA

#### The operating bands for CA_n5-n105 are provided in Table 5.4.3.1-1.

Table 5.4.3.1-1: CA band combination of band n5+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

#### 5.4.3.2	Channel bandwidths per operating band for CA

#### The supported channel bandwidths for CA_n5-n105 are provided in Table 5.4.3.2-1.

Table 5.4.3.2-1: Supported bandwidths per CA band combination of band n5+n105

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

#### 5.4.3.3	Identify the potential MSD issues due to harmonic/harmonic mixing/cross band isolation

Table 5.4.3.3-1/2 summarizes frequency ranges where harmonics/harmonics mixing occur for CA_n5-n105.

Table 5.4.3.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Table 5.4.3.3-2: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on the above analysis, there is no need to study MSD due to harmonics and harmonics mixing for CA_n5-n105.

#### 5.4.3.4	∆TIB,c and ∆RIB,c values

Two-antenna architecture with dual triplexer can be used as a baseline to study Tib and Rib.

Table 5.4.3.4-1: ΔTIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | TBD |
|  | n105 | TBD |

Table 5.4.3.4-2: ΔRIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | TBD |
|  | n105 | TBD |

#### 5.4.3.5	REFSENS exception evaluation for one UL band

TBD

## << End of change 3>>

## << Start of change 4>>

### 5.4.4	Specific for 2 bands UL of CA_n5-n105

#### 5.4.4.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n105

#### 5.4.4.2	UE co-existence studies

Table 5.4.4.2-1 lists Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.4.4.2-1: Band n5 and Band n105 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Based on the above analysis, there is no IMD issue for CA_n5-n105 with 2UL configuration.

Table 5.4.4.2-2 lists the protected bands required for CA_n5-n105 with 2UL configuration.

Table 5.4.4.2-2: Band n5 and Band n105 UL IMD products

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n105 | E-UTRA Band 1, 3, 4, 5, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 66, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 41, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

#### 5.4.4.3	REFSENS exception evaluation for two UL band

It is not required to study the MSD due to two UL IMD interference since there is no two UL IMD issue for CA_n5-n105.

## << End of change 4>>