3GPP TSG-RAN WG4 Meeting #107 	R4-2309041

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.858: Feasibility of FR1 Wide Area BS aspects

Agenda item:	8.20.2.2.1 - Feasibility of FR1 BS aspects

Document for:	Approval

Introduction

The agreements on the implementation feasibility of SBFD in RAN4 #112-bis-e are captured in the WFs in [1] and [2]. The following topics still need to be further studied:

Sub-topic 2-2: BS aspects: Analysis on component capabilities Issue 2-2-1: RF SIC Agreement - For the implementation feasibility of RF SIC for SBFD operation: - RF cancellation can be used in SBFD to mitigate self-interference pre-Rx LNA in terms of reducing non-linearity effects and overall self-interference residue. - FFS on the implementation complexity. Sub-topic 2-3: Remaining issues for RSI dependency on blocking, AGC and ADC Issue 2-3-2: Impact on RSI from analogue sub-band filter Agreement: - FFS the impact on RSI from analogue sub-band filter - FFS the implementation feasibility of sub-band filter in WA and MR BS classes - If no consensus reached, companies are encouraged to provide per-company analysis in their own feasibility section. … Sub-topic 2-4: RSIC capability overall feasibility analysis Issue 2-4-2: Remaining issues for RSIC analysis framework Agreement: N/A Issue 2-4-3: Excel spreadsheet for RSIC capability analysis Agreement: N/A Issue 2-4-4: RSIC Assumption for co-existence study Agreement: - For the self-interference modeling used for coexistence study: - RAN4 to confirm the adoption of the impact of self-interference modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB) to study the impact of SBFD operation on the RF requirements. - Note: The feasibility study on RSIC not concluded yet and will be further evaluated in RAN4.

Furthermore, it has been agreed that companies can provide individual inputs to TR section 10.2 on feasibility of FR1 wide area BS aspects, and a template for capturing the co-site inter-sector co-channel interference impact has been provided in [2].

In this document, Nokia’s views regarding the open issues are presented. Additionally, this document contains a TP to TR 38.858.

Discussion

2.1 Implementation complexity of RF SIC

Sub-topic 2-2: BS aspects: Analysis on component capabilities Issue 2-2-1: RF SIC Agreement - For the implementation feasibility of RF SIC for SBFD operation: - RF cancellation can be used in SBFD to mitigate self-interference pre-Rx LNA in terms of reducing non-linearity effects and overall self-interference residue. - FFS on the implementation complexity.

There has been a proposal of an AFIR type of canceler solution in which the canceler unit is made of M+N individual cancelers, where M is the number of TXs and N the number of RXs. The canceler unit is connected to TX and RX antenna panels. If M=N=64 there will be 64 RF connections from TX panel to the canceler unit and 64 RF connections from canceler unit to RX panel. That might be doable in a common mechanics but in the case that all three units are separate, it is not feasible anymore. Since the distance of the TX and RX antenna panels need to be quite high (due to needed spatial isolation) and receiver and transmitter chains need to be located close to antenna filters (to avoid excessive insertion losses that cannot be compensated), separate TX and RX PWBs are needed.

Valid mathematics have been presented that M+N cancelers are sufficient for RF IC but that it is true only when one set of beamforming coefficients are valid at a time e.g. in the mmW arrays. For MU MIMO there is a need for (M+N)xUxL cancelers where M and N are as above and U is the number or users and L is the average number of layers per use. For a typical 5G case that would be starting from 5120 cancelers. On top of that, all of them have to be updated every time when the beamforming coefficients are updated. That leads to an intolerable complexity and processing burden when solving mathematics for each of those individually.

Observation 1: RF SIC is not feasible due to intolerable complexity and overwhelming processing burden

2.2 Implementation feasibility of sub-band filtering

Sub-topic 2-3: Remaining issues for RSI dependency on blocking, AGC and ADC Issue 2-3-2: Impact on RSI from analogue sub-band filter Agreement: - FFS the impact on RSI from analogue sub-band filter - FFS the implementation feasibility of sub-band filter in WA and MR BS classes - If no consensus reached, companies are encouraged to provide per-company analysis in their own feasibility section.

Sub-band filtering before or after the LNA is not seen feasible as it will have the effect of:

  - High insertion loss before the LNA will increase the receiver noise figure and negate any system gains of SBFD.

  - High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include:

    - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget.

    - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL

    - Group delay distortion close to the cut-off frequencies

    - Incompatibility with a typical multi-carrier gNB design

    - Increased complexity as switches are needed for by-pass in UL slots for full BW

    - Additional space needed in RX chain that is not available in typical gNB design

    - Overall additional power consumption which leads to thermal management issues

    - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance

  - The new sub-band specific filters would be operator’s spectrum specific and locking the spectrum configuration for any further changes or tuning. The existing bandpass filters for the operating band would anyway be required, suggesting that the new filters would double the filter size for the UL antenna panel. This all means higher cost and complexity.

Observation 2: Sub-band filtering is not feasible due to reasons mentioned above

2.3 Inter-sector interference handling – FR1 Wide Area

In the last meeting, a template for capturing the co-site inter-sector co-channel interference impact has been agreed in [2]. In the table below we provide values for FR1 Wide Area BS.

| FR1 (or FR2-1) |  |  |  | Nokia |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 54 dBm |
| Number of co-site co-channel sectors considered |  |  |  | 2 (plus 1 sector for self-interference) |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD, digital filtering Note: List all relevant techniques used in TX |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 60-80 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation between TX/RX panel; cross polarization Note: List all relevant techniques used in the evaluation |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | n.a |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -68…-48 dBm |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -23…-3 dBm (receiver will be blocked) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | 46 dBc |
|  |  | Frequency isolation techniques |  | Digital filtering, FFT frequency selectivity Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Included in the NF model |
|  |  |  | Rx IM3 contribution (dBm) | Note: Receiver linearity may depend on both inter-sector interference and self-interference… each company can explain how they calculate IM3 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | Inf dBm (receiver will be blocked above -25 dBm input level) Note: Each company to explain if/how they have separated CSSI and SI when considering IM3 |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | 0 dB |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | n.a. |
|  | Digital processing interference supression capability |  |  | 0 dB |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | Inf dBc |
| Noise floor ⑩dBm |  |  |  | Inf dBm/CBW |
| Calculated Desensitization (dB) |  |  |  | Inf dB |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |

A Text Proposal included in the Annex is provided into the TR [3] to capture aspects of FR1 wide area base stations.

Proposal 1: Include the provided TP into the TR 38.858.

Conclusion

In this contribution we discuss the feasibility of BS RF FR1 wide area. The following observations are made:

Observation 1: RF SIC is not feasible due to intolerable complexity and overwhelming processing burden

Observation 2: Sub-band filtering is not feasible due to reasons mentioned above

Proposal 1: Include the provided TP into the TR 38.858.

- References

- R4-2306005, “WF on implementation feasibility of SBFD”, Samsung, R4 #106bis-e, Online, April 2023.

- R4-2305917, “WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference”, Ericsson, R4 #106bis-e, Online, April 2023.

- 3GPP TR 38.858 V0.4.0 (2023-04): Study on Evolution of NR Duplex Operation (Release 18)

- Annex

TP to TR 38.858

< Start of TP to TR 38.858 >

10.2 Feasibility of FR1 Wide Area BS aspects

10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

#### 10.2.1.2	Feasibility study on self-interference

### 10.2.1.2.X	Nokia, Nokia Shanghai Bell

#### Frequency isolation techniques

The techniques to achieve sufficient frequency isolation may include:

- Transmitter digital filtering or windowing to clean the UL sub-band. This is required to clean the IFFT output of the linear leakage of the signal, otherwise the sinc spectrum of the IFFT will dominate the emissions on the UL sub-band. Requires new filter design with potentially tighter suppression requirements compared to the channel filter, due to the desire to minimize guard bands between DL and UL sub-bands.

  - Tighter filter suppression requirements may mean longer filter impulse response and lead to signal EVM degradation.

- Transmitter digital pre-distortion to linearize the transmit chain and suppress PA distortion components. Achieving the same performance for sub-band leakage ratio as for ACLR will be more challenging, as the UL sub-band is closer to the DL sub-band(s) than the adjacent channel. The ACLR is averaged over the same bandwidth as the DL channel, with emissions likely decaying somewhat with offset. For inter-sub-band leakage, the offset is generally small, indicating tighter DPD requirements for the same absolute level of emissions. Moreover, the DUD frequency configuration will be challenging due to spectral regrowth from both sides of the UL sub-band, compared to DU configuration or the ACLR case.

  - Higher DPD complexity translates to higher energy consumption.

  - Higher energy consumption leads to increased heating, worse PA performance and thermal management issues. This may require larger and heavier cooling solutions.

Techniques which have been considered, but have significant challenges:

- Analog sub-band filtering in the RF domain. It has been demonstrated by simulations, that in order to achieve meaningful inter-subband filtering, high insertion loss must be accepted.

  - High insertion loss before the LNA will increase the receiver noise figure, and negate any system gains of SBFD.

  - High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include:

    - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget.

    - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL

    - Group delay distortion close to the cut-off frequencies

    - Incompatibility with a typical multi-carrier gNB design

    - Increased complexity as switches are needed for by-pass in UL slots for full BW

    - Additional space needed in RX chain that is not available in typical gNB design

    - Overall additional power consumption which leads to thermal management issues

    - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance

  - The new sub-band specific filters would be operator’s spectrum specific and locking the spectrum configuration for any further changes or tuning. The existing bandpass filters for the operating band would anyway be required, suggesting that the new filters would double the filter size for the UL antenna panel. This all means higher cost and complexity.

#### Spatial isolation techniques

The techniques to achieve sufficient spatial isolation may include:

- Separate TX and RX antennas or antenna arrays. Increased separation from TX to RX will improve isolation.

  - To maintain the same or similar physical size of the antenna, the number of elements per array need to be halved. This reduces the achievable array gain by at least 3 dB in both link directions, and has been demonstrated by simulations to degrade the system performance.

  - To maintain baseline system performance, the number of antenna elements per array must be maintained, leading to an increased antenna size by at least 2x. This in turn means higher weight and wind load, increased complexity, increased trace losses which may need to be compensated, and in general higher cost.

  - Separate TX and RX antenna arrays requires separate PWBs for the TX and RX, leading to a higher cost.

  - Separate TX and RX antenna arrays leads to loss of reciprocity in the DL and UL channels, and makes reliable channel state measurements more difficult and complicated. The extent of this loss has not been studied.

- EM shielding techniques such as wave traps or chokes between the TX and RX arrays.

- Transmit beam nulling. The beamforming coefficients of the transmit beamforming may be modified so, that the energy coupled to the receive antenna elements is minimized.

  - The modification of TX coefficients reduces the transmitted EIRP toward the intended UE, leading to further reduced DL performance unless compensated by increased conducted power.

  - Based on simulations, some TX beams may be affected more than others, leading to potential scheduler restrictions in which UEs may be scheduled during the SBFD time slots.

  - The transmit beam nulling is most effective on the DL sub-band, for which the transmit signal is known and can be beamformed. It is not assumed that transmit beam nulling is effective on the UL sub-band, which contains only unwanted emission components at the transmit side.

#### RF IC techniques and other techniques (before LNA)

The following techniques have been evaluated, but have significant challenges in FR1 wide area base station implementations:

- Analog cancellation in the RF domain. This method may be used to subtract the unwanted coupled TX signal components from the RX signal path, before the LNA. With proper phase shift and amplitude scaling, the TX component may be canceled.

  - The number of cancellation paths in mMIMO base station can become excessively large. Every TX chain will couple to every RX chain, and correspondingly for complete cancellation, the number of cancellation paths becomes number of TRX2. For typical 32 TRX or 64 TRX implementations, the complexity becomes prohibiting.

#### Frequency isolation techniques after LNA

The techniques to improve frequency isolation after the LNA may include:

- Analog baseband filtering for UL sub-band. A zero-IF receiver may include a separate analog baseband lowpass filter to cover the UL sub-band only, for a DUD SBFD configuration. Based on the time slot (DL or SBFD), the receiver will switch between the normal analog filter and the UL sub-band specific filter.

  - This approach restricts multi-carrier operation of the base station. The UL sub-band specific filter allows only the reception of the UL sub-band during SBFD time slots.

#### Digital IC techniques

The following techniques have been evaluated, but have significant challenges in FR1 wide area base station implementations:

- Receive beam nulling. Similarly, as in the transmit beam nulling, modifying the RX beamforming coefficients may be used to minimize coupling from the TX beams to the RX beams.

  - However, the RX beamforming occurs only in the digital domain. There will not be any impact to analog and RF performance requirements.

- Digital cancellation. With knowledge of the TX signal, a properly scaled and phase shifted TX component may be subtracted from the RX signal to improve cancellation performance.

  - The TX signal may be available from observation receiver that is used in the DPD processing. For a mMIMO implementation, the DPD system may utilize only a few observation receivers, that sample the TX chains sequentially. It may be necessary to multiply the number of observation receivers to be able to sample each TX chain, leading to increased cost and energy consumption.

  - Each RX chain contains signals that are coupled from every TX chain. This means that the cancellation signal for each RX chain must be formed of every TX chains. The complexity can easily become extreme in a mMIMO implementation, with 32 or 64 TRXs. The complexity of the cancellation results in high energy consumption.

  - The cancellation may work with different performance for the DL signal fundamental components (i.e. the DL PRBs) than for the unwanted emission components (i.e. leakage on UL sub-band). The DL signal may be easier to cancel than the unwanted emissions. For the unwanted emissions, it is more efficient to cancel them at the TX DPD.

10.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

### 10.2.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

### 10.2.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

### 10.2.2.2.X	Nokia, Nokia Shanghai Bell

#### Frequency isolation techniques

The same frequency isolation techniques apply as for self-interference.

#### Spatial isolation assumptions

The spatial isolation mechanisms for co-site inter-sector case are in principle similar to the self-interference case. In addition, these aspects need to be considered:

- Element-to-element isolation is easier to manage within a single antenna enclosure, where all parameters and physical dimensions can be controlled. Isolation between sectors occurs due to unwanted radiation towards the back of the antenna, which is more difficult to control. The geometry between the antennas of different sectors can be difficult to adjust precisely, meaning that the element coupling can be difficult to predict.

- It has been suggested that EM shielding material between sectors may be used. This is not possible in all installations but may possibly be an option in some installations. The effectiveness of the EM shielding has not been studied.

- Transmit beam nulling across different sectors is theoretically possible, but the practical implementation may be too costly, since it may be necessary to calculate the beamforming vector for each subcarrier.

10.2.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

10.2.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

< End of TP >