3GPP RAN WG4 Meeting #107		R4-2309046

Seoul, Korea, May 22 – 26th, 2023

Agenda item:	7.32.3

Source:	Apple, Globalstar

Title:	RF requirements for the NTN L-/S-band

WI/SI:	NR_NTN_LSband

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2].

During the RAN#106 and RAN4#106bis meetings initial discussions took place and some basic agreements for the RF requirements were made, such as the maximum Tx power, basic MPR requirements, etc [4]. In this paper we present our further proposals for the RF requirements of the new NTN band.

# 2	RF requirements for the NTN L-/S-band

## 2.1	TX requirements

### 2.1.1	Emission requirements (in-band and out-of-band)

There are several regulatory documents that provide emission limits for the frequency ranges outside operational band of 1610-1626.5MHz. For 1559-1605MHz both FCC and ETSI define the emission limit of -70dBW/MHz (-40dBm/MHz). In addition to that, FCC specifies an additional requirement of -80dBW/700Hz (-50dBm/700Hz) for so-called discreet emissions. For the frequency range 1605- 1610MHz, both ETSI and FCC define a linear slope that goes from -70dBW/MHz up to -10dBW/MHz. And for the same frequency range FCC defines a similar slope for discrete emissions that goes from -80dBW/700Hz (-50dBm/700Hz) to -20dBW/700Hz (10dBm/700Hz). As for the frequencies above 1626.5MHz at least ETSI defines a step function going from -30dBm/30KHz down to -30dBm/3MHz.

The summary of the ETSI and FCC requirements can be captured as follows:

-	1559-1605MHz: there are two levels of -40dBm/MHz and -50dBm/700Hz (from ETSI and FCC)

-	1605-1610MHz: there is a slope from -40dBm/MHz to 20dBm/MHz (from ETSI and FCC)

-	1605-1610MHz: there is a slope from -50dBm/700Hz to 10dBm/700Hz (from FCC)

-	1628.5-2200MHz: there is a step function going from -30dBm/30KHz down to -30dBm/3MHz (from ETSI)

Proposal 1a:	Capture additional out-of-band emission requirements based on the FCC and ETSI requirements.

Table 2.1.2-1 and 2.1.2-2 below present how we can capture the corresponding FCC and ETSI requirements in TS 38.101-5.

Table 2.1.2-1: Additional out-of-band emission requirements based on FCC regulations.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 2.1.2-2: Additional out-of-band emission requirements based on ETSI regulations.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

In addition to the out-of-band emission requirements, at least ETSI regulations also define in-band emission limits which are logically separated into two frequency blocks: 1610-1618.25MHz and 1618.25-1626.5MHz. While both in-band emission masks set the lowest limit to -26dBm/30kHz, the latter mask has a steeper slope reaching the level of -26dBm/30kHz already at 1.8MHz offset from the wanted signal. It is also worth noting that in-band emission requirements 1618.25-1626.5MHz also apply to those cases when the channel is partially contained in that frequency range, for instance when the configured channel spans over 1610-1618.25MHz and 1618.25-1626.5MHz boundary.

Proposal 1b:	Capture additional in-band emission requirements based on the ETSI requirements for 1610-1618.25MHz and 1618.25-1626.5MHz frequency ranges.

Table 2.1.2-3: Additional in-band emission requirements for 1610-1618.25MHz.

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE: Spectrum emissions are linearly interpolated in dBm versus frequency offset. |  |  |

Table 2.1.2-4: Additional in-band emission requirements for 1618.25-1626.5MHz.

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE: Spectrum emissions are linearly interpolated in dBm versus frequency offset. |  |  |

### 2.1.2	MPR/A-MPR

During the RAN4#106bis meeting it was agreed that the basic NTN band MPR requirements would also apply to the NTN L-/S-band. As can be seen from the corresponding table below, depending on the waveform and MCS, different MPR requirements will apply allowing a UE to back-off, if needed, its maximum transmission power.

Table 2.1.2-1: Maximum power reduction (MPR) for power class 3.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

As already elaborated in section 2.1.1 above, the NTN L-/S-band UL block is close the GNSS services, for which both ETSI and FCC introduce quite strict requirements on the out-of-band emissions. To ensure that all requirements are met – in fact not only out-of-band but also in-band emission requirements – we conduct a series of simulations to estimate power back-off that could be needed depending on the channel location, size, allocated RBs, etc. All simulations are run with the same common parameters as follows:

-	Tx power: PC3 +23dBm

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI

It should be noted that the reason why we decided to choose the ETSI regulations for the power back-off simulations is because ETSI has stricter in-band emission requirements and have out-of-band emission requirements for frequencies above 1626.5MHz. In other words, our view is that power back-off values for the FCC regulatory domain will be lower (but that will also require additional simulations).

The only varying parameter is the channel size and its centre frequency location as presented below:

-	NR 5MHz channel at the following centre frequencies:

a)	Fc=1612.5MHz

b)	Fc=1615.5MHz

c)	Fc=1620.5MHz

d)	Fc=1624MHz

-	NR 10MHz channel at the following centre frequencies:

a)	Fc=1615MHz.

b)	Fc=1621.5MHz

-	NR 15MHz channel at the following centre frequencies:

a)	Fc=1617.5MHz

b)	Fc=1619MHz

Appendix A, B and C contain the corresponding power back-off figures for different NR channels and centre frequencies, summary for which is presented below:

-	NR channels placed at the leftmost edge of the band might need power back-off, exact value of which depends on the number of the scheduled RBs and their location. As somewhat anticipated, RBs scheduled at the centre of the NR channel do not require any power back-off; but RBs scheduled at the leftmost edge of the configured NR channel do require power back-off to meet out-of-band emission requirements. Similarly, power back-off is needed when all RBs are scheduled to mitigate emissions coming from the full-channel transmissions. It should be noted that some configuration cases require power back-off that exceeds basic MPR requirements, and thus A-MPR is needed.

-	In case of the 5MHz channel, once the channel is shifted to the right, i.e. farther from the left edge of the band where the GNSS services are, the required power back-off goes down a lot and is caused mostly by the in-band emission requirements. For instance, no A-MPR is needed in this case for the 5MHz channel irrespective of the RBs number and their location. However, the 5MHz channel placed at the rightmost edge of the band may need some A-MPR for the RBs scheduled at the right edge of the channel.

-	As for the 10 and 15MHz channels, even if we shift the channel to the right from the leftmost band edge, some configuration cases would still require A-MPR. As in cases for other channels at the band edge, A-MPR is needed mostly for the case when all RBs are scheduled.

Proposal 2a:	Capture power back-off simulation results in the NTN L-/S-band TR.

Referring to the presented power back-off results it is possible to notice that DFT-s-OFDM require much lower values, whereupon PI/2 BPSK perform even 0.5-1dB better comparing to QPSK. However, PI/2 BPSK is not enabled for the NTN band in the current version of TS 38.101-5.  Thus, we suggest considering enabling the PI/2 BPSK modulation also for the NTN bands.

Proposal 2b:	Consider allowing PI/2 BPSK for the NTN bands.

### 2.1.3	NS values

Based on the analysis of the existing regulations and presented power back-off values, our view is that several NS flags are needed summary for which is presented in Table 2.1.3-1 below. As can be seen from the table, the main differentiating factor is FCC and ETSI regulatory domains. As explained in section 2.1.1, even though FCC and ETSI out-of-band emission requirements are similar, they are not the same; and ETSI regulations have stricter in-band emission requirements. Furthermore:

a)	ETSI regulations have different in-band emission masks depending on the operating sub-range, so two NS flags are needed to differentiate between them.

b)	NS_03N applicable range is limited to 1610-1618.25MHz according to the ETSI requirements. And since the total spectrum block size is 8.25MHz, it can accommodate only 5MHz standard NR channel, i.e. the applicable NR channel size is 5MHz.

c)	NS_04N is for the ETSI range 1618.25-1626.5MHz, which can also accommodate only 5MHz standard NR channel. However, ETSI document also clarifies that the same in-band emission requirements will apply if the channel is partially configured in that range. It means that any 10 and 15MHz channel configured in 1610-1626.5MHz range will inevitably fall into the 1618.25-1626.5MHz range and thus they will follow NS_04N requirements.

d)	NS_05N is for the FCC regulatory domain. And since FCC does not introduce any special in-band emission requirements for sub-ranges, same requirements will be applicable for the whole range and for all the channel sizes.

Table 2.1.3-1: Summary of the proposed NS values.

| NS value | Regulatory domain | Applicable range (MHz) | Applicable channel (MHz) | In-band emissions | Out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| NS_03N | ETSI | 1610-1618.25 | 5 | ETSI (1610-1618.25) | ETSI (1559-1610MHz) ETSI (>1628.5MHz) |
| NS_04N |  | 1618.25-1626.5 | 5 | ETSI (1618.25-1626.5) |  |
|  |  | 1610-1626.5 | 10, 15 |  |  |
| NS_05N | FCC | 1610-1626.5 | 5, 10, 15 | FCC (1610-1626.5) | FCC (1559-1610MHz) |

Proposal 3a:	Specify two NS flags to support ETSI regulations for the NTN L-/S-band (differentiating between different in-band emission requirements in 1610-1618.25MHz and 1618.25-1626.5MHz frequency ranges).

Proposal 3b:	Specify NS flag to support FCC regulations for the NTN L-/S-band.

### 2.1.4	UE co-existence emission requirements

A new NTN band [254] UL is right next to the UL of another NTN band n255. Purely from that perspective UE co-existence emission requirements for band [254] can be based on band n255. Nevertheless, there are certain differences for a new band [n254] that are summarised as follows:

-	NR bands n77 and n78 are moved to the "main" list of protected bands;

-	NR bands n54, n104 and n105 are added;

-	E-UTRA bands 31, 54, 72, 73, 87, 88, 103 are added.

As a related topic, we suggest discussing further in RAN WG4 whether we need to add NTN bands [254], n255 and n256 to the list of protected bands. As an example, a new NTN band should also protect n255 and n256, and similarly NTN bands n255 and n256 should be protecting [n254]. In fact, both NTN bands n255 and n256 already have the NR band n53 on the list, which implicitly protects [n254], but for the sake of consistency the 3GPP specifications use to mention all relevant bands.

Proposal 4a:	UE co-existence requirements can be based on existing requirements for band n255.

Proposal 4b:	Additional NR bands n54, n104 and n105 as well as E-UTRA bands 31, 54, 72, 73, 87, 88, 103 are added to the list of protected bands.

Proposal 4c:	To be discussed further whether we need to add NTN bands [n254], n256 and n255 into the list of protected bands.

The corresponding change in the TS 38.101-5 specification will look as presented in the table below.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| [n254] | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n54, n65, n66, n67, n70, n71, n74, n75, n76, n77, n78, n85, n90, n91, n92, n93, n94, n100, n101, n105, n255, n256 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 31, 54, 72, 73, 87, 88, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79, n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n255 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n90, n91, n92, n93, n94, n100, n101, n254 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n256 | NR Band n1, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n26, n28, n29, n30, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n71, n74, n75, n76, n78, n79, n85, n90, n91, n92, n93, n94, n101, n100, n101, n254 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 33, 35, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |

## 2.2	Rx requirements

### 2.2.1	Reference sensitivity level

Table 2.2.1-1 below present a small summary of the 5MHz REFSENS values for several TN/NTN bands at 2.5GHz. For instance, band n53, which has the same frequency range as the new NTN band, has REFSENS of -100dBm. Band n41 has a higher REFSENS values because of its frequency range. And band n7, which occupies same frequency range as band n41, has 0.5dB higher value because it is the FDD band. Thus, the reference sensitivity level for the new NTN band can be devised leveraging existing values specified for the terrestrial band n53 (DL frequency range at 2483.5-2497MHz). Since the new NTN band is FDD, additional 0.5dB insertion loss can be added resulting in -99.5dBm. In fact, this value is perfectly aligned with another NTN band n256, which is also the FDD band operating in the same frequency range.

Table 2.2.1-1: Summary of REFSENS values for the 5MHz NR channel of TN/NTN bands at 2.5GHz.

| Band | Type | DL range | REFSENS |
| --- | --- | --- | --- |
| n256 | NTN FDD | 2170-2200MHz | -99.5dBm |
| n53 | TN TDD | 2483.5-2495MHz | -100dBm |
| n41 | TN TDD | 2496-2690MHz | -98.5dBm |
| n7 | TN FDD | 2620-2690MHz | -98dBm |

Proposal 5:	Define REFSENS values as band n53 REFSENS with 0.5dB insertion loss (i.e. same as for the NTN band n256).

Based on the proposal 4, table below presents REFSENS values for other NR channel bandwidths and SCS.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| [n254] | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

### 2.2.2	In-band and out-of-band blocking requirements

The NTN bands n255 and n256 re-use existing in-band blocking requirements already defined for the terrestrial bands operating in the same frequency range. Thus, we suggest following the same approach and apply the same in-band blocking requirements also for the new NTN band.

Proposal 6a:	Existing in-band blocking requirements can be applied to the new NTN band.

As for the out-of-band blocking requirements, there exist out-of-band blocking requirements for the terrestrial band n53 defined for almost the same frequency range as the new NTN band (see Figure 2.2.2-1 below for the graphical representation of these requirements). Since from the viewpoint of the UE blocking requirements it does not fundamentally matter whether the wanted signal comes from the terrestrial or the satellite base station, it is possible to re-use existing band n53 requirements with the only difference is that the upper band edge is updated to 2500MHz.

Figure 2.2.2-1: Summary of the band n53 out-of-band blocking requirements.

Proposal 6b:	Existing band n53 out-of-band blocking requirements (with the changed upper edge to 2500MHz) can be applied to the new NTN band.

The table below is the corresponding excerpt from the draft running CR illustrating how the out-of-band blocking requirements can be implemented in the specification.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| [n254]2 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations void NOTE 2: Band [n254] power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2580 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

# 3	Conclusions

In this paper we have presented our further considerations on the RF requirements covering both Tx and Rx aspects. The corresponding changes to TS 38.101-5 can be found in [5].

Proposal 1a:	Capture additional out-of-band emission requirements based on the FCC and ETSI requirements.

Proposal 1b:	Capture additional in-band emission requirements based on the ETSI requirements for 1610-1618.25MHz and 1618.25-1626.5MHz frequency ranges.

Proposal 2a:	Capture power back-off simulation results in the NTN L-/S-band TR.

Proposal 2b:	Consider allowing PI/2 BPSK for the NTN bands.

Proposal 3a:	Specify two NS flags to support ETSI regulations for the NTN L-/S-band (differentiating between different in-band emission requirements in 1610-1618.25MHz and 1618.25-1626.5MHz frequency ranges).

Proposal 3b:	Specify NS flag to support FCC regulations for the NTN L-/S-band.

Proposal 4a:	UE co-existence requirements can be based on existing requirements for band n255.

Proposal 4b:	Additional NR bands n54, n104 and n105 as well as E-UTRA bands 31, 54, 72, 73, 87, 88, 103 are added to the list of protected bands.

Proposal 4c:	To be discussed further whether we need to add NTN bands [n254], n256 and n255 into the list of protected bands.

Proposal 5:	Define REFSENS values as band n53 REFSENS with 0.5dB insertion loss (i.e. same as for the NTN band n256).

Proposal 6a:	Existing in-band blocking requirements can be applied to the new NTN band.

Proposal 6b:	Existing band n53 out-of-band blocking requirements (with the changed upper edge to 2500MHz) can be applied to the new NTN band.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

ETSI EN 301 441, "Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements
of article 3.2 of the Directive 2014/53/EU", v2.1.1, June 2016

R4-2306561, "WF on NTN L-/S-band", Apple

R4-2304332, "Introduction of the NTN L-/S-band", Apple, Globalstar

Appendix A: Power back-off simulation results for 5MHz

Figure A-1: Power back-off values for the 5MHz channel at Fc=1612.5MHz.

Figure A-2: Power back-off values for the 5MHz channel at Fc=1615.5MHz.

Figure A-3: Power back-off values for the 5MHz channel at Fc=1620.5MHz.

Figure A-4: Power back-off values for the 5MHz channel at Fc=1624MHz.

Appendix B: Power back-off simulation results for 10MHz

Figure B-1: Power back-off values for the 10MHz channel at Fc=1615MHz.

Figure B-2: Power back-off values for the 10MHz channel at Fc=1621.5MHz.

Appendix C: Power back-off simulation results for 15MHz

Figure C-1: Power back-off values for the 15MHz channel at Fc=1617.5MHz.

Figure C-2: Power back-off values for the 15MHz channel at Fc=1619MHz.