3GPP RAN WG4 Meeting #107		R4-2309092

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	7.33.3

Source:	Apple

Title:	REFSENS issue for FDD bands with n28 UL and n75/n76 DL

WI/SI:	NR_FDD_ULn28_DLn75_n76-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

In RAN #99 meeting, a new spectrum work item for FDD bands based on (SUL + SDL) configuration was approved [1]. The intended new bands are combining n28 UL and n75/n76 DL for FDD operation. In the WID objective, it proposes using the entire n28 UL frequency range from 703MHz to 748MHz for the new bands, and the RF requirements should be derived based on the assumption that UE supports the corresponding band combination(s), i.e., CA_n28-n75 and CA_n28-n76. However, in the current RAN4 specifications [2], CA_n28-n76 has not yet been introduced, and CA_n28-n75 has been specified with n28 frequency range restricted to 703MHz – 733MHz for UL and 758MHz – 788MHz for DL. Therefore, clarifications are needed on whether CA_n28-n76 should be introduced first before the normative work for the corresponding FDD band can be started and also whether the n28 UL should use the entire band range for the intended FDD bands or the restricted range as specified for CA_n28-n75. On the other hand, irrespective of using the entire n28 UL range or only the restricted range, n28 UL 2nd harmonic can both fall into n75 and n76 to cause sensitivity degradation where the impacted frequency range could potentially be relatively wide. Since this type of self-interference in a single band is unprecedented in 3GPP, how to handle the REFSENS impact issue and specify the requirement needs to be addressed.

# Discussion

For the new FDD bands which combine the n28 UL and n75/n76 DL as proposed in a recently approved Rel-18 spectrum work item [1], the WID objective proposes using the entire n28 UL frequency range from 703MHz to 748MHz for the new bands, and the RF requirements should be derived based on the assumption that UE supports the corresponding band combination(s), i.e., CA_n28-n75 and CA_n28-n76. However, CA_n28-n76 has never been introduced in the past, and CA_n28-n75 has been specified with n28 frequency range restricted to 703MHz – 733MHz for UL and 758MHz – 788MHz for DL [2]. Therefore, clarifications are needed on whether CA_n28-n76 should be introduced first before the normative work for the corresponding FDD band can be started and also whether the n28 UL should use the entire band range for the intended FDD bands or the restricted range as specified for CA_n28-n75.

Proposal 1: Clarification is needed on whether CA_n28-n76 should be introduced first before the normative work for the corresponding FDD band can be started.

Proposal 2: Clarification is needed on whether the n28 UL should use the entire band range for the intended FDD bands or the restricted range as specified for CA_n28-n75.

On the other hand, irrespective of using the entire n28 UL range or only the restricted range, n28 UL 2nd harmonic can both potentially fall into n75 and n76 to cause sensitivity degradation, as shown in Table 2-1.

The concern with the UL 2nd order harmonic is that not only the interference power level is relatively high where the resulted MSD with direct hit could be higher than 30 dB without harmonic filter, the interference spectrum range also expands substantially as can be seen by comparing the spectral plots of a 20MHz fundamental signal and its 2nd order harmonic in Figure 2-1a and Figure 2-1b respectively.

| n28 UL Range (MHz) | n28 UL 2nd harmonic range (MHz) | n75 DL range (MHz) | n76 DL range (MHz) |
| --- | --- | --- | --- |
| 703 – 748 (full) | 1406 – 1496 | 1432 – 1517 | 1427 – 1432 |
| 703 – 733 (block A) | 1406 – 1466 | 1432 – 1517 | 1427 – 1432 |

Table 2-1 n28 UL 2nd harmonic frequency range and n75/n76 frequency ranges

Figure 2-1a Spectral profile of fundamental signal with 20 MHz carrier bandwidth

Figure 2-1b Spectral profile of 2nd harmonic for the above 20MHz carrier

Table 2-2 summarizes the 2nd harmonic side-lobe attenuation requirements for different UL carrier bandwidth in order for the induced REFSENS impact to be less than 1 dB, as had been analyzed in [3].

| dB | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| 2nd | 39 | 36 | 34 | 33 |

Table 2-2 UL 2nd harmonic side-lobe attenuation requirements for different UL carrier bandwidth

The above requirements would translate into the frequency exclusion ranges as summarized in Table 2-3 for DL carrier where REFSENS impact is expected to be more than 1 dB.

| MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| 2nd | 30 | 55 | 80 | 110 |

Table 2-3 Frequency exclusion range within which REFSENS impact is more than 1 dB

Due to the relatively wide frequency exclusion ranges, there is a high probability for n75 and n76 DL carrier REFSENS to be affected by n28 UL 2nd harmonic. The REFSENS impact level could range from sub-1 dB to over 30 dB, depending on how much overlap of the UL 2nd harmonic (including the side-lobe) with the DL carrier. As a result, it would not be straightforward to specify the REFSENS requirements for the intended new FDD bands.

Observation 1: For the FDD bands by combining n28 UL and n75/n76 DL, there is a high probability for n75 and n76 DL carrier REFSENS to be affected by n28 UL 2nd harmonic.

Observation 2: For the FDD bands by combining n28 UL and n75/n76 DL, the REFSENS impact level could range from sub-1 dB to over 30 dB, depending on how much overlap of the UL 2nd harmonic (including the side-lobe) with the DL carrier.

Observation 3: It would not be straightforward to specify the REFSENS requirements for the intended new FDD bands by combining n28 UL and n75/n76 DL.

Since this type of self-interference in a single band is unprecedented in 3GPP, how to handle the REFSENS impact issue and specify the corresponding requirement needs to be addressed.

Proposal 3: For the intended new FDD bands by combining n28 UL and n75/n76 DL, how to handle the REFSENS impact caused by UL 2nd harmonic and specify the corresponding requirement needs to be addressed.

# 3	Conclusion

In this contribution, we raise three potential issues for the intended new FDD bands by combining n28 UL and n75/n76 DL, including whether CA_n28-n76 should be introduced first before the normative work for the new frequency band with n76 DL, the inconsistency of n28 UL frequency range between CA_n28-n75 and the WID objective, and how to handle the REFSENS impact caused by UL 2nd harmonic and specify the corresponding requirement.

Proposal 1: Clarification is needed on whether CA_n28-n76 should be introduced first before the normative work for the corresponding FDD band can be started.

Proposal 2: Clarification is needed on whether the n28 UL should use the entire band range for the intended FDD bands or the restricted range as specified for CA_n28-n75.

Observation 1: For the FDD bands by combining n28 UL and n75/n76 DL, there is a high probability for n75 and n76 DL carrier REFSENS to be affected by n28 UL 2nd harmonic.

Observation 2: For the FDD bands by combining n28 UL and n75/n76 DL, the REFSENS impact level could range from sub-1 dB to over 30 dB, depending on how much overlap of the UL 2nd harmonic (including the side-lobe) with the DL carrier.

Observation 3: It would not be straightforward to specify the REFSENS requirements for the intended new FDD bands by combining n28 UL and n75/n76 DL.

Proposal 3: For the intended new FDD bands by combining n28 UL and n75/n76 DL, how to handle the REFSENS impact caused by UL 2nd harmonic and specify the corresponding requirement needs to be addressed.

# 4	References

RP-230791 “New WID for new FDD Bands using the uplink from n28 and the downlink of n75 and n76” Vodafone, 3GPP TSG RAN Meeting #99, March 20th – 23rd, 2023

3GPP TS 38.101-1 V18.1.0 (2023-03)

R4-168285 “B46 REFSENS test frequency exclusion range”, MediaTek Inc., 3GPP TSG RAN WG4 Meeting #80bis, Ljubljana, Slovenia, October 10th – 14th, 2016