3GPP TSG-RAN WG4 Meeting #107	R4-2309132

Incheon KR, May 22 – 26, 2023

Agenda item:	8.23.3.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for 1Rx RedCap UEs without frequency hopping – reduced number of samples

Document for:	Information

# Introduction

RAN4 revised the simulation assumptions for PRS measurement accuracy for 1Rx RedCap UEs without frequency hopping [1]. This paper includes simulation results with reduced number of samples for the following measurements:

- UE Rx-Tx time difference

- PRS-RSRP

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
|  | 52 | 15 | 128 | 1 | 1 | 1 | 57 | 58 |
| FR1 |  |  |  |  |  | 2 | 57 | 57 |
|  | 104 |  | 64 | 1 | 1 | 1 | 30 | 31 |
|  |  |  |  |  |  | 2 | 28 | 31 |
|  | 48 | 30 | 64 | 1 | 1 | 1 | 28 | 30 |
|  |  |  |  |  |  | 2 | 28 | 30 |

Table 1: UE Rx-Tx simulation results for FR1 in AWGN.

# Simulation results for PRS-RSRP

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
|  | 52 | 15 | 128 | 1 | 1 | 1 | 0.5 | 0.9 |
| FR1 |  |  |  |  |  | 2 | 0.4 | 0.6 |
|  | 104 |  | 64 | 1 | 1 | 1 | 0.3 | 0.7 |
|  |  |  |  |  |  | 2 | 0.3 | 0.5 |
|  | 48 | 30 | 64 | 1 | 1 | 1 | 0.5 | 0.9 |
|  |  |  |  |  |  | 2 | 0.4 | 0.7 |

Table 2: PRS-RSRP simulation results for FR1 in AWGN.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
|  | 52 | 15 | 128 | 1 | 1 | 1 | 66 |
| FR1 |  |  |  |  |  | 2 | 66 |
|  | 104 |  | 64 | 1 | 1 | 1 | 38 |
|  |  |  |  |  |  | 2 | 38 |
|  | 48 | 30 | 64 | 1 | 1 | 1 | 38 |
|  |  |  |  |  |  | 2 | 38 |

Table 3: RSTD simulation results for FR1 in AWGN.

# References

[1] R4-2306350, Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH, RAN4#106bis-e