3GPP TSG-RAN WG4 Meeting #107	R4-2309134

Incheon KR, May 22 – 26, 2023

Agenda item:	8.23.3.5

Source: 	Qualcomm Incorporated

Title: 	On requirements for PRS/SRS BW aggregation

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for PRS/SRS bandwidth aggregation in RAN4#106bis-e. The latest agreements and open issues were captured in a WF [1].

In this brief paper we provide our views on the progress in RAN1 and provide high-level proposals on how to define the measurement period requirement.

# Discussion

RAN1 has reached further agreements on the objective of PRS/SRS BW aggregation [2]. Selected agreements are shown below.

Agreement

For PRS bandwidth aggregation across PFLs, select one of the following options in RAN1#113

- Option 2: Per TRP basis and per PRS resource set basis.

  - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked.

  - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied.

- Option 3: Per TRP basis and per PRS resource basis.

  - For each TRP, support new signaling to indicate which PRS resource(s) across PFLs are linked.

  - For the non-linked PRS resources, no aggregation is assumed even if the conditions are satisfied.

Agreement

Support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state.

- FFS the details

Agreement

For PRS resources aggregated across PFLs for DL-TDOA and multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 DL PRS measurement of single PFL with the necessary update.

- FFS: In a measurement report element, single RSRP or single RSRPP is reported

- In a measurement report element, PFL aggregation indication is supported to indicate whether/which measurement is aggregated

- Support new signaling in location information request message to indicate UE whether to perform joint measurement across aggregated PFLs

- Single RSTD reference in assistance data and measurement report is used for PRS bandwidth aggregation measurement

  - FFS RSTD reference is aggregated or not

Agreement

For SRS bandwidth aggregation between SRS in two or three carriers, the aggregated SRS resources are of the same SRS resource-Type.

Agreement

At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA.

Agreement

Introduce new UE capability(-ies) to support PRS bandwidth aggregation measurement

- FFS the details include the processing capability (N, T), the maximum number of PRS resources that can be process in a slots over the aggregation

- FFS the details on the PFL bandwidth combinations, including maximum number of PFLs, the total aggregated bandwidth, etc.

- This is applicable for DL-TDOA and Multi-RTT positioning methods

Agreement

Positioning SRS bandwidth aggregation is supported for UEs in RRC_CONNECTED.

Positioning SRS bandwidth aggregation is supported for UEs in RRC_INACTIVE state.

- For the details, Rel-17 positioning SRS configuration for UE in RRC_INACTIVE state outside initial UL BWP can be the starting point

Agreement

For the case when PRS in one of aggregated PFL is dropped, e.g. because of collision with SSB, select one of the following solutions for LMF based positioning

- Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s)

- Alt. 2: Still perform positioning measurement based on the remaining PRSs in other PFL(s)

- FFS the details and the difference between MG and PPW if PPW is supported

- Note: Up to RAN4 to discuss impact on requirements, if any, for such cases

Agreement

For SRS bandwidth aggregation across two or three carriers, select one of the following options in RAN1#113 meeting

- Option 2: Per SRS resource set basis.

  - Support new signaling to indicate which SRS resource sets across carriers are linked.

  - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied.

- Option 3: Per SRS resource basis.

  - Support new signaling to indicate which SRS resources across carriers are linked.

  - For the non-linked SRS resources, no aggregation is assumed even if the conditions are satisfied

Agreement

For the SRS resources across aggregated carriers for UL-TDOA and Multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 SRS measurement of single carrier with the necessary update

- FFS: Single RSRP or RSRPP is reported for the SRS resources across aggregated carriers

- SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which measurement is aggregated

Agreement

For positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, select one of the following two options:

- Alt. 1: Stop SRS transmission in all aggregated carriers in the same symbol

- Alt. 2: SRS is still transmitted in other carriers in the same symbol

  - FFS: The UE may not be expected to maintain phase continuity across the remaining carriers

- FFS the applicable scenario, e.g. the positioning SRS collides with another higher priority SRS or others

There are still many unanswered questions in the agreements made by RAN1. Overall, we still think that more progress is needed before RAN4 has all the elements needed to specify the measurement requirements. However, already we anticipate some impact to the requirements based on the current RAN1 agreements.

One aspect that will impact the PRS measurement period requirement is the granularity of the indication signaled by the LMF to the UE, requesting PRS measurements with BW aggregation. According to the first agreement above, RAN1 will down-select between two options that both include indication to aggregate per TRP. The difference between the options is whether the indication is provided per resource set or per resource. With either option, in each PFL there could be some resources that are meant to be aggregated with resources in other PFLs and others that are not aggregated.

Observation 1: All the signalling options being considered by RAN1 to request PRS BW aggregation allow a mix of PRS resources to be measured with and without aggregation within one PFL.

Based on the above, the measurement period requirement within gaps would need to be modified to account for such mix of resources within each PFL. The Rel-17 measurement period requirement is the summation of the measurement delay contribution of each PFL, without any aggregation across PFLs. Now, for PRS BW aggregation in Rel-18, additional delay terms would be added to the summation for each group of two or three PFLs (a super-PFL) that contains PRS resources to be aggregated.

Issue 3-2-3: PRS measurement period for PRS/SRS bandwidth aggregation:

Agreements:

- PRS measurement period for PRS/SRS bandwidth aggregation:

- Option 1:

  - Existing PRS measurement period requirements in Rel-17 can be used as baseline for defining corresponding PRS measurement period requirements for PRS/SRS bandwidth aggregation.

    - Details related to e.g. PFLs, PRS periodicity etc., are FFS

- Other options are not precluded.

Proposal 1: To define the PRS measurement period requirement for RSTD and UE Rx-Tx when the LMF requests the UE to aggregate PRS resources across groups of PFLs (super-PFLs), as a baseline, the measurement period can be defined as a summation of two terms:

- The first term equals the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated, according to the LMF request and applicability conditions, are counted in Lavailable_PRS,i for each PFL.

- The second term is the summation of the measurement delays for the super-PFLs. As a baseline, only PRS resources to be aggregated according to the LMF request and applicability conditions, are counted in  Lavailable_PRS,i for each super-PFL.

- FFS whether any additional modifications to the first term are needed due to new capabilities defined by RAN1 for PRS BW aggregation.

- FFS the exact formula for the delay for each super-PFL based on RAN1 agreements on UE capabilities, etc.

- FFS how to handle the scenario where the PRS RSTD reference is not aggregated

- FFS any changes to the measurement requirement for PRS-RSRP and/or PRS-RSRPP when reported as supporting measurements for DL-TDOA and multi-RTT

Proposal 2: Wait for further progress in RAN1 to finalize the definition of the measurement period requirement.

In RAN4#106bis-e, RAN4 discussed impact of PRS/SRS BW aggregation to DL/UL communication. For DL in connected mode, RAN4 agreed that no additional impact is expected beyond the existing requirements within measurement gaps. For RRC_INACTIVE, RAN4 agreed to wait for RAN1 to define prioritization of PRS vs. other DL signals/channels.

Issue 3-1-4: PRS/SRS bandwidth aggregation with and without CA for communication:

Agreements

- Scenarios (if any) in which the CA for communication can impact the ongoing PRS measurements, which are performed in measurements gaps:

  - The PRS aggregation (DL part) across the PFLs within gaps RRC connected neither impacts the CA for communication and nor it is impacted by the CA for communication.

  - Any impact of the CA for communication on the SRS aggregation (UL part) is up to RAN1 discussion.

Issue 3-2-5: Priority between PRS measurement and CA for communication in RRC inactive:

Agreements:

- Wait for RAN1 agreement on priority between the PRS measurements and data communication in RRC_INACTIVE state.

For SRS aggregation in connected mode, RAN1 has agreed that transmission of positioning SRS for aggregation is decoupled from ULCA from the perspective of capability. i.e. a UE could support positioning SRS for aggregation and not support ULCA in a given band. The implication is that a UE may be configured to transmit SRS outside of UL carriers/BWP. In our view, RAN4 needs to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation.

Proposal 3: RAN4 to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation.

# Conclusions

Observation 1: All the signalling options being considered by RAN1 to request PRS BW aggregation allow a mix of PRS resources to be measured with and without aggregation within one PFL.

Proposal 1: To define the PRS measurement period requirement for RSTD and UE Rx-Tx when the LMF requests the UE to aggregate PRS resources across groups of PFLs (super-PFLs), as a baseline, the measurement period can be defined as a summation of two terms:

- The first term equals the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated, according to the LMF request and applicability conditions, are counted in Lavailable_PRS,i for each PFL.

- The second term is the summation of the measurement delays for the super-PFLs. As a baseline, only PRS resources to be aggregated according to the LMF request and applicability conditions, are counted in  Lavailable_PRS,i for each super-PFL.

- FFS whether any additional modifications to the first term are needed due to new capabilities defined by RAN1 for PRS BW aggregation.

- FFS the exact formula for the delay for each super-PFL based on RAN1 agreements on UE capabilities, etc.

- FFS how to handle the scenario where the PRS RSTD reference is not aggregated

- FFS any changes to the measurement requirement for PRS-RSRP and/or PRS-RSRPP when reported as supporting measurements for DL-TDOA and multi-RTT

Proposal 2: Wait for further progress in RAN1 to finalize the definition of the measurement period requirement.

Proposal 3: RAN4 to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation.

# References

[1] R4-2306349, WF on RRM requirements for RedCap positioning and PRS/SRS BW aggregation, RAN4#106bis-e

[2] Session notes for 9.5 Expanded and Improved NR Positioning, RAN1#112bis-e