3GPP TSG-RAN WG4 Meeting #107	R4-2309214

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	8.14.4.2

Source:	Ericsson

Title:	Discussions on ATG mobility requirements

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss the open issues of IDLE mode and CONNECTED mode mobility requirements based on the outcome of last meeting which were summarized in [1].

# 2	Discussion

## 2.1	Mobility in RRC_IDLE/INACTIVE

IDLE/INACTIVE mode neighbour cell measurements

Regarding the neighbour cell measurements, following options were identified at last meeting [1]:

Issue 2-1-1-1: Cell re-selection mechanism - Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation. - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority. - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection. - Option 4: - If ATG UE altitude higher than certain threshold, ATG UE perform measurement - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement - Option 5: No need to specify distance, altitude or speed-based enhancements to cell re-selection mechanism during Release 18. - If needed, such mechanisms may be discussed in future releases. Agreements: Candidate options for FFS: - Option 1 - Option 1+4 - Option 5

It shall be noted that the legacy IDLE/INACTIVE state neighbour cell measurement procedures were designed for classical terrestrial network which has important differences compared to the ATG network. For example, the ATG cell size is significantly larger compared to classical terrestrial cell size and also the UE mobility behaviour is also different (e.g. the aircraft typically follows a predefined path while a handheld devices moves with a less speed and in any direction). Therefore, reusing the legacy neighbour cell mechanism may lead to unnecessary complication and redundancy. Thus, we share the view with other companies that the there is no need to frequently measure on the neighbour cells as cell changes may not happen frequently as in classical terrestrial operation. The measurements can be skipped based on the received or detected assistance information or based on the distance between the reference location of the serving cell and the position of the UE (note that ATG UE is GNSS capable). This way the ATG operation can be simplified a lot. Therefore, we support option 1.

In addition, we also support option 4 which is especially useful in CONNECTED mode where the UE identifies, measures and reports the result of the measurement to the serving network. Such frequent measurement procedures are not really needed at lower altitude, e.g. when the aircraft is taking off or landing or parked at the ground. This type of neighbour cells are needed when the aircraft is flying at high altitude which means the speed is also expected by very high at this stage and it is therefore more likely to detect new cells. Hence, we also support option 4.

Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

    - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

    - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

    - If ATG UE altitude higher than certain threshold, ATG UE perform measurement

    - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement

- Cell reselection measurement requirements

It was agreed to define two set of requirements for ATG with following assumptions on the multicarrier requirements [1]:

Issue 2-1-1-4: Cell re-selection measurement requirements Agreements: - Keep previous agreements for number of carriers to be monitored. - For intra-frequency measurement, apply legacy R15 cell-reselection requirement - For inter-frequency measurement, - Define two set of requirements for ATG - Set 1: legacy R15 cell-reselection requirement - Set 2: R16 HST cell-reselection requirement - Details of signalling, associated UE capabilities and how to switch between the two sets of requirements are FFS - For set 2, FFS whether to use only R17 HST cell detection delays or measurement and evaluation delays as well.

RAN4 needs to discuss the criteria associated with first and second set of requirements in the above agreements, e.g. under what conditions is the ATG UE required to meet the set 1 requirements and set 2 requirements respectively. Another issue that needs discussions is the signalling need.

Our interpretation of the above agreements is that UE determines which requirements to meet based on the indication received from the serving network node.

The difference between the set 1 and set 2 requirements is that set 2 requirements are more stringent than set 1 requirements, e.g. UE would need to identify, measure and evaluate much faster in set 2 compared to set 1 since the UE is moving at high speed. Our view is that set 2 requirements shall include both cell detection and measurements (including serving cell evaluation).

In the legacy NR requirements, a high-speed indicator (highSpeedMeasFlag-r16) is transmitted by the NW to the UE based on which the UE meets the high-speed requirements. Examples of questions that need discussions is why the ATG UE under high speed is required to meet stringent requirements compared to ATG UE not operating under high-speed? In the former case, since the UE is flying over a predefined path with relatively fixed speed it may be sufficient to measure on the serving cell and initiate the neighbour cell measurements in set 1. Following the similar method, RAN4 needs to discuss the need for an ATG indicator informing the UE about which set of requirements to meet.

Proposal 3: RAN4 to request RAN2 to define ATG indicator (via system information) which is used to determine which set (set 1 or set 2) of requirements to meet.

- UE meets set 2 requirements when ATG indicator is received.

- Otherwise, UE meets set 1 requirements.

Proposal 4: Set 2 requirements comprises both R17 HST cell detection, measurements and evaluation requirements.

The above proposals means that the set of requirements UE shall meet is decided by the ATG indicator signaled by the serving network. Therefore, there is no need to introduce separate UE capability for enhanced requirements. The indicator communicated via system information since they apply to IDLE mode UEs.

Proposal 5: No need to define separate UE capability for enhanced cell reselection measurement requirements.

## 2.1	Mobility in RRC_CONNECTED

Location-based conditional handover

RAN4 has earlier agreed to introduce the legacy Rel-16 conditional handover feature for ATG0, but whether to support location based conditional handover remains open. Location-based conditional handover for ATG was discussed at previous meeting which resulted in following outcome [1]:

Issue 2-2-1: location-based CHO mechanism - Option 1: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition) - Option 2: Location based CHO is supported for A2G in Rel-18, and details of requirements are FFS - Option 3: Using BS location information and UE location information, CHO can be used for UE located on the blind area of ATG BS. - Option 4: Not to specify location-based CHO, such mechanism can be specified in future releases

Our view is that location based conditional handover can be useful for ATG scenario, where the UE can be configured to trigger a handover based on thresholds associated with the location of the UE wrt a reference location configured by the serving NW. Especially given that the ATG cells are large, operated at high altitude and high speed and on a dedicated network and may follow certain type of deployment mode, it may be beneficial to introduce requirements for location-based conditional handover. T was also discussed at last meeting whether the existing R17 NTN signalling can be reused to support location-based CHO for ATG. Even if the corresponding R17 NTN signalling cannot be reused directly, RAN4 can still request RAN2 to introduce similar signalling support for ATG. Nevertheless, details of the requirements can be discussed after RAN4 reaches a conclusion on whether to support location-based CHO.

Proposal 6: Location based CHO is supported for A2G in Rel-18, and details of requirements are FFS

# Conclusion

In this contribution we have discussed the open issues of mobility requirements of ATG. Based on the discussions we have made following observation and proposals:

- Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

    - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

    - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

    - If ATG UE altitude higher than certain threshold, ATG UE perform measurement

    - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement

Proposal 3: RAN4 to request RAN2 to define ATG indicator (via system information) which is used to determine which set (set 1 or set 2) of requirements to meet.

- UE meets set 2 requirements when ATG indicator is received.

- Otherwise, UE meets set 1 requirements.

Proposal 4: Set 2 requirements comprises both R17 HST cell detection, measurements and evaluation requirements.

Proposal 5: No need to define separate UE capability for enhanced cell reselection measurement requirements.

Proposal 6: Location based CHO is supported for A2G in Rel-18, and details of requirements are FFS

# References

R4-2306344, “WF on NR ATG RRM requirements”, CMCC

# Appendix

3GPP TSG-RAN WG4 Meeting # 107					         R4-230xxxx

# Incheon, South Korea, 22 – 26 May 2023

Title:	LS on introducing of ATG indicator

Release:	Rel-18

Work Item:	NR_ATG-Core

Source:	RAN4

To:	RAN2

Contact person:

Name:	Santhan Thangarasa

E-mail Address:	santhan.thangarasa@ericsson.com

# 1   Overall Description:

RAN4 has discussed the RRM requirements for ATG UE according to the work item objectives stipulated in RP-230279. Unlike legacy NR devices, ATG UE is a specialized equipment that is mounted on the aircraft and it may therefore support additional and different types of features compared to those supported by the legacy NR UE.

RAN4 has agreed to define two sets of requirements which the UE shall meet in RRC IDLE and INACTIVE states based on an indicator transmitted by the gNB. RAN4 has therefore identified a need to introduce a 1-bit indicator transmitted via system information for ATG UEs in RRC IDLE and INACTIVE modes. The received indicator is used by the ATG UE to determine which set of requirements to meet.

# 2   Actions

To RAN2:

RAN4 would like to respectfully request RAN2 to take the above information into account and introduce necessary signaling for the indicator.

# 3   Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #108	       	August 2023         			Toulouse, France

TSG-RAN WG4 Meeting #108-bis				October 2023	 	               Xiamen, China