3GPP TSG-RAN WG4 Meeting # 107		R4-2309224

# Incheon, Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RMC for eMTC test cases over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RMCs for cat-M are currently missing in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: FD-FDD RRC re-establishment test case for eMTC over NTN Change 2: HD-FDD RRC re-establishment test case for eMTC over NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to introduce RRM test cases without RMC for cat-M. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: A.3.1.3A, A.3.1.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### A.3.1.3A	MPDCCH Reference Channels for Cat-M1 UEs for Satellite access

#### A.3.1.3A.1	FDD in CEModeA

Table A.3.1.3A.1-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeA

| Parameter | Unit | Value |
| --- | --- | --- |
| MPDCCH Reference channel | - | R.xx FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas | - | 1 |
| DCI Format | - | 6-1A |
| Transmission Type | - | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 |
| Aggregation level | ECCE | 16 |
| Maximum number of repetitions | - | 8 |
| Frequency hopping | - | OFF |
| MPDSCH Narrowband |  | 1st |
| MPDCCH start subframe | subframes | 1 |
| MPDCCH start symbol | symbols | 2 |
| Payload (without CRC) | Bits | Note 1 |
| Cell ID | - | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |

#### A.3.1.3A.2	HD-FDD in CEModeA

Table A.3.1.3A.2-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeA

| Parameter | Unit | Value |
| --- | --- | --- |
| MPDCCH Reference channel | - | R.xx HD-FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas | - | 1 |
| DCI Format | - | 6-1A |
| Transmission Type | - | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 |
| Aggregation level | ECCE | 16 |
| Maximum number of repetitions | - | 8 |
| Frequency hopping | - | OFF |
| MPDSCH Narrowband |  | 1st |
| MPDCCH start subframe | subframes | 1 |
| MPDCCH start symbol | symbols | 2 |
| Payload (without CRC) | Bits | Note 1 |
| Cell ID | - | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |

#### A.3.1.3A.4	FDD in CEModeB

Table A.3.1.3A.4-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeB

| Parameter | Unit | Value |
| --- | --- | --- |
| MPDCCH Reference channel | - | R.yy FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas | - | 1 |
| DCI Format | - | 6-1B |
| Transmission Type | - | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 |
| Aggregation level | ECCE | 24 |
| Maximum number of repetitions | - | 128 |
| Frequency hopping | - | OFF |
| MPDSCH Narrowband |  | 1st |
| MPDCCH start subframe | subframes | 1 |
| MPDCCH start symbol | symbols | 2 |
| Payload (without CRC) | Bits | 18 |
| Cell ID | - | Note 1 |
| Note 1: Cell ID shall depend upon the test configuration. |  |  |

#### A.3.1.3A.5	HD-FDD in CEModeB

Table A.3.1.3A.5-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeB

| Parameter | Unit | Value |
| --- | --- | --- |
| MPDCCH Reference channel | - | R.yy HD-FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas | - | 1 |
| DCI Format | - | 6-1B |
| Transmission Type | - | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 |
| Aggregation level | ECCE | 24 |
| Maximum number of repetitions | - | 128 |
| Frequency hopping | - | OFF |
| MPDSCH Narrowband |  | 1st |
| MPDCCH start subframe | subframes | 1 |
| MPDCCH start symbol | symbols | 2 |
| Payload (without CRC) | Bits | 18 |
| Cell ID | - | Note 1 |
| Note 1: Cell ID shall depend upon the test configuration. |  |  |

### A.3.1.4A	PDSCH Reference Channel for Cat-M1 UEs for Satellite access

#### A.3.1.4A.1	FDD in CEModeA

Table A.3.1.4A.1-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.aa FDD | R.bb FDD |
| Carrier bandwidth | MHz | 1.4 | 1.4 |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 |
| Maximum number of repetitions |  | 16 | 1 |
| Frequency hopping |  | OFF | OFF |
| PDSCH Narrowband |  | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Cell ID |  | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.4A.2	HD-FDD in CEModeA

Table A.3.1.4A.2-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.aa HD-FDD | R.bb HD-FDD |
| Carrier bandwidth | MHz | 1.4 | 1.4 |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 |
| Maximum number of repetitions |  | 16 | 1 |
| Frequency hopping |  | OFF | OFF |
| PDSCH Narrowband |  | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Cell ID |  | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.4A.4	FDD in CEModeB

Table A.3.1.4A.4-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeB

| Parameter | Unit | Value |
| --- | --- | --- |
| PDSCH Reference channel |  | R.cc FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocksNote1 |  | 2 |
| Allocated subframes per Radio Frame | - | 10 |
| Modulation | - | QPSK |
| Target coding rate | - | 1/3 |
| Information Bit Payload |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 |
| Number of Code Blocks per Sub-Frame |  |  |
| Sub-Frames 0 ~ 9 | - | 1 |
| Maximum number of repetitions | - | 192 |
| Frequency hopping | - | OFF |
| PDSCH Narrowband | - | 1st |
| PDSCH start symbol | symbols | 2 |
| Cell ID |  | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |

#### A.3.1.4A.5	 HD-FDD in CEModeB

Table A.3.1.4A.5-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeB

| Parameter | Unit | Value |
| --- | --- | --- |
| PDSCH Reference channel |  | R.cc HD-FDD |
| Carrier bandwidth | MHz | 1.4 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocksNote1 |  | 2 |
| Allocated subframes per Radio Frame | - | 10 |
| Modulation | - | QPSK |
| Target coding rate | - | 1/3 |
| Information Bit Payload |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 |
| Number of Code Blocks per Sub-Frame |  |  |
| Sub-Frames 0 ~ 9 | - | 1 |
| Maximum number of repetitions | - | 192 |
| Frequency hopping | - | OFF |
| PDSCH Narrowband | - | 1st |
| PDSCH start symbol | symbols | 2 |
| Cell ID |  | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |

<End of Change 1>