3GPP TSG-RAN WG4 Meeting # 107	R4-2309246

Incheon, KR, May 22 – May 26, 2023

Agenda Item:			8.3.1

Source:					Qualcomm Incorporated

Title:                        TP to TR 38.871 on RRM test method

Document for:		Approval

# Introduction

In RAN4#106bis-e, the test methods for FR2 multi-Rx UE were discussed and the WF was approved in [1]. In this meeting, we provide TP to TR 38.871 based on the agreements on RRM test method.

Proposal: To approve the draft TP to TR 38.871 in the appendix.

# References

- R4-2305887, “WF on FR2 OTA test enhancements SI”, Qualcomm, RAN4#106-Bis-e, Apr. 2023

# Appendix: Draft TP to TR 38.871

-----------------------------------------------------------Start  of the changes------------------------------------------------------------

# 6	UE RRM testing methodology for multi-Rx chain DL reception

## 6.1	General

<Editor’s note: general aspects related to the UE RRM testing methodology for multi-Rx chain DL reception can be captured in this clause>

## 6.2	Measurement setup

<Editor’s note: outcome of measurement setup for UE RRM testing can be captured in this clause>

### 6.2.1 	Baseline setup

The measurement setup for UE RF testing described in section 5.2.2, i.e., measurement setup with full degree of freedom for AoA1 with fixed angular offset(s) between AoA1 and AoA2, can be reused as the baseline setup for UE RRM testing.  The baseline measurement setup of UE RRM testing is capable of two-simultaneous AoAs testing. DFF, IFF and IFF+FDD are the permitted test methods for UE RRM testing. Examples of measurement setup are illustrated in Figure 5.2.2-2 and Figure 5.2.2-3.

For RRM baseline measurement setup, the reference point is located at the centre of the QZ. The following two modes for useful signals (S) and noise signals (N) configuration have been identified and shall be supported by the multi-Rx chain DL reception RRM test system:

-	Mode 1 (SNR emulation): Test system transmits useful signals (S) and noise signals (N) to emulate target SNR condition.

-	Mode 2 (noise-free transmission): Test system transmits only useful signals (S).

### 6.2.2	Test parameters

#### 6.2.2.1	Test parameters for Mode 1

For Mode 1, the SINR at baseband for multi-DCI with overlapping scheme is given as follows:

SINR1BB=S1*G1*LossS2+NG2*Loss+N*G1*Loss+Noise_floor,                                                        (1)

where S1 and S2 are signal level from AoA1 and AoA2 respectively. G1 and G2 are the antenna gain from AoA1 and AoA2 respectively. Loss is the pathloss between probe and DUT. Noise_floor is the total noise at DUT baseband receiver. N is the artificial noise at the reference point from AoA1 and AoA2. The artificial noise level is identical from AoA1 and AoA22.

Since the wanted noise is set 6dB above UE thermal noise, therefore the total noise of baseband Noise_floor can be ignored when calculating the SINR. And the SINR can be rewritten as:

SINR1BB≈S1*G1*LossS2+NG2*Loss+N*G1*Loss=1S2S1+1SNR1*G2G1+1SNR1,                                                (2)

where SINR1 is the SINR from probe 1, and the key parameter affecting the SINR at baseband is G1/G2.

Similarly, the SINR at baseband for multi-DCI with partially overlapping scheme is given as:

- SINR1≈1 α*S2S1+1SNR1*G2G1+1SNR1,						       	        	        				(3)

where α indicates the ratio of overlapping resources (0<α<1).

For multi-Rx with non-overlapping scheme, the interference between two AoAs can be ignored. Then the test parameters for Mode 1 in the legacy RRM test methodology specified in [3] can be reused.

#### 6.2.2.2	Test parameters for Mode 2

For Mode 2, the SINR at baseband for multi-DCI with overlapping scheme is given as follows:

SINR1BB=S1*G1*LossS2*G2*Loss+Noise_floor,                                                                      (4)

where S1 and S2 are signal level from AoA1 and AoA2 respectively. G1 and G2 are the antenna gain from AoA1 and AoA2 respectively. Loss is the pathloss between probe and DUT. Noise_floor is the total noise at DUT baseband receiver.

If we set the signal level from interfering AoA direction at reference point is 6dB higher than UE thermal noise, then the approximate SINR can be rewritten as follows:

SINR1BB≈S1S2*G2/G1.                                                                                  (5)

Same as for Mode 1, and the key parameter affecting the SINR at baseband is G1/G2.

Similarly, the SINR at baseband for multi-DCI with partially overlapping scheme is given as follows:

SINR1BB≈S1α*S2*G2G1,                                                                                   (6)

where α indicates the ratio of overlapping resources (0<α<1).

For multi-Rx with non-overlapping scheme, the interference between two AoAs can be ignored. Then the test parameters for Mode 2 in the legacy RRM test methodology specified in [3] can be reused.

-----------------------------------------------------------End  of the changes------------------------------------------------------------