3GPP TSG-RAN WG4 Meeting # 107	R4-2309248

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	8.27.2

Source: 		Qualcomm Incorporated

Title: 	Calibration simulation results for above 10GHz bands

Document for:	Approval

# Introduction

In RAN4#106-bis-e, NTN-TN co-existence simulation assumptions were discussed and RAN4 agreed to consider the simulation assumptions in [1]. It was agreed to start the calibration phase for the NTN and TN simulations based on the agreed way forward in [2]. In this paper, we provide our simulation results for NTN and TN as single system for the calibration purposes only.

# 2. Discussion

In this document we provide the simulation results for the calibration purposes only in terms of SINR and coupling loss as agreed in [2] for NTN and TN as single systems in the Ka band. The simulation results are attached in the annex as an excel sheet for the comparison with other companies. However, the simulation results are preliminary and can be modified with further calibration. In the following we list the simulated cases for the calibration as well as the used parameters and assumptions.

- The simulated cases:

  - NTN GEO with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

  - NTN LEO 600 with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

  - NTN LEO 1200 with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

  - TN in the Urban environment with DL/UL at 17 GHz

  - TN in the Urban environment with DL/UL at 27 GHz

- Simulation parameters and assumptions are summarized in Table 1 and Table 2 for TN and NTN respectively:

Table 1: Summary of the TN simulation parameters and assumptions

| TN Parameter | Value |
| --- | --- |
| Carrier Frequency | 17 and 27 GHz |
| ISD | 300 m |
| UE to BS minimum distance | 35 m |
| Channel BW | 200 MHz |
| Number of UEs/cell | 1DL/3UL |
| BS antenna height | 25m |
| BS conducted power | 46 dBm, assuming 22 dBm for element 22+10*log10(16*8*2) |
| Noise figure | 10 dB for BS/UE |
| Mechanical down-tilt | 10 deg |
| BS antenna elements | 16x8 |
| Element gain | 5.5 dBi |
| Ohmic losses | 2 dB |
| (dv, dh) | (0.5λ, 0.5λ) |
| Theta 3dB/Phi 3dB | 90 deg/90 deg |
| Polarization gain | 3 dB |
| UE antenna height | 1.5m |
| UE indoor/outdoor ratio | 100% outdoor |
| UE power | 22.4 dBm |
| UE element gain | 5 dBi |
| UE number of elements | 2x2 |
| Path loss | 3GPP TR 38.803 (UMa) |
| Handover margin | 3 dB |

Table 2: Summary of the NTN simulation parameters and assumptions

| NTN Parameter | Value |
| --- | --- |
| Carrier Frequency | 17 GHz DL / 27 GHz UL |
| Satellite | GEO / LEO 600 / LEO 1200 |
| Altitude | 35786 / 600 / 1200 km |
| Channel BW | 200 MHz |
| Number of UEs/cell | 1 DL/10 UL |
| Beam diameter | 110 / 20 / 40 km |
| Satellite elevation | 90 deg |
| 3dB beamwidth or HPBW (Half-Power Bandwidth) of main central beam | 0.1765 / 1.7647 / 1.7647 |
| EIRP density | 40 / 4 / 10 dBW/MHz |
| Satellite TX gain | 58.5 / 38.5 / 38.5 dBi |
| Satellite antenna diameter | 5.9 / 0.6 / 0.6 m |
| Satellite noise figure | 5.9 dB for all types for calibration only |
| VSAT antenna height | 22.5 m |
| UE TX power | 33 dBm |
| UE noise figure | 4 dB |
| UE TX antenna gain | 43.2 |
| UE RX antenna gain | 39.7 |
| UE antenna diameter | 0.6 m |
| UE antenna temperature | 150 k |
| Path loss | 3GPP TR 38.821 with Urban environment atmospheric losses and the scintillation losses not considered for calibration |
| Handover margin | 3 dB |

It should be noted that the collected CDF statistics for coupling loss and the SINR are for the NTN UEs in the inner beam only as done in the previous studies for S-band.

It should also be noted that we also tested the L-ESIM with changing only the height to 1.5 m instead of 22.5 m for VSAT. The results of L-ESIM are identical assuming the same antenna model, given that we didn’t include them in the calibration results in the annex. By changing the height later in the coexistence studies, we may see a difference in the results due to the different propagation model between TN BS to VSAT and TN BS to L-ESIM. However, RAN4 shall further discuss if it is needed to assume different antenna model for L-ESIM or not.

Observation 1: By changing the height only for the NTN UE from 22.5 m to 1.5 m to capture the L-ESIM assumptions without changing the antenna model will lead to same results for single system.

Proposal 1: RAN4 to consider phased array antenna model for L-ESIM. Satellite industry is encouraged to provide detailed parameters for the phased array antenna model for L-ESIM.

# 3.	Conclusion

In this paper, we provided the calibration simulation results in terms of SINR and coupling loss as agreed in [2] for NTN and TN as single systems in the Ka band.

We provide calibration results for the following cases:

- NTN GEO with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

- NTN LEO 600 with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

- NTN LEO 1200 with fixed VSAT in the Urban environment with DL at 17 GHz and UL at 27 GHz

- TN in the Urban environment with DL/UL at 17 GHz

- TN in the Urban environment with DL/UL at 27 GHz

Also, we provide the simulation parameters and assumptions for TN and NTN in Table 1 and Table 2 respectively.

Based on the simulation results, we have the following observation and proposal:

Observation 1: By changing the height only for the NTN UE from 22.5 m to 1.5 m to capture the L-ESIM assumptions without changing the antenna model will lead to same results for single system.

Proposal 1: RAN4 to consider phased array antenna model for L-ESIM. Satellite industry is encouraged to provide detailed parameters for the phased array antenna model for L-ESIM.

# 4.	References

- R4-2306003, Simulation assumptions for NTN co-existence study in bands above 10GHz, 106-bis-e meeting, Samsung

- R4-2306002, WF on [311] NR_NTN_enh_Part3, Samsung

# 5.	Annex

The calibration simulation results.