3GPP TSG-RAN4 Meeting #107		R4-2309271 Incheon, South Korea, 22th – 26th May 2023

Source: 	Qualcomm Inc.

Title: 	System parameters for dedicated spectrum

Agenda Item:	8.15.2

Document for:	Approval

# Introduction

RAN has agreed a work item for NR support for dedicated spectrum less than 5 MHz for FR1 [1]. Part of the work item objectives for RAN4 is to specify system parameters. In this contribution system parameters for the dedicated spectrum are discussed and proposals for the design principles are made.

# 2	Discussion

- Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

The WI objectives which are relevant to RAN4 RF work are copied below.

From the WI objectives it can be observed that while the RAN4 requirements are intended to cover spectrum allocation from approximately 3 MHz up to below 5 MHz, the RF channel bandwidth impact is limited. Only new channel bandwidth is optional 3 MHz channel bandwidth, while re-use of 5 MHz channel bandwidth shall take place for other use cases.

Synchronization raster design for the 12 RB in 3 MHz channel

In this document we first analyse the extraordinary less than full channel bandwidth scenarios as sync raster choice for those has little flexibility. Later on, we look at the complete raster design holistically taking into account also compatibility with the extraordinary transmission bandwidths. The support for 12 RB and 20 RB transmission bandwidth is needed only in band n100.

RAN has made further agreements and communicated those to RAN1 and RAN4 [2, 3]. In the context of synchronization raster design, the key new information is that PBCH bandwidth is 12 PRBs in 3 MHz channel in band n100, while it is still open whether 12 or 15 PRBs should be used in other bands. Secondly, PBCH bandwidth is 20 PRBs for 5 MHz channel bandwidth. In addition to these agreements, RAN also discussed value of 12 RB transmission bandwidth in n100 as it would allow for more GSM-R carrier to be used in the early phase of the migration towards NR usage [4]. Therefore, synchronization raster needs to be designed taking into account this information.

There is decision needed on which RB position within the 15 RB RF channel and which position within the operating band is used when 12 RB transmission bandwidth is used. We see that it is not reasonable to enable 12 RB transmission bandwidth for every RB position, as this would greatly increase the number of required raster points and make the initial search very cumbersome. As the motivation is to enable more GSM-R carriers in n100, it would make sense to consider only supporting the low edge of the 3 MHz channel and only the low edge of the operating band for the 12 RB transmission, as this would place the NR signal furthest away from the GSM-R signals and the remaining contiguous spectrum would be available for more GSM-R carriers.

This configuration is illustrated in Figure 2:

Figure 2: 12 PRBs transmission bandwidth within 3 MHz RF channel

The exact selection of the sync raster point needs to consider also the ECC Decision (20)02 [5]. The decision states that the lower edge of the lowest Resource Block shall be ≥ 919.6 MHz, i.e. 200 kHz above the low edge of the operating band. Given that the minimum guard band for 3 MHz channel bandwidth is only 142.5 kHz, the lowest RF channel position in the operating band would violate the ECC decision. Therefore, the RF channel position to be used is the one centered on 920.0 MHz, with sync raster point at 920.73 MHz.

Observation 1: ECC Decision (20)02 state that lower edge of the lowest resource block shall be ≥ 919.6 MHz, which means that 3 MHz channel bandwidth cannot be placed immediately adjacent to the low edge of the operating band.

Proposal 1: Specify a single sync raster point at 920.73 MHz to support 12 RB transmission bandwidth with 3 MHz RF channel. This raster point is specified only for band n100.

Synchronization raster design for 20 RB in 5 MHz channel

Next 20 RB in 5 MHz RF channel bandwidth is discussed. Similarly as for 12 RB, we think it is sufficient to enable the 20 RB transmission bandwidth only at the low edge of the operating band and at the low edge of the RF channel. It should be noted that PBCH is not punctured for 5 MHz channel bandwidth.

Observation 2: SSB puncturing shall not be used for 5 MHz channel bandwidth according to RAN decision.

To enable 20 PRB allocation with unpunctured SSB at the low edge of the channel, sync raster point(s) needs to be such that the lower edge of the SSB aligns with the low edge to lowest PRB. There are three options how this can be enabled.

First way is to use the lowest sync raster point which is already specified for n100, i.e. GSCN 2303. When GSCN 2303 is used, the SSB aligns with 20 lowest PRB of 5 MHz channel when channel raster point is set at 922.1 MHz and low edge of the RF channel is therefore at 919.6 MHz. This means that 200 kHz at the low edge of the band would not be used.

Second way is to aim to take also the lowest 200 kHz of the band into use. This would require one new sync raster point placed 200 kHz below GSCN 2303, i.e. both sync and channel raster point are 200 kHz lower compared to the first option. This option of using a new sync raster point can leave no spectrum unused at the low edge of the band.

Finally as a third option, more raster points could the added to enable 20 RB allocation in any channel position. In all options, kSSB  will be always equal to zero as SSB will fully align with the PRB grid.

The most reasonable option is option 2, as it does not waste 200 kHz at the bottom of the band, and it also minimizes the need for new raster points as only one new raster point is needed for the special use case in n100. The single raster point relative to rel-15 raster design has been illustrated in Figure 3.

Figure 3: Sync raster to support  5 MHz ChBW with 20 RB allocation

Proposal 2: Specify one new sync raster point 200 kHz below GSCN 2303 at 921.45 MHz, allowing use of 5 MHz channel centered at 921.9 MHz. This raster point is specified only for band n100.

Observation 3: kSSB will be always equal to zero also for 5 MHz supporting narrowband allocation.

Floating raster design for 3 MHz RF channel bandwidth.

When transmission bandwidth in n100 increases to 15 PRB but PBCH bandwidth remains at 12 PRB, it is possible to take advantage of the floating raster design where single sync raster point covers multiple RF channels. In previous meeting there were multiple proposals for the floating raster design and they were captured in the agreed WF in [6]. The floating raster can be generally written as

  - N * 600kHz + M * 50 kHz + A kHz, M ϵ {1,3,5}, A = TBD.

and the most critical open aspect is which value to set for parameter A. The different raster points are illustrated together in Figure 4.

Figure 4: Different rasters, A=300 kHz in the figure.

Figure 4 works as a visual aid to understand the impact of selection of A. The different offsets and overlaps for different values of A have been summarized in Table 2.

| Value of A | Min offset to rel-15 raster (case 4 in Figure 4) | Min offset to 12 RB sync raster point (case 1 in Figure 4) | Min offset to 20 RB sync raster point (case 1 in Figure 4) |
| --- | --- | --- | --- |
| A = 300 kHz | 100 kHz | 20 kHz | 0 kHz |
| A = 280 kHz | 80 kHz | 0 kHz | 20 kHz |
| A = 345 kHz | 55 kHz | 65 kHz | 45 kHz |

It can be seen that if A=300 kHz, the floating raster points will be very close to the point reserved for 12 RB operation and overlap with the point reserved for 20 RB operation. Especially the overlap is extremely harmful, as this is an overlap between two different PBCH bandwidths and would require the UE to go through blind decoding. A = 300 kHz should therefore be avoided. In case this design option is pursued, it is preferable to puncture the overlapping floating raster points even though it results in some RF channel positions not being available.

Observation 4: A = 300 kHz will result in sync raster points for different PBCH bandwidths to overlap resulting in blind decoding, and therefore A = 300 kHz is to be avoided.

A second possibility would be to use A = 280 kHz. This introduces 20 kHz offset between sync raster points for different PBCH bandwidths. The overlap case happens with same PBCH bandwidth of 12RBs, so it is not harmful as the one punctured SSB pattern for 3MHz channel bandwidth does not require UE blind detection.

Observation 5: A = 280 kHz will result in sync raster points for the same PBCH bandwidth to overlap.

Thirdly, if A = 345 kHz, at least 45 kHz frequency offset is kept between all different sync raster points. For this reason, A = 345 kHz is the best option. This is also aligned with the criteria approved in [6].

Observation 6: A = 345 kHz maintains at least 45 kHz frequency offset between all different rasters in n100, and in all the other operating bands maintains 55 kHz frequency offset to rel-15 raster. According to the criteria in the approved WF from RAN4#106bis-e, A = 345 kHz is the preferred design.

Finally, if there is desire to further minimize sync raster points in band n100, it could be even considered to treat 3 MHz channel bandwidth similarly as 12 RB and 20 RB transmission bandwidths, and only specify one sync raster point for 15 RB transmission bandwidth for 3 MHz channel bandwidth.

Proposal 3: Specify floating raster N * 600kHz + M * 50 kHz + 345 kHz, M ϵ {1,3,5} for all operating bands in scope of the WI to support 3 MHz channel bandwidth.

Although there is offset between new and legacy sync rasters, it may be not 100% accurate to rely on the PSS/SSS to detect the cells for legacy UEs and new UEs. It is desired to further consider the early indication to avoid legacy UEs camping on the cells with less than 5MHz.

Observation 7: Despite the frequency offset between rasters, early indication to avoid legacy UEs camping on cells with less than 5 MHz operation is useful.

Proposal 4: Send LS to RAN1 to consider early indication to avoid legacy UEs camping on the cells with less than 5MHz operation.

Capturing synchronization raster to RAN4 specifications

The guiding principle in defining the sync raster points have been that each sync raster point is identified with a unique global synchronization channel number (GSCN) and a set of formulas is defined how GSCN links to the absolute frequency. Those formulas cannot be used when new sync raster frequencies are being added amongst the existing raster. Still, it is necessary to have the GSCN definition in place as in addition to RAN4 purposes it is also used by RAN2 for example in dl-CarrierFreq in SIB4 as well as in absoluteFrequencySSB descriptions.

From UE implementation perspective, there is also a need to avoid any confusion or overlap with legacy raster given that 3 MHz channel bandwidth is optional and new Rel-18 feature which should not impact legacy UEs.

Observation 8: New GSCN-values need to be defined with clear distinction to legacy raster

For higher layer RAN2 signaling, the value communicated in RRC is ARFCN value which corresponds to the GSCN frequency. Therefore, preferred approach on defining the new GSCN in a manner which is compatible with RAN2 is to use ARFCN values also in RAN4. In practice this would mean that for the new sync raster points GSCN is defined to be equal to ARFCN-value point to the absolute synchronization raster frequency. This ARFCN value can and will be different from the ARFCN value pointing to channel raster.

This approach has multiple benefits. Firstly, no new complex definitions are needed but existing and proven-to-work raster points are used. Secondly, the ARFCN value range is disjoint from existing GCSN as well as any potential extensions of current framework to frequency ranges which have been already studied in 3GPP, such as 7-24 GHz.

Observation 9: Using ARFCN values also to indicate GSCN is compatible with RAN2 design and provides a disjoint value range compared to existing GSCN.

An example how this would look like in the specification is provided below. Note that some bands and notes have been removed from the existing tables to make the content more readable fit to smaller space. Additionally, the detailed NREF values will depend on puncturing. New content is in red.

***** Example specification text *****

#### 5.4.3.3		Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n26 | 15 kHz | Case A | 2153 – <1> – 2230 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n85 | 15 kHz | Case A | 1826 – <1> – 1858 |
| n100 | 15 kHz | Case A | 2303 – <1> – 2307 |
| n106 | 15 kHz | Case A |  |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. |  |  |  |

For operating bands n26, n28, n85, n100 and n106 synchronization raster shall include GSCN = NREF and SSREF = NREF*5 kHz. The applicable NREF are defined in table 5.4.3.3-2. GSCN defined in table 5.4.3.3-2 are applicable only for 3 MHz channel bandwidth.

Table 5.4.3.2-2: Additional GSCN parameters for the global frequency raster for 3MHz ChBW in n106, n26, n28, n85 and n100

| Operating band | SS Block SCS | SS Block pattern1 | Range of NREF |
| --- | --- | --- | --- |
| n106 | 15 kHz | Case A |  |
| n26 | 15 kHz | Case A |  |
| n28 | 15 kHz | Case A |  |
| n85 | 15 kHz | Case A |  |
| n100 | 15 kHz | Case A |  |
|  |  |  | (N * 600 + M * 50 + 345)/5 , N=1534:1538, M ϵ {1,3,5} |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. |  |  |  |

***** Example specification text ends *****

As seen from the example, using ARFCN values also for GSCN purposes can be easily done.

Proposal 5: Define unique GSCN-values for dedicated spectrum less than 5 MHz using ARFCN-values.

# Conclusions

In this contribution synchronization raster for dedicated spectrum less than 5 MHz was discussed. Following observations and proposals were made:

Observation 1: ECC Decision (20)02 state that lower edge of the lowest resource block shall be ≥ 919.6 MHz, which means that 3 MHz channel bandwidth cannot be placed immediately adjacent to the low edge of the operating band.

Observation 2: SSB puncturing shall not be used for 5 MHz channel bandwidth according to RAN decision.

Observation 3: kSSB will be always equal to zero also for 5 MHz supporting narrowband allocation.

Observation 4: A = 300 kHz will result in sync raster points for different PBCH bandwidths to overlap resulting in blind decoding, and therefore A = 300 kHz is to be avoided.

Observation 5: A = 280 kHz will result in sync raster points for the same PBCH bandwidth to overlap.

Observation 6: A = 345 kHz maintains at least 45 kHz frequency offset between all different rasters in n100, and in all the other operating bands maintains 55 kHz frequency offset to rel-15 raster. According to the criteria in the approved WF from RAN4#106bis-e, A = 345 kHz is the preferred design.

Observation 7: Despite the frequency offset between rasters, early indication to avoid legacy UEs camping on cells with less than 5 MHz operation is useful.

Observation 8: New GSCN-values need to be defined with clear distinction to legacy raster

Observation 9: Using ARFCN values also to indicate GSCN is compatible with RAN2 design and provides a disjoint value range compared to existing GSCN.

Proposal 1: Specify a single sync raster point at 920.73 MHz to support 12 RB transmission bandwidth with 3 MHz RF channel. This raster point is specified only for band n100.

Proposal 2: Specify one new sync raster point 200 kHz below GSCN 2303 at 921.45 MHz, allowing use of 5 MHz channel centered at 921.9 MHz. This raster point is specified only for band n100.

Proposal 3: Specify floating raster N * 600kHz + M * 50 kHz + 345 kHz, M ϵ {1,3,5} for all operating bands in scope of the WI to support 3 MHz channel bandwidth.

Proposal 4: Send LS to RAN1 to consider early indication to avoid legacy UEs camping on the cells with less than 5MHz operation.

Proposal 5: Define unique GSCN-values for dedicated spectrum less than 5 MHz using ARFCN-values.

# References

[1]	RP-222645, NR support for dedicated spectrum less than 5 MHz for FR1, Nokia, Nokia Shanghai Bell

[2]	RP-230780, Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

[3]	RP-230781, Response to LS from RAN4 on spectrum less than 5 MHz

[4]	RP-230189, UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, Union International Chemins de Fer

[5]	ECC Decision (20)02, Harmonised use of the paired frequency bands 874.4- 880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)

[6]	R4-2306615, WF on system requirements, Nokia, Nokia Shanghai Bell