3GPP TSG-RAN4 Meeting #107	        R4-2309272 Incheon, South Korea 22nd – 26th May 2023

Source: 	Qualcomm Inc.

Title: 	NS_47 measurements and A-MPR for PC1.5

Agenda Item:	4.1

Document for:	Approval

# Introduction

Power class 1.5 for NS_47 in band n41 was discussed in previous meeting, with WF agreed in [1]. NS_47  additional emission requirements require A-MPR and in this contribution simulation results are provided and A-MPR table is proposed.

# Discussion

PC1.5 is always using two PA solution to satisfy the power class output power requirement. As current simulation methods do not accurately capture the effects of PAs coupling to each other via antennas, measurement are needed to quantify the A-MPR. Additional emission requirements for NS_47 are shown in Table 1.

Table 1: Additional requirements for NR channels assigned within 2545 - 2575 MHz for "NS_47"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 30 |  |
| 2530 ≤ f ≤ 2535 | -25 | 1 MHz |
| 2505 ≤ f ≤ 2530 | -30 | 1 MHz |

To assess the need for A-MPR with NS_47 requirements in n41 we have performed measurements considering both APT and ET operation modes for the PA.

Based on the measurement results and the simulation results from [2], we propose A-MPR for NS_47 with PC 1.5 as shown in Table 3. We have also taken into account the CIM3 consideration from [3] in the proposal. In the proposed values it was also taken into account that PC1.5 A-MPR shall not be more than 3 dB increased from PC2, to avoid cases where PC1.5 UE would be transmitting less power than PC2 UE. While and additional A-MPR region was proposed in [2], we did not see the need for it in measurements.

Table 3: Proposed A-MPR for PC1.5 for NS_47

| Modulation/Waveform | A1(dB) |  |  | A2(dB) |  |  | A3(dB) |  |  | A4(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 |
|  | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ |
|  | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 13 | ≤ 5.5 | ≤ 8.5 | ≤ 11 | ≤ 2 | ≤ 5 | ≤8 | ≤ 3 | ≤ 6 | ≤ 8.5 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 |  | ≤ 5.5 | ≤ 8.5 |  | ≤ 2 | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 |  | ≤ 5.5 | ≤ 8.5 |  |  | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 |  | ≤ 6 | ≤ 8.5 |  |  | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 |  | ≤ 6 | ≤ 8.5 |  |  | ≤ 5 |  |  | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 | ≤ 12.5 |  | ≤ 5 |  | ≤ 4 | ≤ 7 | ≤ 9.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  | ≤ 5 |  | ≤ 4 | ≤ 7 |  |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  | ≤ 5 |  |  | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  |  |  |  | ≤ 7 |  |

The A-MPR regions are unchanged from other power classes and as defined as shown in Table 4.

Table 3: A-MPR regions for NS_47

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |

In the WF [1], it was mentioned that applicable release is still open. In our view the change should be made to the open release. UEs from earlier releases could still support PC1.5 in n41 with NS_47 via release independence mechanism.

# Conclusions

In this contribution A-MPR for n41 with PC1.5 and NS_47 additional requirements were discussed. Based on the discussion we propose

Proposal 1: Adopt A-MPR for NS_47 in n41 with PC1.5 as according to table 3

Table 3: Proposed A-MPR for PC1.5 for NS_47

| Modulation/Waveform | A1(dB) |  |  | A2(dB) |  |  | A3(dB) |  |  | A4(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 |
|  | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ | Outer/ |
|  | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner | Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 13 | ≤ 5.5 | ≤ 8.5 | ≤ 11 | ≤ 2 | ≤ 5 | ≤8 | ≤ 3 | ≤ 6 | ≤ 8.5 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 |  | ≤ 5.5 | ≤ 8.5 |  | ≤ 2 | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 |  | ≤ 5.5 | ≤ 8.5 |  |  | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 |  | ≤ 6 | ≤ 8.5 |  |  | ≤ 5 |  | ≤ 3 | ≤ 6 |  |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 |  | ≤ 6 | ≤ 8.5 |  |  | ≤ 5 |  |  | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 | ≤ 12.5 |  | ≤ 5 |  | ≤ 4 | ≤ 7 | ≤ 9.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  | ≤ 5 |  | ≤ 4 | ≤ 7 |  |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  | ≤ 5 |  |  | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 |  | ≤ 7 | ≤ 10 |  |  |  |  |  | ≤ 7 |  |

Proposal 2: Make the A-MPR change to the open release.

# References

[1]	R4-2306541, WF on PC1.5 NS_47 and SUL PC2 specification impacts, Skyworks Solutions Inc.

[2]	R4-2300346, Considerations on PC1.5 for NS_47, Apple Inc.

[3]	R4-1907167, “n41 A-MPR for N-star (Japan)”, Qualcomm Incorporated