3GPP TSG-RAN WG4 Meeting #107	R4-2309288

Incheon, KR, May 22nd – 26th, 2023

Source:			Rohde & Schwarz

Title:			Fast TRS method based on RSRPB

Agenda Item:	8.16.2.4

Document for:	Approval

# Introduction

The work item on Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) [1] was revised during RAN #98-e meeting with the following objective in the core part:

The following objectives are considered in this WI:

- Enhancements of TRP TRS test methodology

[…]

  - Study and specify the testing time reduction methodology for TRP and TRS testing

We present in this contribution a new method for Fast TRS based on the Received Power per Branch (RSRPB).

# Background

OTA sensitivity testing (i.e. TRS) based on Receive Signal Strength (RSS) has been used broadly in the past as a very efficient method to reduce the test time, but it presents some issues for technologies and UEs implementing Rx Diversity. The main reason for it is the definition of RSS for multiple receivers (e.g. RSRP for E-UTRA and NR), which provides flexibility in the way the received power per antenna connector is combined. Here is an excerpt of the SS-RSRP for NR as defined in 38.215 [2].

### 5.1.1	SS reference signal received power (SS-RSRP)

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The number of resource elements within the measurement period that are used by the UE to determine SS-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

According to this definition, it is not possible to stablish a correlation between the RSRP per grid point and the actual sensitivity in case that Rx Diversity is used, which would drive eventually to a wrong calculation of the TRS.

Observation 1: traditional Fast TRS methods based on RSS (e.g. RSRP for NR) cannot be used for devices implementing Rx Diversity.

On the other hand, the combination of individual TRS measurements (or TIS) per receiver into a combined TRS (or C-TIS) was demonstrated in CTIA [3] and could enable the traditional RSS based method if tested per receiver. Even though, this approach per receiver becomes unpractical due to the increase in test time (i.e. x4 in case of four receivers) and the need for proprietary software to control the activation/deactivation of each receiver at a time, without mentioning the additional uncertainty due to the test of each receiver separately.

Observation 2: traditional Fast TRS testing per receiver based on RSS (e.g. RSRP for NR) is unpractical.

These issues can be solved by adopting the test procedure described in next section, which uses the Received Power per Branch (SS-RSRPB) defined for NR FR1 in clause 5.1.18 of TS 38.215 [2] instead of RSRP.

### 5.1.18	SS reference signal received power per branch (SS-RSRPB)

| Definition | SS reference signal received power per branch (SS-RSRPB) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals (SS). The measurement time resource(s) for SS-RSRPB are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. For SS-RSRPB determination demodulation reference signals for physical broadcast channel (PBCH) in addition to secondary synchronization signals may be used. SS-RSRPB using demodulation reference signal for PBCH shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. SS-RSRPB shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If higher-layers indicate certain SS/PBCH blocks for performing SS-RSRPB measurements, then SS-RSRPB is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, SS-RSRPB shall be measured from each antenna connector of the UE. For frequency range 2, SS-RSRPB shall be measured for each receiver branch based on the combined signal from antenna elements corresponding to the receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency |

NOTE 1:	The number of resource elements within the measurement period that are used by the UE to determine SS-RSRPB is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:	This UE measurement is defined only for conformance test purposes. It is described along with test control entity signalling in [14].

This method could be extended to LTE/E-UTRA for cases where the UE enables RSAP reporting as described in sections 5.9 and 9.1 of 3GPP TS 36.509 [4].

Observation 3: Fast TRS method based on RSRPB for NR FR1 could be extended to LTE/E-UTRA.

Proposal 1: approve the usage of Fast TRS method based on RSRPB as described in section 3.

# Test procedure

The Fast TIS based on RSRPB proposed in this contribution consists of four steps:

# RSRPB Pattern Measurement

Following the same approach as defined for TRS in section 7.4.1 of TR 38.870 [5], a spherical scan measurement is performed but, instead of performing a sensitivity search at each point, a fixed DL power level is used and n RSRPB measurements are recorded for the n receiver branches in the UE, for each grid point and polarization.

# RSRPB Pattern Linearization

The RSRPB pattern measurements in step 1 must be linearized independently for each receiver. In order to do so, the UE is oriented to the location of the peak RSRPB for receiver i and RSRPBi measurements are recorded in 1dB steps for a sufficient power range (e.g. 10dB) from the peak RSRPBi measured in step 1. This procedure is repeated n times for the n receiver branches in the UE.

# Sensitivity search

In order to obtain the peak EIS, to be used later TRS calculation, the UE is oriented to the location and polarization of the maximum of the combined RSRPB from step 1, defined as

RSRPBcombined=maxRSRPB1+…+RSRPBn

In this position, a single sensitivity search is performed as defined in step 3) in section 7.5.2.1 of TR 38.870 [5].

# Final TRS Calculation

As a final step, the EIS value per point in the grid is calculated as follows (in linear units):

EISθ,ϕ=11EIS1RSRPB1,linθ,ϕ+…+1EISnRSRPBn,linθ,ϕ

Being RSRPBi,linθ,ϕ=Piθ,ϕ·c+m , where P is the DL power level corresponding to RSRPBi in the linearization trace, and c and m are the coefficients of the Best Fit Least-Squares (BFLS) regression line to which the linearization trace can be fitted.

The results shall be normalized to the peak EIS value obtained in step 3 and the final TRS value is calculated following the definition in section 5.2.1 of TR 38.870 [5].

# Proposal

In this contribution, the following observations and proposals were made:

Observation 1: traditional Fast TRS methods based on RSS (e.g. RSRP for NR) cannot be used for devices implementing Rx Diversity.

Observation 2: separate TRS testing per receiver based on RSS is unpractical.

Proposal 1: approve the usage of Fast TRS method based on RSRPB as described in section 3.

# References

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)”, vivo, 3GPP TSG RAN Meeting #98-e, December 2022

- 3GPP TS 38.215 v17.3.0 (March 2023)

- CPWG170313-3, “The Relationship Between Combined and Individual TIS”

- 3GPP TS 36.509 v17.3.0 (March 2023)

- 3GPP TR 38.870 v0.3.0 (May 2023)

Only available to CTIA OTA group members