3GPP TSG-RAN WG4 #107																			 R4-2309297

Incheon, South Korea, May 22nd – 26th 2023

Agenda item:	8.12.4

Source: 	Qualcomm Incorporated

Title: 	Demodulation Tests for non-collocated intra-band CA Deployments

Document for:	Discussion

# Introduction

The introduction of demod tests for Type II UEs supporting intra-band non-collocated CA deployments was discussed in RAN4#106-bis-e and a WF to further study this issue was agreed in [1]. In this paper we further analyze the definition of such a test.

# Discussion

The introduction of a demod test to check the performance of a type II UE under power imbalance and large RTD was left FFS [1]. The RF requirements for such a UE are defined with a power imbalance of 25dB and an sensitivity degradation of 1dB. Hence, these UEs will already have to pass an RF test with such a power imbalance. However, this test will not check the throughput on the stronger carrier, hence, introduction of a demod test to check the throughput on both CCs would be needed to check that the UE correctly handles both CCs.

Observation 1: A demod test would be needed to check that the UE correctly handles both CCs at the same time.

Regarding the propagation conditions, a static channel (basically AWGN) should be used to ensure that the power imbalance is constant and predictable. It would be very difficult to define the requirements if an environment in which the power imbalance goes above 25dB is chosen.

Observation 2: A static channel (AWGN) should be used for this test.

Similarly to most demod tests, artificial noise should be introduced such that the SNR on both CCs is predictable and throughput performance can be set and checked. In order to ensure the testability of such requirements, the SNR of the weaker CC should be set below or close to the noise floor while the SNR of the stronger CC should be set 25dB higher. The actual signal levels should be further studied.

Observation 3: Artificial noise should be added in this test, the SNR of the weaker CC should be set around below or around 0dB while the stronger CC should be set 25dB higher. Actual signal levels should be further discussed.

One important aspect of the Type II UE capability is that it should be able to handle an MRTD of 33us. Hence, the test should have the RTD between the CCs set to 33us.

Observation 4: RTD between the CCs in the test should be set to 33us.

# Conclusion

In this paper we briefly analyzed the introduction of a demod test to check the performance of  Type II UEs in a non-collocated intra-band CA deployment scenario. We made the following observations:

Observation 1: A demod test would be needed to check that the UE correctly handles both CCs at the same time.

Observation 2: A static channel (AWGN) should be used for this test.

Observation 3: Artificial noise should be added in this test, the SNR of the weaker CC should be set below or around 0dB while the stronger CC should be set 25dB higher. Actual signal levels should be further discussed.

Observation 4: RTD between the CCs in the test should be set to 33us.

# References

- R4-2305891, “WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario”, Ericsson, RAN4#106-bis-e