3GPP TSG-RAN WG4 Meeting #107                                                               R4-2309320

Incheon, Korea Republic, May 22nd – 26th, 2023

Source:	Samsung

Title:	Discussion on non-collocated FR1 inter-band EN-DC/NE-DC with overlapping DL bands

Agenda item:			4.4

Document for:	Discussion

# 1. Introduction

From Rel-16 and 17, UE requirements for non-co-located deployment for FR1 intra-band non-contiguous EN-DC was introduced while the requirements were limited to two Rx ports per band.

In Rel-18 a new WI is approved which focused on non-collocated FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band NR-CA deployment scenarios [1]. Based on the Rel-18 discussion, it had been identified that some maintenance works are expected on the requirement for Rel-16 Type-2 UE for EN-DC, but it was agreed to be discussed out of the scope of Rel-18 WI:

Online session (Wed / Mar 01) in RAN4#106 - Agreements - Do not consider MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in the scope of this work item

In this contribution, we would like to further share our analysis on RRM maintenance aspects for non-collocated FR1 inter-band EN-DC with overlapping DL bands.  In the appendix of this contribution, we provided the contents which has been given in R4-2300448 already on the analysis of Rel-15 Inter-band EN-DC/NE-DC with overlapping DL bands for information purpose.

# 2. Rel-16/17 Inter-band EN-DC/NE-DC with overlapping DL bands for non-collocated deployment

In Rel-16, RAN2 has introduced the following UE capability, i.e., interBandMRDC-WithOverlapDL-Bands-r16 (Type-2 UE which indicates 2 layer/2 Rx Chain per CC supported for non-collocated EN-DC or NE-DC), based on RAN4’s Feature List input (R4-2016850, as below Table 1a), while the corresponding RAN4 RF requirement has only been introduced in Rel-16 TS38.101-3 (in the time frame of Rel-17). Specifically, 25dB power imbalance and 1dB REFSENS relaxation for Type-2 UE in-band blocking requirements of inter-band EN-DC with overlapping DL bands have been specified in TS38.101-3.

Table 1a. UE Feature List corresponding to IE interBandMRDC-WithOverlapDL-Bands-r16

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-19 | FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum | Type 1 UE: supports FDD-TDD or TDD-TDD inter-band operation with overlapping or partially DL bands with MRTD<3us and intra-band EN-DC requirements apply. Type 2 UE: supports FDD-TDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with an MR-DC MRTD according to clause 7.6.2 in 38.133 and applicable inter-band RF requirements. If absent the UE is a type 1 UE. |  | Yes | N/A | UE only supports FDD-FDD or TDD-TDD inter-band operations with overlapping or partially DL bands with MRTD<3us and intra-band EN-DC requirements apply | Per band combination | N/A | FR1 only | N/A |  | Optional with capability signalling |

Table 1b. UE capability for IE interBandMRDC-WithOverlapDL-Bands-r16

For RRM requirement, the clause in TS38.306 have removed the MRTD<3us restriction for Type-2 EN-DC UE. However, which requirement shall be applicable for Type 2 EN-DC UE is not fully clarified. Based up the following RAN4 agreement in RAN4#105 [7], the MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type-2 UE shall be 33 and 35.21us respectively:

Issue 1-2-2: MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE - Agreements - MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE - MRTD=33us - MTTD=35.21us Issue 1-2-4: MRTD/MTTD requirement applicability for Rel-15/16/17 UE: - Proposals - Option 1: - FFS the applicability of existing MRTD/MTTD requirements for different UE capabilities, i.e. UE supporting interBandMRDC-WithOverlapDL-Bands-r16 or not. - FFS whether to have further clarification in the spec.

Next question (as agreed in WF [7] under Issue 1-2-4) will be: whether/how the Rel-16 RRM requirement shall be revisit to match RAN2 spec TS38.306 and the above agreement in [7]. Here we propose to have the following text proposal to clarify.

Proposal 1: The following text proposals for MTTD/MRTD are captured in TS38.133 from Rel-16, to clarify Type-2 UE supporting Inter-band EN-DC with overlapping DL bands for non-collocated synchronous deployment.

#### 7.5.2.1 Minimum Requirements for inter-band synchronous EN-DC The requirements in this clause apply as a reference for inter-band synchronous EN-DC. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell for inter-band synchronous EN-DC as shown in Table 7.5.2.1-1 1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC. For E-UTRA TDD-NR TDD inter-band synchronous EN-DC with overlapping DL bands, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.5.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16. Table 7.5.2.1-1 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 35.21. 15; 30; 35.21. 15; 60; 35.21. 15; 120; 35.21. <unchanged content omitted> #### 7.6.2.1 Minimum Requirements for inter-band synchronous EN-DC The requirements in this clause apply as a reference for inter-band synchronous EN-DC. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from an E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to SCG at the UE receiver for inter-band synchronous EN-DC as shown in Table 7.6.2.1-1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC. For E-UTRA TDD-NR TDD inter-band synchronous EN-DC with overlapping DL bands, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.6.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16. Table 7.6.2.1-1: Maximum receive timing difference requirement for inter-band synchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) (Note1); Maximum receive timing difference (µs). 15; 15; 33. 15; 30. 15; 60. 15; 120. Note 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}..

Proposal 2: Similar text proposals for MTTD/MRTD are captured in TS38.133 from Rel-16 for Type 2 NE-DC UE.

# 3. Conclusion

In this contribution, we provided our viewpoints on the RRM aspects for this work item, accordingly the following observations and proposals are obtained:

For Rel-16/17 Inter-band EN-DC with overlapping DL bands for non-collocated deployment:

Proposal 1: The following text proposals for MTTD/MRTD are captured in TS38.133 from Rel-16, to clarify Type-2 UE supporting Inter-band EN-DC with overlapping DL bands for non-collocated synchronous deployment.

#### 7.5.2.1 Minimum Requirements for inter-band synchronous EN-DC ... For E-UTRA TDD-NR TDD inter-band synchronous EN-DC with overlapping DL bands, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.5.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16. <unchanged content omitted> #### 7.6.2.1 Minimum Requirements for inter-band synchronous EN-DC ... For E-UTRA TDD-NR TDD inter-band synchronous EN-DC with overlapping DL bands, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and as shown in Table 7.6.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 2: Similar text proposals for MTTD/MRTD are captured in TS38.133 from Rel-16 for Type 2 NE-DC UE.

# 4. Reference

[1] RP-222309 (revision of RP-221809), “New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN#97-e.

[2] R4-2211795, “RAN4 RF/RRM work plan for NonCol_intraB_ENDC_NR_CA WI”, KDDI, RAN4#104-e.

[3] R4-2214458, “WF on NonCol_intraB_ENDC_NR_CA”, KDDI, RAN4#104-e.

[4] R4-2217733, “WF on NonCol_intraB_ENDC_NR_CA for NR-CA Type-2 UE”, KDDI, RAN4#104-bis-e.

[5] R4-2217734, “WF on NonCol_intraB_ENDC_NR_CA for NR-CA and EN-DC New Type UE”, KDDI, RAN4#104-bis-e.

[6] R4-2220537, “WF on the requirement for intra-band non-collocated CA/EN-DC”, KDDI, RAN4#105.

[7] R4-2220398, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, HiSilicon, Samsung, RAN4#105.

[8] R4-2303696, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN4#106.

[9] R4-2303225, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, RAN4#106.

[10] R4-2306605, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN4#106-Bis-e.

[11] R4-2306335, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, RAN4#106-Bis-e.

# Appendix-1: Rel-15 Inter-band EN-DC/NE-DC with overlapping DL bands (Same Content Provided in R4-2300448)

Firstly, it should be noted that the terminology “intra-band EN-DC” cause confusion, which has been corrected. The band combination under discussion is EN-DC band combination DC_42_n77: Because of the different band numbering systems for NR and LTE, DC_42_n77 is inter-band EN-DC with overlapping DL bands but essentially equivalent to intra-band EN-DC. This has been identified in RAN4#104-e (agreement in [4]) and the wording in WID has also been corrected in RAN#97-e.

The discussion on the necessity of non-collocated inter-band EN-DC with overlapping DL bands deployment can date back to Rel-15, during which operators proposed the possible non-collocated scenario, while due to time limitation RAN4 concluded the simple assumption, i.e., “only co-located deployment” is allowed. Accordingly, the following restriction is provided in Rel-15 RAN4 requirement, which forbid non-collocated deployment for inter-band EN-DC with overlapping DL bands: (1) MRTD ＜3us; (2) Power imbalance between downlink carriers is no larger than 6dB.

Observation A-1: For Rel-15 UE supporting inter-band EN-DC/NE-DC with overlapping DL bands:

- Only co-located deployment is applied;

- MRTD < 3us shall be applicable.

Next, we would like to examine the existing RAN4 requirement to clarify why 3us MRTD (captured in the clause for intra-band EN-DC) is applicable for Rel-15 Inter-band EN-DC with overlapping DL bands. Specifically, this can be based on the following clause in TS38.101-3:

#### 5.5B.4.1 Inter-band EN-DC configurations within FR1 (two bands) Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands) EN-DC configuration; Uplink EN-DC configuration (NOTE 1); Single UL allowed. .... DC_42A_n77A3,4,9 DC_42A_n77C3,4,9 DC_42C_n77A3,4,9 DC_42C_n77C3,4,9 DC_42D_n77A3,4,9 DC_42E_n77A3,4,9; N/A; N/A. DC_42A_n78A3,4,9 DC_42A_n78C3,4,9 DC_42C_n78A3,4,9 DC_42C_n78C3,4,9 DC_42D_n78A3,4,9 DC_42E_n78A3,4,9; N/A; N/A. .... ... NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band non-contiguous EN-DC apply. When UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. ... NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. ....

Observation A-2: With the clarification in Note 4 of Table 5.5B.4.1-1 in TS38.101-3, for the band combo of DC_42_77 or DC_42_78, the minimum requirements for intra-band non-contiguous EN-DC apply.

Observation A-3: With the clarification note in TS38.101-3 for DC_42_77 or DC_42_78, the applicable requirement for Rel-15 UE supporting inter-band EN-DC with overlapping DL bands shall be the below highlighted clause in section 7.6.3 of TS38.133:

### 7.6.3 Minimum Requirements for intra-band EN-DC For intra-band EN-DC, only co-located deployment is applied. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2]. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.3-1 for E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16]. Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) Note1; Maximum receive timing difference (µs). 15; 15; 3. 15; 30; 3. 15; 60; 3. NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}..

Proposal A-1: RAN4 shall acknowledge the clarification note in TS38.101-3 is the basis for 3us MRTD requirement (defined in section 7.6.3) being applicable for Rel-15 UE for DC_42_77 or DC_42_78 (i.e., inter-band EN-DC/NE-DC with overlapping DL bands).