3GPP TSG-RAN WG4 Meeting # 107	                                 	                        R4-2309365

# Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.5.3.1.2

Source:	Apple Inc.

Title:	Further Discussion on PDSCH Demodulation Performance Requirements for 8Rx UEs in FR1

Release:	Rel-18

Document for:	Discussion

# Introduction

Following the discussion at RAN4#106bis-e, we use this contribution to clarify our position on the still unresolved issues. We refer to our results regarding PDSCH demodulation requirements for UEs supporting 8Rx based on last meeting agreements (for detailed simulation results, please refer to [2]). Finally, we want to point out that no impairments have been considered yet. This WI is focused on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers

# Discussion

During the discussion and its outcome captured in [1], we continue here with the unresolved items on PDSCH demodulation performance requirements and the scenarios we will consider for the final requirements. From our point of view, despite meaningful work on down-selecting parameters, we still need to settle on Rank-2, Rank-4 and Rank-8 scenarios.

Propagation conditions for Rank 2 test

- Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B

- Option 2: Only TDLC300-100 ULA Medium B

From our point of view, only a single propagation scenario is needed. This, since the WI should focus on high number of layers and high spectral efficiency. Previously, it was agreed that since the target UEs for this WI are CPE/FWA/vehicle/industrial devices, one way to address the vehicle use case would be to define a requirement targeting low-rank high-delay high-doppler scenarios. This is well address by TDLC300-100 ULA Medium B.

Observation#1: Vehicle propagation scenario is well address by a low-rank high-delay high-doppler propagation. No other relevant scenarios are needed for Rank-2.

Proposal#1: Define TDLC300-100 ULA Medium B as single propagation scenario for Rank-2 vehicle operation.

MCS and antenna correlation for Rank 2 test

- Proposals for TDLC300-100 ULA Medium B

  - Option 1: MCS 2, 7 (Table 2)

  - Option 2: MCS 19 (Table 1)

During RAN4#106 and RAN4#106bis-e it was agreed that a single additional requirement would be added to cover each of the considered down-selected number of MIMO layers: Rank-2, Rank-4 and Rank-8. Due to this, we believe that Option 2 MCS19 is what better represents our previous agreement.

Proposal#2: Define only MCS19 (Table 1) for the Rank-2 TDLC300-100 ULA MediumB requirement.

MCS for Rank 4 test

- Option 1: MCS 13 and MCS26 (Table 1)

- Option 2: Only MCS17 (Table 1)

On a similar note from RAN4#106 and RAN4#106bis-e, we believe Option 2: Only MCS17 is what better represents our previous agreement.

Proposal#3: Define only MCS17 (Table 1) for Rank-4 TDLC30-10 ULA Low requirement.

Finally, with respect to the Rank-8 requirement, we are rather indifferent on whether MCS13 or MCS17 should be used, since both values represent reasonable SNR points.

Single MCS configuration for Rank 8 test

- Option 1: MCS13

- Option 2: MCS17

Proposal#4: Define either MCS13 (Table 1) or MCS17 (Table 1) for Rank-8 TDLC30-10 ULA Low requirement.

# Conclusion

In this contribution, we provide our views on the remaining unresolved issues for 8Rx UE demodulation requirements. Our observations and proposals are summarized as below:

Observation#1: Vehicle propagation scenario is well address by a low-rank high-delay high-doppler propagation. No other relevant scenarios are needed for Rank-2.

Proposal#1: Define TDLC300-100 ULA Medium B as single propagation scenario for Rank-2 vehicle operation.

Proposal#2: Define only MCS19 (Table 1) for the Rank-2 TDLC300-100 ULA MediumB requirement.

Proposal#3: Define only MCS17 (Table 1) for Rank-4 TDLC30-10 ULA Low requirement.

Proposal#4: Define either MCS13 (Table 1) or MCS17 (Table 1) for Rank-8 TDLC30-10 ULA Low requirement.

# References

- R4-2305888, “WF on 8RX UE performance requirements”, Huawei, HiSilicon

- R4-2309368, “Further Simulation Results for PDSCH Demodulation Requirements for 8Rx UEs”, Apple Inc.