3GPP TSG-RAN WG4 Meeting # 107	R4-2309372

# Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.12.4

Source:	Apple Inc.

Title:	Further Discussion on Demodulation Requirements of Type-2 UEs in Intra-band Non-collocated EN-DC/NR-CA

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

The Intra-band Non-collocated EN-DC/NR-CA WI was approved during RAN#95 meeting. The focus of this WI is to study the feasibility of supporting non-collocated scenarios, including the minimum PDSCH performance requirements. The work has been delimited to bands n77/n78. This is, band combinations DC_42_n77/n78 for EN-DC, and CA_n77(2A) and CA_n78(2A) for NR-CA. About minimum PDSCH performance requirements, they should be obtained based on the agreed conditions of applicable maximum received time difference (MRTD) requirement and maximum power imbalance between component carriers.

# 2. Discussion

RAN4 RF core part has discussed a number of types of UE for this scenario. Details are included in the table below:

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

The Type-3 UE is now descoped from Rel-18 since there isn't enough time to thoroughly examine the system assumptions, complexities, and needed performance requirements. As a result, some argue that the WI should keep working on defining standards for Type-2 and Type-4 UEs. We feel that we should only decide performance requirements for Type-2 UEs because Type-4 UEs have a different form factor and may require extra assumptions.

Proposal 1: RAN4 should discuss the need for demodulation requirements only for Type-2 UEs

Furthermore, the power imbalance test in 38.101-4, "Minimum requirements for carrier aggregation with power imbalance" -- 6dB between PCell and SCell -- is intended for Intra-band Non-contiguous. The requirements in this section are based on Type-1 UE, with the fundamental assumption being that the UE shares a single RF component (e.g., LNA, mixier, AGC, etc.) to receive two carriers. In this type of RF design, the UE adjusts the LO frequency between two carriers, where the image of one component carrier falls into the other component carrier. As a result, performance degradation owing to power imbalance is indeed expected.

Observation 1: Existing test cases in 38.101-4 with 6dB power imbalance between SCell and PCell are only meant for Type-1 UEs.

However, during the remainder of this WI, we will analyze the need for additional UE demodulation requirements for Type-2 UEs. Here, Type-2 UEs are capable for independent RF component to receive two independent component carriers. In this way, the test falls back to a typical CA test, but now with the applicability of MRTD and Power Imbalance values that have been decided before: 33us and 25dB.

Observation 2: Type-2 UE performance should be measured as any EN-DC NR-CA tests case, with the only difference of the extra requirements of MRDT and Power Imbalance.

Propose 2: For Type-2 UE, set the received time difference between two carriers to MRTD = 33 us for PDSCH demodulation requirements.

Proposal 3: For Type-2 UE, choose a set of MCSs such that the difference of required SNR to achieve XX% of the maximum throughput is equal or less than 25dB.

Proposal 4: For Type-2 UE, consider 70% of the maximum throughput as the figure of merit

See a similar example taken from Table 8.2.1.1.1-4 in 36.101.

In addition to this, we want to stress out that the same procedure for CA should be applied here. This means, one carrier is measured at a time. No aggregate throughput is needed to be measured. This would also create some strange precedent how CA tests have been conducted in the past.

Observation 3: Previous EN-DC NR-CA tests will always measure the per-carrier throughput, measuring only one carrier at a time.

Subsequently, our previous discussion and proposals should also apply to the EN-DC scenario. The only difference here is that the throughput is only measured in the NR carrier and not in both. Otherwise, the scenario is the same.

Proposal 5: Extend the same assumptions for the EN-DC case. Here, current practice is only to measure throughput from the NR carrier.

Finally, during the upcoming RAN4#107 meeting, we should also discuss whether to define these performance requirements at all. Some companies have brought the issue that RF will define REFSENS tests for Type 2 UE. Hence, it’s important to ask ourselves the hard questions of what would be the extra value that an additional test would bring, to the overall verification activities.

Proposal 6: RAN4 to discuss how much of the overall goal can be achieved via REFSENS tests, whether we need to consider dedicated demodulation tests, and how to properly incorporate the 33us MRTD aspect.

# 4. Conclusion

Our observations and proposals about Type-2 Performance Requirements for Intra-band Non-collocated EN-DC/NR-CA are summarized below:

Proposal 1: RAN4 should discuss the need for demodulation requirements only for Type-2 UEs

Observation 1: Existing test cases in 38.101-4 with 6dB power imbalance between SCell and PCell are only meant for Type-1 UEs.

Observation 2: Type-2 UE performance should be measured as any EN-DC NR-CA tests case, with the only difference of the extra requirements of MRDT and Power Imbalance.

Propose 2: For Type-2 UE, set the received time difference between two carriers to MRTD = 33 us for PDSCH demodulation requirements.

Proposal 3: For Type-2 UE, choose a set of MCSs such that the difference of required SNR to achieve XX% of the maximum throughput is equal or less than 25dB.

Proposal 4: For Type-2 UE, consider 70% of the maximum throughput as the figure of merit

Observation 3: Previous EN-DC NR-CA tests will always measure the per-carrier throughput, measuring only one carrier at a time.

Proposal 5: Extend the same assumptions for the EN-DC case. Here, current practice is only to measure throughput from the NR carrier.

Proposal 6: RAN4 to discuss how much of the overall goal can be achieved via REFSENS tests, whether we really need to consider dedicated demodulation tests, and how to properly incorporate the 33us MRTD aspect.

# References

- R4-2305891	WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario

- R4-2304086 Work plan for demodulation performance part of WI intra-band non-colocated EN-DC/NR-CA deployment

- R4-2305989, Email discussion summary for [106bis-e][320] NonCol_intraB_ENDC_NR_CA_Demod, Ericsson