3GPP TSG-RAN4 Meeting #107	R4-2309426

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on improvement on ScellSCG setup delay.

Agenda item:	8.25.4

Document for:	Discussion

# Introduction

In this paper, we share our view on the idea of measurement during Idle/Inactive mode for fast SCell/SCG setup.

# Discussion

Solution based on existing measurement

Issue 2-2-1: overall solution for UE which is configured with EMR measurement. - Agreements: - FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. Issue 2-2-3: definition of ‘valid’ in solution based on existing measurement - Agreements: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition.

When UE report measurement results for existing measurement, the measurement result is valid when UE meet the measurement delay and accuracy requirements. It should be also applied to mobility scenario. Since measurement results are filtered from previous data UE can decide the measurement result can be reported or not. Even if EMR capable UE, it is up to UE implementation performing measurement after T331 expiry. UE can make own decision how frequently perform measurement depending on filtered measurement results. Some UE may need small number of samples, or some UE may need more number of samples during IDLE/INACTIVE state. Therefore, RAN4 cannot specify any time constraint to evaluate whether measurement result is valid or not.

Some companies propose to use variation of serving cell RSRP/RSRQ. However, it cannot be directly applicable due to different radio characteristics between FR1 and FR2.

Observation : Depending on required number of samples to measure, UE may perform measurement less frequently during IDLE/INACTIVE state while meeting the measurement delay and accuracy requirements. However, the measurement results can be considered as Invalid if any time constraint is defined for valid measurement between time to perform measurement and reporting. FR1 Serving cell RSRP/RSQ variation is difficult to apply to estimate FR2 variation due to different radio characteristics.

Proposal :  RAN4 does not need to define any additional criteria to check valid measurement results other than measurement delay and accuracy requirements.

Some companies propose that indication of availability or validity indication is required. From our understanding, whether UE report measurement results or not is enough to let NW make decision for following steps. When UE has measurement results to report, the UE does not need to indicate “measurement result is valid/available”. The UE can report the measurement results.

Moreover, there is no benefit let NW to know the UE had measurement results but somehow the UE decide not to report. UE just not report in such case.

Even for UE who perform enhanced measurement, when UE does not report at certain occasion it can imply UE continues performing the measurement.

As some companies proposed during the last meeting we agree that UE is able to make decision whether the current measurement results are valid or not, the necessity of introducing additional indication is not observed.

Observation: Reporting measurement results or not is enough for NW to make decision for following steps.

Proposal: No need to define indication of valid/available measurement results.

Solution based on enhanced measurement

Issue 2-3-2: feasibility of enhanced measurement which starts from RRC setup/resume procedure. - Candidate solutions: - FFS: Feasibility of additional measurement during RRC connection setup is questionable. For instance, Rx beam sweeping is still needed for additional measurement during RRC connection setup unless target cell has been measured recently. However, if target cell has been measured recently before RRC connection setup, UE could directly send that measurement result, instead of doing another round of measurement. - FFS: It is not clear how much gain would be achieved by allowing UE perform enhanced measurement just dozens of milliseconds in advance, compared with the case network configure MO upon UE enters to connected mode. Moreover whether UE can acquire a complete measurement result during dozens of milliseconds is questionable. Issue 2-3-3: definition of ‘valid’ for solutions based on enhanced measurement - Candidate solutions: - Option 1: If the time span from the ending point of EMR measurement (T1) to starting point of additional measurement (T2) is less than [Tvalid] seconds, the measurement result obtained during IDLE/INACTIVE mode can be regarded as valid and useful result. - Option 2: - Available IDLE/INACTIVE measurements are considered valid, if they fulfil L3 measurement accuracy requirements. - When the UE has been stationary since the last measurement (no time limitation), measurement results can be considered valid and re-evaluation round can be quick. - UE shall not report measurements that do not fulfil the validity criteria. UE can still report the availability status about ongoing measurements.

Some companies raise concern about UE behavior upon receiving new MO in RRC CONNECTED state. We agree that UE shall follow new MO in RRC CONNECTED state however it does not mean the enhanced measurement from RRC IDLE/INACTIVE state cannot be continued. Such concern can be resolved from NW configuration perspective. If NW provide the measurement information for enhanced measurement at previous RRC CONNECTED state and NW knows UE is performing measurement from RRC IDLE/INACTIVE state, then NW can configure the MO for the same target frequency that UE is measuring. The UE shall be able to continue performing measurement after RRC CONNECTED state.

Observation 3: UE behavior is clear when NW configure MO properly as following example scenario:

- NW provide the measurement information for enhanced measurement at RRC configuration before IDLE/INACITVE state

- NW knows UE is performing measurement after RRC CONNECTED state from RRC IDLE/INACTIVE when NW triggered the enhanced measurement but UE does not report during RRC IDLE/INACTIVE (e.g in msg5)

- NW configures the MO at RRC CONNECTED for the same target frequency that UE is measuring.

- The UE can continue performing measurement even after RRC CONNECTED state.

UE expect to perform FR2 enhanced measurement without gap when PCell is FR1.

Proposal 3: There is no feasibility issue to continue performing enhanced measurement after RRC CONNECTED state from configuring proper MO such as

- If NW provide explicit measurement information for enhanced measurement, NW configure MO at RRC CONNECTED state for the same target frequency that UE is performing from RRC IDLE/INACTIVE.

For UE who perform enhanced measurement, the measurement result is valid when UE meet measurement delay and accuracy requirements. No need to define additional criteria to determine valid measurements.

Proposal 4: Measurement result is valid when UE meet the measurement delay and accuracy requirements. No need to define additional criteria.

Issue 2-3-4: starting point of the enhanced measurement - Candidate solutions: - Option 1: - In case of MT-call, NW can trigger UE to perform FR2 enhanced measurement. The triggering command can be included in paging. (Note: signaling details up to RAN2). - In case of MO-call, UE determine whether performing additional/enhanced measurement is required. - UE initiate additional/enhanced measurement when volume of MO-data exceeds the threshold. Otherwise, the measurement is not required. (Note: signaling details up to RAN2). - NW can provide threshold information of MO data volume for fast CA/DC setup upon RRC release - Option 2: - Validation of available IDLE/INACTIVE mode measurements may start at RRC connection setup/resume. - Option 3: - Whether to perform addition measurement starting from RRC connection setup/resume procedure can be up to UE implementation. - Option 4: - For MT originating call, UE starts to perform additional measurement after paging reception. And for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1. Issue 2-1-3: Scell delay improvement target - Candidate solutions: - FFS: RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay) Issue 2-3-5: ending point of the enhanced measurement - Candidate solutions: - The ending point of the enhanced measurement is FFS - Option 1: When UE sends RRCResumeComplete or SecurityModeComplete - Option 2: At the reception of the RRC CONNECTED mode measurement configuration (the 1st RRC_reconfiguration message) - Option 3: During RRC CONNECTED state Issue 2-1-6: others - Candidate solutions: - FFS: RAN4 shall clarify that the Idle/Inactive measurement ending point to improve Scell/SCG setup delay shall be at the reception of the connected mode measurement configuration, here say for example upon receiving the 1st RRC_reconfiguration message. (E///)

As some companies propose the separate starting point of measurement for MT call or for MO call. However, to define measurement requirement simple, we prefer to define unified measurement requirement for both MT and MO call.

Proposal 5: RAN4 can consider defining unified delay requirement for both MT and MO call, starting from after Msg1 transmission.

From our understanding, the definition of delay is Scell setup delay from RRC setup/resume complete. This delay excludes RRC setup/resume delay. If NW has measurement report or measurement report available indication in msg5, it is up to NW implementation when Scell setup is initiated from receiving the measurement report.

Observation 4: The definition of delay is Scell setup delay from RRC setup/resume complete. The delay exclude RRC setup/resume delay.

Proposal 6: RAN4 does not need to specify where the delay is less than 100ms, even close to 20ms scenario. It is up to NW implementation.

As long as UE meet measurement delay requirement, the ending point within measurement period of enhanced measurement ending is up to UE implementation. Some UE may have measurement results much earlier than measurement delay requirement, some UE may have valid measurement result at the end of measurement period.

Observation 5: It is up to UE implementation when finish measurement within measurement delay requirements. Therefore, it is not proper to discuss about ending point of the enhanced measurement. But RAN4 shall define measurement delay requirements starting from msg1 transmission.

Proposal 7: RAN4 shall define measurement delay requirement instead of defining measurement ending point.

Issue 2-3-6: Reporting of the measurement results - Candidate solutions: - FFS: Validation / re-evaluation measurement results should be made available to the network as soon as the validation has been completed regardless of the maximum validation delay. - FFS: Reporting of the validated measurements should be enhanced to allow the UE to report early measurement results as soon as validation is completed. RAN4 to inform RAN2 about the need for such enhancement. Issue 2-3-7: network assistant information - Candidate solutions: - FFS: NW shall provide explicit information such as target frequency and/or Cell ID, and/or target SSB info for additional/enhanced measurement on FR2. The frequency/band lists can be overlapped with the list of cell reselection or EMR. Signaling details are up to RAN2. - FFS: NW can provide explicit information such as target frequency and/or Cell ID, and/or target SSB info for measurements in RRC configuration before idle mode or SIB. - FFS: For the additional measurement, candidate frequency information can be provided by NW. - FFS: NW can provide threshold information of MO data volume for fast CA/DC setup upon RRC release.

To perform enhanced measurement for fast Scell/SCG setup, UE shall know the target frequency to measure. Otherwise, the measurement results may not be useful to NW. Therefore, NW shall provide measurement information explicitly. The measurement information can include target frequency, Cell ID, and SSBinfo. It can be provided in RRC configuration or SIB.

Observation 6: If UE does not receive measurement information from NW, the measurement results may not be useful to NW for initiating fast Scell/SCG setup.

Proposal 8: NW shall provide explicit information for enhanced measurement. The measurement information can include target frequency, Cell ID, target SSB info.

If UE has available measurement results then UE can report the measurement results in msg5 for inactive state or UE can indicate availability of measurement report in msg5 for Idle state. By assuming NW trigger the additional measurements, NW should know UE behavior by checking measurement results/ indication in msg5. If UE does not report, NW expect UE will continue performing measurement even after RRC CONNECTED state. Then NW can configure reporting configuration at RRC CONNECTED state.

Observation 7: UE can follow existing measurement report procedure if measurement result is available. such as UE can report measurement results in msg5 during INACTIVE state or UE can indicate measurement report is available in msg5 and report after RRC security complete.

Proposal 9: UE can report measurement results in msg5 during INACTIVE state or UE can indicate measurement report available in msg5 and report after RRC security complete if measurement result is available.

Proposal 10: NW expect the UE continues performing measurement even after RRC CONNECTED, if NW does not receive measurement report or does not receive measurement report available indication in msg5.

Issue 2-3-8: number of samples, including whether Rx beam sweeping is needed - Candidate solutions: - Option 1: - Enhanced measurement requires at least 8 SSB samples and additional [X] samples of SSB burst. (FFS on X, e.g 2) - Option 2: - Number of samples UE needs to measure can be depending on UE radio conditions and measurement conditions. - During re-evaluation/validation measurements, UE is not expected to perform full beam-sweeping and hence, the scaling factor associated with the beam sweeping can be reduced. - Option 3: - During the additional measurement, for the further validity check, reduced samples and (or) reduced beam sweeping factors can be considered for the results obtained within the last [X] seconds. Reduced samples or reduced beam sweeping factors may not be considered for results obtained more than [X] seconds ago. Also, for the results obtained within the last [Y] (Y<X) seconds, it can be treated as valid results without additional measurements. - Option 4: - Both introducing UE capability for lower Rx beam sweeping factor like Rel-17 positioning and using the previous beam information obtained in the early measurement can be the candidate options. - Option 5: - As baseline, RAN4 shall not reduce the scaling factor of Rx beam sweeping when defining requirements for the new measurement during RRC connection setup/resume. - Option 5a: - To guarantee the measurement accuracy, the measurement samples are not supposed to be reduced. Not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure. Issue 2-3-10: periodicity of reference signal - Candidate solutions: - Option 1: - Enhanced measurement period can be based on SSB period instead of SMTC for the frequency. SSB period can be provided by NW or default SSB period (20ms) can be applied. - Option 1a: - Enhanced measurement period can be based on SSB period instead of SMTC. - Option 2: FFS

To avoid additional measurement accuracy issue, beam sweeping factor reduction is not preferred. However, UE can use SSB periodicity if NW provide SSB information on target band.

Proposal 11 : RAN4 does not consider beam sweeping factor reduction during enhanced measurements.

Proposal 12 : RAN4 consider SSB period for the enhanced measurement. The measurement period requirements can be defined as (8 + [X]) TSSB ms.

Issue 2-3-12: applicability rules to perform enhanced measurement - Candidate solutions: - Option 1: - The enhanced measurement is not applied when PCell is FR2. - The enhanced measurement is applied when target cell SNR > [Y] dB (e.g Y = 6 dB). - UE is allowed to stop enhanced measurement upon RA procedure problem (e.g msg2 reception failure). - Option 2: FFS

Although performing FR2 measurement is not interrupting to FR1 serving cell, there is an issue upon RA procedure based on PCell condition such as UE fail to receive msg2 or decode msg2 transmission. Then, UE should prioritize RA procedure instead of performing FR2 measurement. Also, less number of samples can be supported when the additional measurement for the target FR2 cell performed in cell  good condition (e.g SNR > 6 dB).

Proposal 13: RAN4 shall define applicability rules to perform enhanced measurement.

  - The enhanced measurement is applied when target cell SNR > [Y] dB (e.g Y = 6 dB).

  - UE allowed to stop enhanced measurement upon RA procedure problem (e.g msg2 reception failure).

Issue 2-3-1: overall solution - Candidate solutions: - Option 1: - NW provides information for additional measurements. It can be preconfigured upon RRC release, or it can be broadcasted. - Additional measurement is triggered upon MO/MT call under certain conditions. - UE perform additional measurements after transmitting msg1 and measurement can be done during RRC connected. - Option 2: FFS

Overall solution

- NW provide explicit measurement information for enhanced measurement.

- NW trigger enhanced measurement based on NW demands.

- UE start performing additional measurement from msg1 transmission

- If UE has measurement results to report, UE can report in msg5. NW can initiate SCell/SCG setup.

- If UE does not report in msg5, NW expect UE continue performing measurement for remaining measurement period. NW configure the MO at RRC CONNECTED for the same target frequency that UE is performing measurement during IDLE/INACTIVE state.

- NW can initiate SCell/SCG setup after UE report enhanced measurement results.

# Conclusions

Observation 1: Depending on required number of samples to measure, UE may perform measurement less frequently during IDLE/INACTIVE state while meeting the measurement delay and accuracy requirements. However, the measurement results can be considered as Invalid if any time constraint is defined for valid measurement between time to perform measurement and reporting. FR1 Serving cell RSRP/RSQ variation is difficult to apply to estimate FR2 variation due to different radio characteristics.

Proposal 1:  RAN4 does not need to define any additional criteria to check valid measurement results other than measurement delay and accuracy requirements.

Observation 2: Reporting measurement results or not is enough for NW to make decision for following steps.

Proposal 2: No need to define indication of valid/available measurement results.

Observation 3: UE behavior is clear when NW configure MO properly as following example scenario:

- NW provide the measurement information for enhanced measurement at RRC configuration before IDLE/INACITVE state

- NW knows UE is performing measurement after RRC CONNECTED state from RRC IDLE/INACTIVE when NW triggered the enhanced measurement but UE does not report during RRC IDLE/INACTIVE (e.g in msg5)

- NW configures the MO at RRC CONNECTED for the same target frequency that UE is measuring.

- The UE can continue performing measurement even after RRC CONNECTED state.

UE expect to perform FR2 enhanced measurement without gap when PCell is FR1.

Proposal 3: There is no feasibility issue to continue performing enhanced measurement after RRC CONNECTED state from configuring proper MO such as

- If NW provide explicit measurement information for enhanced measurement, NW configure MO at RRC CONNECTED state for the same target frequency that UE is performing from RRC IDLE/INACTIVE.

Proposal 4: Measurement result is valid when UE meet the measurement delay and accuracy requirements. No need to define additional criteria.

Proposal 5: RAN4 can consider defining unified delay requirement for both MT and MO call, starting from after Msg1 transmission.

Observation 4: The definition of delay is Scell setup delay from RRC setup/resume complete. The delay exclude RRC setup/resume delay.

Proposal 6: RAN4 does not need to specify where the delay is less than 100ms, even close to 20ms scenario. It is up to NW implementation.

Observation 5: It is up to UE implementation when finish measurement within measurement delay requirements. Therefore, it is not proper to discuss about ending point of the enhanced measurement. But RAN4 shall define measurement delay requirements starting from msg1 transmission.

Proposal 7: RAN4 shall define measurement delay requirement instead of defining measurement ending point.

Observation 6: If UE does not receive measurement information from NW, the measurement results may not be useful to NW for initiating fast Scell/SCG setup.

Proposal 8: NW shall provide explicit information for enhanced measurement. The measurement information can include target frequency, Cell ID, target SSB info.

Observation 7: UE can follow existing measurement report procedure if measurement result is available. such as UE can report measurement results in msg5 during INACTIVE state or UE can indicate measurement report is available in msg5 and report after RRC security complete.

Proposal 9: UE can report measurement results in msg5 during INACTIVE state or UE can indicate measurement report available in msg5 and report after RRC security complete if measurement result is available.

Proposal 10: NW expect the UE continues performing measurement even after RRC CONNECTED, if NW does not receive measurement report or does not receive measurement report available indication in msg5.

Proposal 11: RAN4 does not consider beam sweeping factor reduction during enhanced measurements.

Proposal 12: RAN4 consider SSB period for the enhanced measurement. The measurement period requirements can be defined as (8 + [X]) TSSB ms.

Proposal 13: RAN4 shall define applicability rules to perform enhanced measurement.

  - The enhanced measurement is applied when target cell SNR > [Y] dB (e.g Y = 6 dB).

  - UE allowed to stop enhanced measurement upon RA procedure problem (e.g msg2 reception failure).

Proposal: Overall solution based on enhanced measurement.

- NW provide explicit measurement information for enhanced measurement.

- NW trigger enhanced measurement based on NW demands.

- UE start performing additional measurement from msg1 transmission.

- If UE has measurement results to report, UE can report in msg5. NW can initiate SCell/SCG setup.

- If UE does not report in msg5, NW expect UE continue performing measurement for remaining measurement period. NW configure the MO at RRC CONNECTED for the same target frequency that UE is performing measurement during IDLE/INACTIVE state.

- NW can initiate SCell/SCG setup after UE report enhanced measurement results.

# References

[1] R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple, RAN4 #106bis-e.