3GPP TSG-RAN4 Meeting #107	R4-2309429

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Agenda item:	10.1.2

Document for:	Discussion

# Introduction

In RAN2 LS (R2-2304562), it indicates misalignment between RAN2 specification and RAN4 specification about UE behavior if a paging occasion conflicts with a CG-SDT occasion.

# Discussion

In latest TS38.133 [1], RAN4 specified that UE is allowed to drop the CG-SDT transmission and monitor if paging occasion overlaps with CG-SDT transmission for RedCap UE in HD-FDD mode.

#### 5.1B.2.6 Maximum interruption in paging reception The requirements in clause 4.2B.2.6 shall apply for RedCap UEs. For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

From our understanding, HD-UE does not expect overlapping between any paging occasion and CG-SDT occasions according to clause 17.2 at TS38.213 [2].

[TS38.213, 17.2 Half-Duplex UE in paired spectrum] A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.

Observation: HD-UE does not expect overlapping between any paging occasion and CG-SDT occasions based on clause 17.2 at TS38.213. NW should avoid configuring any paging occasions and any CG-SDT transmission at the same occasions for a HD-FDD RedCap UE.

Proposal 1: NW should not configure CG-SDT occasions overlaps with all paging occasions for a HD RedCap UE.

Proposal 2: If NW configure CG-SDT occasions overlaps with any paging occasions, it is up-to UE implementation whether to monitor the paging during the overlapping paging occasions.

# Conclusion

Observation: HD-UE does not expect overlapping between any paging occasion and CG-SDT occasions based on clause 17.2 at TS38.213. NW should avoid configuring any paging occasions and any CG-SDT transmission at the same occasions for a HD-FDD RedCap UE.

Proposal 1: NW should not configure CG-SDT occasions overlaps with all paging occasions for a HD RedCap UE.

Proposal 2: If NW configure CG-SDT occasions overlaps with any paging occasions, it is up-to UE implementation whether to monitor the paging during the overlapping paging occasions.

# References

[1] TS 38.133 V17.9.0

[2] TS 38.213 V17.5.0

[3] R2-2304562, LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs, Ericsson, RAN2 #121bis-e