3GPP TSG-RAN WG4 Meeting # 107							              R4-2309439

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.14.2.2

Source: 	CMCC

Title: 	Discussion on ATG UE Tx requirements

Document for:	Discussion

# 1. Introduction

After RAN4 #106-bis meeting, a good process had been reached for ATG UE Tx requirements. In the WF [1], there are still some issues that need to be discussed in this meeting. This paper will focus on these issues.

# 2. Discussion

## 2.1. ATG UE types

During the #106-bis meeting, we reached an agreement that introduce two UE types to distinguish the antenna types for ATG UE. In order to define two UE types, we suggest adding two UE types as shown in Table 1, and new suffix information to TS38.101-1. In this way, two ATG UE types for different antenna types could be defined, and two sets of RF requirements for ATG UE could be defined later in the TS. We also welcome other definitions of ATG UE types to be discussed during this meeting.

Table 1. Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation(CA) with UL MIMO |
| I | RedCap |
| J | ATG UE (with omni-direction antenna) |
| K | ATG UE (with antenna array) |

Proposal 1: Suggest defining two ATG UE types as shown in Table 1, and other definitions of ATG UE types are also welcome to be discussed during this meeting.

## 2.2. Minimum output power

Last meeting, we have the WF:

<Way forward>: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

In the ATG network aircraft altitude to derive cell coverage is 3km. However, as ATG network deployment in figure 1, the minimum distance between ATG BS and UE is not 3km. Because the beam of the ATG BS in Figure 1 will not transmit vertically upwards and the ATG BS will not serve the aircraft directly above. Considering the following table 1, the minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

Observation 1: As ATG network deployment in figure 1, the minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

Table 2. minimum distance for Figure 1

|  | Non Sub-array model | Sub-array model |
| --- | --- | --- |
| Angle of aircraft at minimum altitude and maximum distance from ATG BS | 1.5° | 1.5° |
| BS vertical coverage range | 25° | 10° |
| Mechanical uptilt | 14° | 6.5° |
| The minimum distance (km) | 6.7 | 15.1 |

In our opinion, considering that the possibility of ATG serving the aircraft directly above in the future still exists, we could use 3km as the minimum distance between ATG UE from BS.

Proposal 2: Considering the possibility of ATG serving the aircraft directly above in the future still exists, 3km could be used as the minimum distance between ATG UE from BS.

Figure 1. ATG network deployment

After determining the minimum distance as 3km between ATG BS and ATG UE, the minimum output power could be calculated. The calculation parameters and results were shown in table 3, -13dBm/MHz and -17dBm/MHz could be used as the minimum output power. Based on these calculations, the minimum output power requirement of -25 dBm for 2 GHz and -20 dBm for 4 GHz should be enough.

Table 3. Minimum output power for ATG UE Tx

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | 3km | 3km |
| PL | 108dB | 114dB |
| ATG BS antenna gain | 25dBm | 25dBm |
| ATG UE antenna gain | 0dBm | 17dBm |
| Thermal noise | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | -13dBm/MHz | -17dBm/MHz |

Proposal 3: -25dBm/MHz could be used as the minimum output power for 2GHz and -20dBm/MHz could be used for 4GHz.

## 2.3. Transmit modulation quality

Figure 2. CL CDF results for ATG UL 2GHz

Figure 3. CL CDF results for ATG UL 4GHz

We have given the link budget result for ATG UE 256QAM, and the result shows that 256 could be supported. Figure 2 and Figure 3 show the CL CDF results for ATG UL. Based on the result, 2GHz CPE UL with 36dbc has 25% probability for 256QAM and CPE with 4GHz has 29% probability for 256QAM. Consider of these results, 256 QAM should be supported for ATG UE.

Proposal 4: 256 QAM should be supported for ATG UE.

## 2.4. P-Max

Table 4. P-Max information element

-- ASN1START -- TAG-P-MAX-START P-Max :: = INTEGER (-30..33) -- TAG-P-MAX-STOP -- ASN1STOP

The P-Max in TS 38.331 is INTEGER (-30..33), which could represent 6 bits. But consider of the output power for ATG, at least 7 bits are needed. So the P-Max could not be reused, and the P-Max for ATG could be define as INTEGER (-63..64).

Proposal 5: The P-Max for ATG could be define as INTEGER (-63..64).

# 3. Conclusions

In this contribution, ATG UE Tx requirements are discussed with the following proposals:

Proposal 1: Suggest defining two ATG UE types as shown in Table 1, and other definitions of ATG UE types are also welcome to be discussed during this meeting.

Table 1. Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation(CA) with UL MIMO |
| I | RedCap |
| J | ATG UE (with omni-direction antenna) |
| K | ATG UE (with antenna array) |

Observation 1: As ATG network deployment in figure 1, the minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

Proposal 2: Considering the possibility of ATG serving the aircraft directly above in the future still exists, 3km could be used as the minimum distance between ATG UE from BS.

Proposal 3: -25dBm/MHz could be used as the minimum output power for 2GHz and -20dBm/MHz could be used for 4GHz.

Proposal 4: 256 QAM should be supported for ATG UE.

Proposal 5: The P-Max for ATG could be define as INTEGER (-63..64).

# 4. References

[1] R4-2306609, WF on ATG UE RF requirements, Huawei, HiSilicon