3GPP TSG-RAN WG4 Meeting # 107							              R4-2309443

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.36.2

Source: 	CMCC

Title: 	Discussion on NR UAV requirements

Document for:	Discussion

# 1. Introduction

In the RAN#99 meeting, the last WID [1] on NR support for UAV has added RAN4 as a secondary WG to study and specify necessary UE types and additional OOBE requirements for aerial UEs. Last meeting we made good progress in the WF [2]. This paper will continue to discuss other issues.

# 2. Discussion

2.1. UAV Abbreviation

At the last meeting, the following requirement was reached for UAV.

Agreement: RAN4 shall define and specify specific requirements in TS 36.101 (LTE) and TS 38.101-1 (NR) for aerial UEs

And the CR for TS 38.101-1 on additional OOBE requirements for aerial UEs was approved at the last meeting, but there is no agreement on the definitions for Aerial UE (UAV). In our opinion, the difference between UVA and regular UE is just the OOBE requirement for CEPT countries. Consider of the objective in the WID that specify the signaling to support subscription-based aerial-UE identification, we think the NR UAV could be defined as LTE.

Proposal 1: We propose to define UAV definition in the NR Spec as:

Aerial UE (UAV): The UE with an aerial subscription.

We have reached an agreement on regional requirements that should be considered. In our opinion, the ECC OOBE requirements only apply to UAVs, and all UAVs with support for specific NS defined for the requirements. This NS will be based on regional requirements. Considering it is the last meeting for this topic, the NS only applies to UAVs should be defined at this meeting, and we define an NS for UAVs as below.

Proposal 2:

##### Requirement for network signalling value "NS_[TBD1]" When "NS_[TBD1]" is indicated in the cell, the power of any UE emission for channels assigned within 1710-1785 MHz shall not exceed the levels specified in Table 6.5J.1-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.1-1: Additional requirements within 1710-1785 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 1675 ≤ f ≤ 1710; -40; 1 MHz. NOTE: This NS_[TBD1] is only applied for Aerial UE in CEPT region.. ##### Requirement for network signalling value "NS_[TBD2]" When "NS_[TBD2]" is indicated in the cell, the power of any UE emission for channels assigned within 2500-2570 MHz shall not exceed the levels specified in Table 6.5J.2-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.2-1: Additional requirements within 2500-2570 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 2690 ≤ f ≤ 2900; -50; 1 MHz. NOTE: This NS_[TBD2] is only applied for Aerial UE in CEPT region.. ##### Requirement for network signalling value "NS_[TBD3]" When "NS_[TBD3]" is indicated in the cell, the power of any UE emission for channels assigned within 2570-2620 MHz shall not exceed the levels specified in Table 6.5J.3-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.3-1: Additional requirements within 2570-2620 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 2690 ≤ f ≤ 2900; -50; 1 MHz. NOTE: This NS_[TBD3] is only applied for Aerial UE in CEPT region..

2.2. Power class

<Agreement >: • RAN4 shall further discuss whether the existing UE power classes can be applied for UAVs or UAV specific power class(es) needs to be defined.

According to the ECC 309, comparing the distribution of transmission power for aerial UEs and ground UEs: aerial UEs operate with lower transmission power (than ground UEs) because of a lower coupling loss with their serving BS (due to no obstacles within the path). The simulation parameters have not been given, but based on the description from ECC, regional OOBE requirements are at least applicable for PC2 and PC3.

Proposal 3: Regional OOBE requirements are at least applicable for PC2 and PC3.

# 3. Conclusions

In this contribution, some issue for RAN4 are discussed with the following proposals.

Proposal 1: We propose to define UAV definition in the NR Spec as:

Aerial UE (UAV): The UE with an aerial subscription.

Proposal 2：

##### Requirement for network signalling value "NS_[TBD1]" When "NS_[TBD1]" is indicated in the cell, the power of any UE emission for channels assigned within 1710-1785 MHz shall not exceed the levels specified in Table 6.5J.1-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.1-1: Additional requirements within 1710-1785 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 1675 ≤ f ≤ 1710; -40; 1 MHz. NOTE: This NS_[TBD1] is only applied for Aerial UE in CEPT region.. ##### Requirement for network signalling value "NS_[TBD2]" When "NS_[TBD2]" is indicated in the cell, the power of any UE emission for channels assigned within 2500-2570 MHz shall not exceed the levels specified in Table 6.5J.2-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.2-1: Additional requirements within 2500-2570 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 2690 ≤ f ≤ 2900; -50; 1 MHz. NOTE: This NS_[TBD2] is only applied for Aerial UE in CEPT region.. ##### Requirement for network signalling value "NS_[TBD3]" When "NS_[TBD3]" is indicated in the cell, the power of any UE emission for channels assigned within 2570-2620 MHz shall not exceed the levels specified in Table 6.5J.3-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. Table 6.5J.3-1: Additional requirements within 2570-2620 MHz Protected range (MHz); Spectrum emission limit (dBm); Measurement bandwidth. 2690 ≤ f ≤ 2900; -50; 1 MHz. NOTE: This NS_[TBD3] is only applied for Aerial UE in CEPT region..

Proposal 3: Regional OOBE requirements are at least applicable for PC2 and PC3.

# 4. References

[1] RP-230782, Revised WID on NR support for UAV, Nokia, Nokia Shanghai Bell

[2] R4-2306641, WF on NR Support for UAV, Nokia, Nokia Shanghai Bell