3GPP TSG-RAN WG4 Meeting # 107							                                   R4-2309446

Incheon, Korea, May 22 – May 26, 2023

Agenda Item:	8.14.3

Source: 	CMCC

Title: 	TP for TR 38.876 to introduce ATG BS Rx requirements

Document for:	Approval

- Introduction

After discussion in RAN4 meeting, some agreements have been achieved. This TP updates the RF requirements for ATG BS.

- Reference

[1] RP-221369, Revised WID on Air-to-ground network for NR, CMCC.

- TP to TR 38.876

<Start of TP>

7.2.3 Rx requirements

#### 7.2.3.1 Reference sensitivity level

For reference sensitivity level because it is expected that the BS technology will be the same and hence the noise factor the same. The existing requirements specified in clause 7.2 of TS 38.104[5] can be reused for ATG BS type1-C, 1-H, and clause 10.3 of TS 38.104[5] can be reused for ATG BS type1-O.

#### 7.2.3.2 Dynamic range

For dynamic range because it is based on assumptions on the maximum interference level around the BS. Detailed investigation has not been made, but since the main difference between an ATG BS and TN BS is the tilt, it is not expected to be greater and may be expected sometimes to be similar to a TN BS. The existing requirements specified in clause 7.3 of TS 38.104[5] can be reused for ATG BS type1-C, 1-H, and clause 10.4 of TS 38.104[5] can be reused for ATG BS type1-O.

7.2.3.3 ACS

Based on the simulation results, the ATG adjacent channel selectivity (ACS) reuse the legacy FR1 ACS (i.e. 46dBc) requirement. The existing requirements specified in clause 7.4 of TS 38.104[5] can be reused for ATG BS type1-C, 1-H, and clause 10.5 of TS 38.104[5] can be reused for ATG BS type1-O.

<End of TP>