3GPP TSG-RAN WG4 Meeting #107	R4-2309497

Incheon, Korea 22nd – 26th May 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FWA PC1 n41 NS_04 A-MPR simulation summary

Agenda Item: 			7.16.2

Document for:	Discussion

# 1	Introduction

In [1], we presented simulation results of FWA PC1 A-MPR for band n41 with NS_04. In this contribution, we summarize those results to help formulate the A-MPR specification.

# 2	NS_04 emission requirements

NS_04 defines both an additional spectrum emission mask and an additional spurious emission mask:

Table 6.5.2.3.2-3: n41 and n90 SEM with "NS_04"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 |  | 10 |  | 15 |  |  | 20 |  | 25 |  |  | 30 |  |  | 35 |  | 40 | 45 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 | -10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -10 |  |  |  |  | 1 MHz |
| ± 1 - 5 |  |  |  |  |  |  |  |  |  |  |  |  |  | -10 |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  |  |  |  |  |  |  |  |  |  |  |  |  | -13 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5.2.3.2-1 for CP-OFDM and 6.5.2.3.2-2 for DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.3.3.1-1: Additional requirements for "NS_04"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 MHz |  |
| 2495 ≤ f < 2496 | -13 | 1 % of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

# 3	Summary of simulation results

The 5 MHz channel bandwidth does not need A-MPR but all others do.

The additional SEM of NS_04 does not require A-MPR in PC1. Thus, A-MPR can be avoided if the channel is located at a sufficient distance from the additional spurious mask.

Figure 1 presents examples of typical A-MPR triangle plots for PC1 NS_04. In the simulation, the channel was located at the lower edge of n41, i.e., closest to the additional spurious emission mask. Only those allocations are plotted where required back-off exceeds the specified MPR.

Figure 1: Examples of PC1 A-MPR for NS_04, simulated at the lower edge of n41
(the plots show only datapoints that exceed the MPR)

The RB allocations that require A-MPR can be divided into the following regions:

Edge region: narrow allocations at lower channel edge, with spectral regrowth hitting the additional SEM; this region requires the largest A-MPR values.

Regrowth region: wide allocations at or near lower channel edge with spectral regrowth hitting the additional spurious mask

IMD3 region: narrow allocations near the lower channel edge with IMD3 (of allocation and its image) hitting the additional spurious mask

CIM3 region: narrow allocations near the upper channel edge with CIM3 hitting the additional spurious mask

The regrowth and IMD3 regions were already present in the NS_04 A-MPR definition for PC3, PC2, and PC1.5. For PC1, there are two new regions, the edge region and CIM3 region. The allocation regions are depicted in Figure 2, following the way how the regions have been specified for PC3 / NS_04.

Figure 2: Example of the allocation regions; the edge region is too small to be visible;
“MPR” indicates the region where no A-MPR is applicable.

The edge region applies only to the 100 MHz channel and only to the 1RB@0 allocation. This is caused by two reasons: (i) the small minimum guard band width of the 100 MHz channel and (ii) the fact that in the nearest NS_04 spurious mask segment, the measurement bandwidth is proportional to channel bandwidth, thus being stricter for wider bandwidths.

Figure 3 shows the parameter start which defines the position of the right-hand edge of the regrowth region in the NS_04 A-MPR definition for PC3 (shown as red in Figure 2). At first sight, the simulated curves look suspiciously inconsistent, with the values of start fluctuating up and down as function of channel bandwidth. However, the reason for the apparent inconsistency is the inconsistent minimum guard band widths (TS 38.101-1, Table 5.3.3-1). The rising trend of the curves can be explained by the fact that the measurement bandwidths are proportional to channel bandwidth.

To avoid A-MPR in the regrowth region, a frequency offset is needed between the lower edge of n41 and lower channel edge. The needed offset is shown in Figure 4.

Figure 3: Parameter start for the regrowth region (from TS 38.101-1, subclause 6.2.3.2)

Figure 4: Frequency offset needed to avoid A-MPR in the regrowth region

In the IMD3 region, the maximum allocation bandwidth that requires A-MPR is shown in Figure 5. The step in simulated values for OFDM between 50 and 60 MHz is caused by the change of minimum SCS from 15 kHz to 30 kHz. The highest allocation position for the IMD3 region is determined by whether the IMD3 emission reaches the strictest segment of the additional spurious mask. This is shown in Figure 6. Figure 7 shows the frequency offset needed for avoiding IMD3-based A-MPR.

Figure 5: Maximum allocation bandwidth for IMD3 region

Figure 6: Maximum RBstart for IMD3 region

Figure 7: Frequency offset needed to avoid A-MPR in the IMD3 region

Figure 8 presents the maximum allocation bandwidth in the CIM3 region. The reach of CIM3 is determined by the distance of the allocation from the upper channel edge, in a manner analogous to IMD3. This is shown in Figure 9. The frequency offset needed to avoid A-MPR in the CIM3 region is shown in Figure 10.

Figure 8: Maximum allocation bandwidth for CIM3 region

Figure 9: Maximum bandwidth of unallocated RBs above the allocation

Figure 10: Frequency offset needed to avoid A-MPR in the CIM3 region

Finally, Table 1 shows the maximum required A-MPR for each waveform and modulation in each allocation region. These A-MPR values should be applied unless a frequency offset from lower band edge prevents violating the additional spurious emission mask. N/A in the table means that no A-MPR is needed.

Table 1: A-MPR values for PC1

| Modulation/Waveform |  | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge region | IMD3 region | CIM3 region | Regrowth region |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 11 | ≤ 2 | N/A | ≤ 3.5 |
|  | QPSK | ≤ 11 | ≤ 2 | N/A | ≤ 4.0 |
|  | 16 QAM | ≤ 11 | ≤ 2.5 | ≤ 2.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 11 | ≤ 3 | ≤ 3.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 11 | N/A | N/A | N/A |
| CP-OFDM | QPSK | ≤ 11 | ≤ 2.5 | ≤ 10.5 | ≤ 6.0 |
|  | 16 QAM | ≤ 11 | ≤ 3 | ≤ 10.5 | ≤ 6.0 |
|  | 64 QAM | ≤ 11 | N/A | ≤ 10.5 | ≤ 6.0 |
|  | 256 QAM | ≤ 11 | N/A | ≤ 11 | N/A |

# 4	Conclusions

We presented a summary of grid simulation results for FWA PC1 NS_04 in band n41 and identified four allocation regions which need different back-off values. The A-MPR definition can be derived from this summary.

# 5	References

[1] R4-2305678, FWA PC1 n41 NS_04 A-MPR simulation results, Nokia, RAN4#106bis-e