3GPP TSG-RAN WG4 #107	R4-2309511

May 22nd ‒ 26th, 2023

Incheon, KR

Agenda item:	9.6.3

Source: 	Qualcomm Incorporated

Title: 	Specification of wider channel bandwidths for FR1 NTN UE

Document for:	Discussion

# Introduction

NR NTN supports channel bandwidths of up to 20 MHz for Bands n255 and n256 in FR1.  Furthermore, a new band has been proposed in the L and S band frequency range tentatively denoted as Band [n254] with support for up to 15 MHz channel bandwidth.  This contribution considers the impact of the larger channel bandwidths to the UE receiver requirements.

# Discussion

It is well understood that the link budget between the satellite and the UE on the ground is extremely challenging due to the large path loss between the two.  While the uplink receives the most attention due to the limited power available at the UE, the downlink also deserves consideration.  Uplink and downlink from the UE’s perspective can be scheduled for a single RB or small number of RB’s to concentrate energy in a narrow bandwidth and increase the power spectral density; however, the downlink must be transmitted by the SAN in its entirety to serve all UE’s within the cell.  Therefore, the wider bandwidths in the downlink require the SAN to be able to transmit at higher power.  Since power availability is limited at the SAN (power is derived from solar panels of finite dimension), the downlink bandwidth cannot increase arbitrarily while maintaining the necessary downlink PSD to reach the UE.  Thus, wider bandwidths require both high transmit power and very high antenna gain.  This implies a very large satellite, especially for GEO where the free space pass loss is 190+ dB not to mention other channel impairments such as rain fade, scattering, etc.

A link budget is provided in Table 6.1.3.3-1 of TR 38.821 for a number of different cases.  For convenience, the table is copied below but only the cases for FR1 S-band are included.

Table 6.1.3.3-1: Link budgets results

| Case | Transmission mode | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC4 | DL | 2.0 | 103.8 | -31.6 | 30.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC5 | DL | 2.0 | 99.0 | -31.6 | 10.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC9 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC10 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC14 | DL | 2.0 | 84.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC15 | DL | 2.0 | 80.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC19 | DL | 2.0 | 98.3 | -31.6 | 30.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC20 | DL | 2.0 | 93.5 | -31.6 | 10.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC24 | DL | 2.0 | 72.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC25 | DL | 2.0 | 68.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC29 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| SC30 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |

For these cases, the frequency range is FR1 at 2 GHz.  All of GEO, LEO-600, and LEO-1200 orbits are represented in the TR.  The assumptions for the UE parameters are found in Table 6.1.1.1-3 of TR 38.821 where the UE antenna gain is 0 dBi for omni-directional coverage, the noise figure is 7 dB, and the antenna temperature is 290 K leading to a G/T ratio of -31.6 K-1.  Moreover, the link budget assumes antenna elevation of 90 degrees to the GEO satellite and 45 degrees to the LEO.  With these assumptions, the link budget indicates 0 dB CNR at the UE demodulator for cases SC4 and SC5 with a GEO satellite.  With a dual receiver in the UE, the combined CNR could be improved to 3 dB.  With 3 dB CNR, the modulation and code rate will have to be very low for robustness which means only low data rates are achievable.  With LEO-600 in SC9 and SC10, the CNR with dual receiver equipped UE is 9.6 dB which can support more moderate data rates.  The assumptions, however, are somewhat idealistic.  For example, the noise figure assumed by RAN4 in specifying requirements is 9 dB rather than 7 dB assumed in the link budget calculation.  Thus, the resulting G/T will be lower for a 9 dB noise figure.  The practical antenna gain of an actual handheld device in FR1 is expected to be lower than 0 dBi and the elevation angle to the satellite can be considerably shallower than assumed above.  All of the above will lead to a lower CNR available at the UE in practice than suggested by the link budget.  Moreover, the link budget above also assumes the SAN is able to maintain a constant EIRP density as shown in Tables 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821, irrespective of bandwidth.  In other words, the required output power at the SAN scales up with increasing bandwidth.  For example, case SC4 shows a required Tx EIRP of 103.8 dBm for a GEO satellite serving 30 MHz of bandwidth at 2 GHz.  To support 15 MHz channel bandwidth would require a Tx EIRP of 100.8 dBm.  The PA must be able to deliver even higher power since power backoff is needed for ensure the linearity needed for multi-carrier OFDM waveforms while meeting adjacent and spurious emission requirements.  It is not clear what SAN would be able to support this – for example, may be limited to SAN with certain size and payload and power capacity.  The TS 38.108 does not specify a value for the maximum TRP at the SAN, but it is left to manufacturer declaration.  EIRP is also not specified.  More information from SAN manufacturers and/or satellite operators is welcomed on the expected Tx EIRP.

There is nothing fundamentally invalid about specifying and allowing wider bandwidths for NR NTN so long as power is available.  However, it should be understood that the wider bandwidths may not be achievable or may yield poor and unreliable performance given real-world constraints.  There is a strong dependency on the Tx EIRP available at the SAN since the link budget is very challenging especially for GEO satellites.  Very large GEO satellites with high antenna gain may be able to support wider bandwidths.  LEO constellations with lower path loss are more likely to be able to support wider bandwidths.  Similarly, UE’s with larger form factors (i.e., not handheld) able to accommodate higher gain antennas and better radio performance are also more likely to be able to support wider bandwidths.  Several options are listed below for discussion.

Option 1:  15 MHz and 20 MHz bandwidths are mandatory for UE’s intended for operation in a LEO constellation, but optional for UE’s intended for operation in a GEO constellation.  This implies UE type distinction or capability signaling.

Option 2:  15 MHz and 20 MHz bandwidths are mandatory for larger UE form factors (i.e., CPE), but optional for handheld UE form factors.  This implies UE type distinction or capability signaling.

Option 3:  15 MHz and 20 MHz bandwidths are mandatory for the UE, but a cautionary note is included in the specifications to indicate that wider bandwidths may not deliver higher data rates or reliable service with handheld device form factors.

Option 4:  15 MHz and 20 MHz bandwidths are specified as usual.  RAN4 will not provide any further guidance in its specifications recognizing that system performance and network planning are the responsibility of the satellite operator.

# Conclusion

A link budget for the downlink from TR 38.821 has been reviewed to understand the expected performance for wider channel bandwidths.  It was found that especially for GEO satellites, the ability to deliver enough power to reach the UE’s on the ground is very challenging.  The Tx EIRP is required to be at least 100.8 dBm to support 15 MHz and even this is with somewhat idealistic assumptions and yields only 0 dB CNR per Rx chain.  Thus, only the lowest MCS’s are expected to be available limiting the data rate to the UE.

Since the 38.108 specification does not provide a requirement on output power, feedback from SAN manufacturers and/or satellite network operators is welcomed to shed more light on the expected deployment.  Due to the uncertainty in the efficacy of larger bandwidths for NTN, several options have been presented for further discussion

Option 1:  15 MHz and 20 MHz bandwidths are mandatory for UE’s intended for operation in a LEO constellation, but optional for UE’s intended for operation in a GEO constellation.  This implies UE type distinction or capability signaling.

Option 2:  15 MHz and 20 MHz bandwidths are mandatory for larger UE form factors (i.e., CPE), but optional for handheld UE form factors.  This implies UE type distinction or capability signaling.

Option 3:  15 MHz and 20 MHz bandwidths are mandatory for the UE, but a cautionary note is included in the specifications to indicate that wider bandwidths may not deliver higher data rates or reliable service with handheld device form factors.

Option 4:  15 MHz and 20 MHz bandwidths are specified as usual.  RAN4 will not provide any further guidance in its specifications recognizing that system performance and network planning are the responsibility of the satellite operator.

# Reference