3GPP TSG-RAN WG4 Meeting #107 	R4-2309572

Incheon, Korea meeting, May 22nd – 26th, 2023

Agenda Item: 8.32.3

Source: MediaTek inc.

Title: Discussion on further NR RedCap UE complexity reduction

Document for: Discussion and decision

# Introduction

RAN4 discussed the topics for rel-18 RedCap RRM requirements and the issues are captured in the way forward (WF) [1]. This contribution paper provides further discussion on the issues in this WF.

# Discussion

The discussion is mainly divided into three main items, which are (i) eDRX for INACTIVE mode, and (ii) RRM impact due to baseband reduction.

## Discussion on eDRX for INACTIVE mode

The open issues are captured below:

### Defining of requirements for eDRX cycle > 10.24 sec in FR1 and FR2 Extended eDRX range requirements in RRC INACTIVE - INACTIVE state requirements for eDRX cycle > 10.24 sec - The sample number needed to perform the serving cell and intra/inter-frequency neighbor cell detection/evaluation/measurements for corresponding IDLE mode eDRX cycles can be reused for INACTIVE mode eDRX cycles. When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements - Wait for the RAN2 decisions on the T value and PTW length design before defining serving cell measurements requirements When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements The detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of: - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec - FFS if eDRX_Inactive 20.48sec When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements - Measurements/evaluation period shall be constrained to be within a single PTW irrespective of whether IDLE or INACTIVE eDRX PTW is being used. - Note: Wait for the RAN2 decisions on the T value and PTW. When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements The number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) may belong to more than one PTW. When to measure when IDLE and INACTIVE eDRX PTW do not coincide RAN4 to wait for RAN2 progress to discuss following: - Whether to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding. Transition requirements RAN4 to discuss the need for transition requirements (UE behavior and corresponding requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec)) after the INATIVE eDRX with PTW requirements are finalized. CG-SDT requirements with PTW - FFS whether to define requirements for CG-SDT for Rel-18 eRedCap UE with PTW CG-SDT requirements with PTW: RSRP2 time range Following option is to be discussed after RAN4 has identified whether new requirements are to be specified for Rel-18 eRedCap with PTW: - RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2.

When configured with both IDLE and INACTIVE eDRX configurations for neighbouring cell measurements

To our understanding, measurement requirements for inactivate state for intra/inter frequency Redcap UE are based on inactive DRX or inactive eDRX when inactive eDRX is configured. In addition, the detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of both (i) INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec and (ii) eDRX_Inactive  20.48sec.

- The detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of both (i) INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec and (ii) eDRX_Inactive  20.48sec.

CG-SDT requirements with PTW and CG-SDT requirements with PTW: RSRP2 time range

The eDRX cycle >10.24 can go up to approximately 2.9 hours, while CG-SDT may require much faster transmission/reception. It is not clear to us how the UE can achieve to measure for RSRP1 and RSRP2 in the same PTW. Besides, if RSRP1 and RSRP2 are measured with time difference up to 2.9 hours then it is unlikely they will be matching the same configuration. Hence, more discussion is needed to understand the UE procedure to perform RSRP2 before defining the requirements.

- More discussion is needed in RAN4 on how to achieve RSRP1 and RSRP2 within long eDRX cycle with PTW.

## Discussion on baseband reduction

The open issues are captured below:

### RRM impact due to baseband reduction Impact on CGI reading requirements - FFS whether to define CGI reading requirements for Rel-18 eRedCap UEs - If CGI reading requirements for Rel-18 eRedCap UEs are introduced - If SIB1 is scheduled within 5MHz BW - Reuse existing requirements for SIB1 reading for the case - FFS for the requirements for the case of SIB1 scheduled with BW > 5MHz Impact on paging requirements No impact on paging reception requirements due to baseband BW reduction. Impact on THARQ There is not any impact on THARQ due to baseband BW reduction.

RRM impact of baseband BW reduction in FR1

For SIB1 reading: The RRM requirements define the number of samples needed to read SIB1, which is based on threshold SINR, however, the RRM requirements don’t define how much time is needed to process the SIB1 reading. For RedCap rel-18, the reception bandwidth can be kept as original bandwidth and hence there is no impact on the reception of SIB1 samples, however, the baseband processing bandwidth is smaller than normal NR UE, thus perhaps the processing time for SIB1 may take longer compared to existing NR UE. However, this processing time duration is not captured in RRM requirements in TS 38.133. Thus, RAN4 RRM doesn’t need to define further requirements.

Observation: The processing time duration for SIB1 is not captured in RRM requirements in TS 38.133.

- RAN4 shall reuse existing requirements for SIB1 (CGI) reading.

# Summary

In this contribution, we have the following proposals:

Proposal 1: The detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of both (i) INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec and (ii) eDRX_Inactive  20.48sec.

Proposal 2: More discussion is needed in RAN4 on how to achieve RSRP1 and RSRP2 within long eDRX cycle with PTW.

Proposal 3: RAN4 shall reuse existing requirements for SIB1 (CGI) reading.

# References

- R4-2306365, “WF on R18 RedCap RRM requirements”, Ericsson, e-meeting, March 2023.