3GPP TSG RAN WG4 Meeting #107	R4-2309643

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.29.5

Source: 	Ericsson

Title:	Further analysis of RRM requirements for network-controlled repeater

Document for:	Discussion

# Introduction

In the last RAN4 meeting a WF on NCR RRM was approved containing several agreements [1]. Most of the RRM issues have been settled except few remaining issues. In this paper, we further analyze these remaining RRM issues related to NCR-MT.

# Analysis of RRM requirements

## Adaptive beamforming for NCR access link

Following was agreed regarding the adaptive beamforming for NCR access link in the last meeting [1]:

- Agreements

  - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

It was proposed to define switching delay for switching the access link beam. This refers to the time required by the NCR-Fwd to switch the access link beam from the moment it has received the beam switching command (e.g. DCI or PDSCH). The access link beam is used to serve the UEs on the access link. The beam switching does not require the NCR-Fwd to acquire SSB or any other signal on the backhaul or control link.

In RAN1#112, RAN1 made the following agreement:

Agreement RAN1#112

For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and:

- Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor].

Note: This k is different from the parameter used to define the Slot offset for the time resource.

Based on the above agreements all the indication for time resources for beams are relative to slot n+k. Where, “k” as stated in the agreement is declared by the vendor. Therefore the access link beam switching delay depends on the NCR implementation and does not need to be specified.

## RLM requirements

Following was agreed related to RLM/BFD/BFR in the last meeting [1]:

- Agreements

  - For LA NCR-MT

    - Define RLM/BFD/BFR requirements

  - For WA NCR-MT

    - Do not define BFD/BFR requirements

    - FFS for RLM requirements.

According to the above agreement, there is no consensus whether the RLM requirements should be defined for WA NCR class.

The WA NCR deployment is based on extensive network planning. It is a fixed node and is deployed to cover wider area. Therefore, the radio conditions between the gNB and the WA NCR-MT will remain consistent over time and practically the beams will not change over time. Therefore, the radio link failure for the WA NCR-MT is highly unlikely or it may occur very rarely. A possible reason (if any) for the radio link failure could be related to the limitation in the network planning. However, such problem should be resolved by enhancing the network planning rather than constraining the NCR-MT to perform regular monitoring of the radio link.

Due to the above reasons, it is not necessary for the NCR-MT to regularly measure on the SSB or CSI-RS to monitor the radio link quality for the purpose of RLM. Such periodic measurement/sampling increases the NCR-MT implementation complexity without providing any benefit since the radio link quality will be within the acceptable quality. The RLM is also defined as an optional feature for the NCR-MT. In RAN1#111, RAN1 made the following agreement:

Agreement RAN1#111

As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported

FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

We therefore do not support defining any requirements for the RLM. At least no requirements should be defined for the WA NCR since the beams are configured according to the well-planned deployment of the NCR node in a wide area environment.

# Summary

The following are the observations and the proposals related to RRM requirements for NCR-MT:

Adaptive beamforming for NCR access link:

- Observation #1: The switching of the access beam link does not require the NCR-Fwd to acquire SSB or any other signal on the backhaul or control link.

- Observation #2: The switching delay for the access beam link at the NCR-Fwd depends on the NCR implementation and declared by the manufacturer.

- Proposal #1: Do not define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

RLM for WA NCR:

- Observation #3: The RLM procedure is optional for the NCR-MT on the C-link.

- Observation #4: Radio link and beams on the C-link do not practically change over time. Therefore, radio link failure will happen very rarely on the C-link.

- Observation #5: RLM will require the NCR-MT to periodically measure/sample the NCR-MT increasing the implementation complexity without providing any benefit.

- Proposal #2: Do not specify requirements for RLM for WA NCR class on the C-link.

# References

- R4-2306360, WF on NCR-MT RRM requirements, ZTE Corporation

- R4-2306253, Topic summary for [106-bis-e][222] NR_netcon_repeater, ZTE Corporation