3GPP TSG-RAN WG4 Meeting # 107	R4-2309666

Incheon, KR, May 22 – May 26, 2023

Source: 	Qualcomm Incorporated

Title: 	ATG UE Demod

Agenda item:	8.14.5.2

Document for:	Discussion

- Introduction

In this paper, we share our considerations on the items on UE Demodulation requirements with ATG, based on the WF [1].

# ATG UE Demod requirements applicability

To decide whether or not newly introduced demodulation requirements for ATG apply to UEs under test, it is our view that RAN4 should consider the outcome of the discussions in RF related to ATG capabilities, and whether it’s possible to reuse any capability to identify ATG devices

Proposal 1: For the applicability of the ATG UE Demod requirements, follow ongoing discussion in RF on related ATG UE capabilities;

Regarding the test scope, the WF in [1] contains some references to NTN UEs and related channels. In our understanding and according to the WID, ATG UEs are assumed to be connected only to TN (ground-based BS). To that end, we don’t see a motivation to introduce any applicability rule between the two features

Observation 1: According to the WID, ATG UEs only connect to ground-based BS;

Proposal 2: Based on Observation 1, RAN4 should not introduce applicability rules for between ATG and NTN UEs;

# Test Scope for Legacy Requirements

In this section we are proposing how to choose the demodulation test scope among already existing demodulation tests for PDSCH, PDCCH and CSF for ATG devices.

Proposal 3:On the applicability of legacy PDSCH and PDCCH requirements, RAN4 should select only a subset of legacy requirements applicable to ATG UEs;

Proposal 4: For legacy PDSCH Demod requirements applicable to ATG UEs, RAN4 to choose 1 test case per Modulation Order among the existing requirements for PDSCH Mapping Type A only, for FDD and TDD;

Proposal 5: For legacy PDSCH Demod requirements applicable to ATG UEs, RAN4 should consider Single Carrier requirements only;

Proposal 6: For legacy PDCCH Demod requirements applicable to ATG UEs, RAN4 to choose 1 test case per TDD and 1 test case per FDD among the existing requirements;

Proposal 7: Do not test legacy CSI reporting requirements for ATG UEs;

# Other Items

## TDD Pattern

In the past meetings it was discussed whether to introduce requirements with TDD patterns specific for ATG devices. If new patterns that can be configured according to spec are proposed, RAN4 should consider them but it should be specified in the spec that those TDD patterns apply to ATG only

Proposal 8: RAN4 to further consider using new TDD pattern specific for ATG. If a new TDD Pattern is introduced for ATG UEs, support adding a note that specifies that the pattern is for ATG scenario used only.

## Channel Model

In the previous meeting it has been agreed to consider a Single path channel with no fading and constant doppler shift. In our view adding a constant doppler shift to model residual error after UE compensation is not going to adding anything to the coverage, because we should expect the constant shift to be corrected by the UE as soon as frequency tracking information is available, and is only going to confuse the test setup.

To that end, we propose to reuse the same approach that was followed for UE frequency pre-compensation in NTN demodulation requirement and assume perfect correction, without a constant doppler shift.

Proposal 9: RAN4 to assume error-free DL frequency shift pre-compensation for UE demodulation, and no Doppler shift assumed in the test;

## CBW

For CBW, we have not seen a motivation so far not to reuse the same CBW as in legacy requirements.

Proposal 10: RAN4 to use 10 MHz CBW for FDD, 40MHz CBW for TDD to keep alignment with legacy requirements;

- Conclusions

Proposal 1: For the applicability of the ATG UE Demod requirements, follow ongoing discussion in RF on related ATG UE capabilities;

Observation 1: According to the WID, ATG UEs only connect to ground-based BS;

Proposal 2: Based on Observation 1, RAN4 should not introduce applicability rules for between ATG and NTN UEs;

Proposal 3:On the applicability of legacy PDSCH and PDCCH requirements, RAN4 should select only a

subset of legacy requirements applicable to ATG UEs;

Proposal 4: For legacy PDSCH Demod requirements applicable to ATG UEs, RAN4 to choose 1 test case per Modulation Order among the existing requirements for PDSCH Mapping Type A only, for FDD and TDD;

Proposal 5: For legacy PDSCH Demod requirements applicable to ATG UEs, RAN4 should consider Single Carrier requirements only;

Proposal 6: For legacy PDCCH Demod requirements applicable to ATG UEs, RAN4 to choose 1 test case per TDD and 1 test case per FDD among the existing requirements;

Proposal 7: Do not test legacy CSI reporting requirements for ATG UEs;

Proposal 8: RAN4 to further consider using new TDD pattern specific for ATG. If a new TDD Pattern is introduced for ATG UEs, support adding a note that specifies that the pattern is for ATG scenario used only.

Proposal 9: RAN4 to assume error-free DL frequency shift pre-compensation for UE demodulation, and no Doppler shift assumed in the test;

Proposal 10: RAN4 to use 10 MHz CBW for FDD, 40MHz CBW for TDD to keep alignment with legacy requirements;

- References

[1] R4-2305899, 3GPP TSG-RAN WG4 Meeting #106bis-e, Online Meeting, 17th April- 26th April, 2023