3GPP TSG-RAN WG4 Meeting #107	R4-2309673

Incheon, Korea, May 22 - 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on general issues for Case 1 and Case 2 requirements

Agenda item:	8.10.2.1

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on general open issues as discussed at RAN4 #106bis-e [2] related to Case 1 and Case 2 requirements.

Discussion

Agreements and open issues

Following agreements and open issues are listed in [2].

- Sub-topic 2-1: Scope and general issues - Issue 2-1-1: Which Type of MG is considered together with Pre-MG/NCSG in the WI? < Wayforward >: - Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI. - Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR. - Issue 2-1-2: If Type-1 MG is considered in this WI, which RRM requirements shall be applied? < Agreement >: - If Type-1 MG is considered in this WI, similar to that of Rel-17 Concurrent MG WI, “no requirements” could be applied for the scenario when Type-1 MG is colliding with Pre-MG/ NCSG in the same FR.

Proposal

Based on agreed issue 2-1-2, no collision requirements will be specified for the gap combinations Type-1 MG + Pre-MG or Type-1 MG+NCSG, provided these gap combinations are considered in the specification. This is aligned to Type-1 MG and Type-2 MG gap combination specified in Rel-17 for concurrent gaps.

TS 38.133 refers to the latter combination in section 9.1.8.3:

The requirements of concurrent measurement gaps in section 9 shall not apply when a gap without assigned priority is configured simultaneously with any other gap(s) that affect serving carriers in the same FR and the measurement gaps are colliding with each other.

In case no requirements are specified in the collision case, the UE behavior is not specified and hence is implementation specific. Thus, the network needs to account for preventing collisions between gaps by configuring non-overlapping measurement gap patterns. In our view, this effort is not significant compared to the flexibility to use Type-1 MG (legacy MG) together with Pre-MG or NCSG. For instance, this allows to configure for Case 1 Type-1 MG for inter-frequency, inter-RAT and positioning measurements and Pre-MG for intra-frequency measurements. Or for Case 2 intra-frequency and inter-frequency measurements for NCSG and Type-1 MG for inter-RAT and positioning measurements.

Thus, the following proposal is made.

RAN4 to adopt option 2a in issue 2-1-1, i.e. to consider Type-1 MG without specifying collision requirements for the gap combinations Type-1 MG + Pre-MG and Type-1 MG + NSCG.

Conclusion

This paper has presented Nokia’s views on general open issues as discussed at RAN4 #106bis-e [2] related to Case 1 and Case 2 requirements. As part of this discussion, we make the following proposal:

- RAN4 to adopt option 2a in issue 2-1-1, i.e. to consider Type-1 MG without specifying collision requirements for the gap combinations Type-1 MG + Pre-MG and Type-1 MG + NSCG.

- References

- RP-230755, Revised WID: Further enhancements on NR and MR-DC measurement gaps and measurements without gaps, Intel, MediaTek

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek