3GPP TSG-RAN WG4 Meeting#107	R4-2309684

Icheon, Korea 22nd May – 26th May, 2023

Source: 	Qualcomm Incorporated

Title: 	Views on General Aspects for FR2 Multi-Rx

Agenda item:	8.8.4.1

Document for:	Discussion

# 1. Introduction

After the conclusion of RAN4#106bis-e, there were a few open issues concerning the general aspects [1]. In this paper, we share our views on these remaining open issues.

2. General Aspects for Demodulation Requirements

2.1 New correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context

During the last meeting, the formulation of a new correlation matrix for FR2 multi-Rx was discussed. Below we present our view on this issue.

TRP1

TRP2

γ

γ

h11 h12h21h22

h33h34h43h44

h31h32h41h42

h13h14h23h24

TRP1

TRP2

γ

γ

h11 h12h21h22

h33h34h43h44

h31h32h41h42

h13h14h23h24

Fig. 1 FR2 multi-Rx system containing 2 TRPs and 1 UE

Let us consider a FR2 multi-Rx system, where there are two TRPs and one UE with 2 modules containing two receivers, Rx1 and Rx2, intended for each TRP, as shown in Fig. 1. Let us assume that each TRP has 2 Tx antennas and each module at the UE side has 2 Rx antennas. Therefore, the overall channel matrix is given by,

H=h11 h12h21h22h13h14h23h24h31h32h41h42h33h34h43h44                                          (1)

where the intended channels from TRP1 and TRP2 are given by blue and green matrices, respectively, and the cross-talk terms are given by matrices in red. The transmitted signal and precoder from TRPs are given by x2×1TRPi and P2×2TRPi, respectively, where iϵ1,2. Also, the power ratio between the intended channel and cross-talk is denoted by γ, as shown in Fig. 1.

After re-arranging the channel matrix H in eqn. (1), we arrive at

H=h11 h31h21h41h13h33h23h43h12h32h22h42h14h34h24h44				(2)

Let us consider h4×1TRP1,1  denotes the stacked channel vector between TRP1 to Rx antennas placed within the module 1.  Similarly, the stacked channel vector between the TRP2 and Rx antennas  (placed within module 2) is given by h4×1TRP2,2 . The cross-talk terms, i.e., between TRP1 to Rx antennas within module 2 and TRP2 to Rx antennas within module 1 are denoted by  h4×1TRP1,2 and h4×1TRP2,1 , respectively.

Therefore, the received signal vector y4×1 can be written as

y4×1=h4×1TRP1,1 h4×1TRP1,2 h4×1TRP2,1 h4×1TRP2,2  P2×2TRP102×202×2P2×2TRP2x2×1TRP1x2×1TRP2+n4×1

=H4×4P4×4x4×1+n4×1

where n4×1 is i.i.d (independent and identically distributed) Gaussian noise vector. In our view, to simplify the formulation of the correlation matrix, the potential correlation between the modules, i.e., the inter-module correlation can be ignored. With this simplification, we can treat the correlation matrices for the desired and cross-talk terms separately, where the correlation matrix corresponding to each column of matrix H in eqn. (2), RMIMOi,j, i,jϵ1,2, can be obtained as in 38.101-4 for a given number of Tx and Rx antennas with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient).

Considering the desired channel to cross-talk power ratio of γ, the corresponding normalized MIMO channel can be realized as follows

h16×1=vec(H4x4)=11+γ2diag(1γγ1)⊗RMIMOi,ja                                       (3)

where each element of a is i.i.d. Rayleigh faded.

Observation 1: Inter-module correlation between the UE RX antenna modules can be ignored.

Proposal 1: Consider the correlation model assuming cross-talk between the TRPs and UE Rx antenna modules as presented in eqn. (3).

h16×1=vec(H4x4)=11+γ2diag(1γγ1)⊗RMIMOi,ja

Proposal 2: Assume the γ value, i.e., the cross-talk terms to be at least 10dB weaker than the direct channel links (between the TRPs and UE antenna modules).

2.2 Test Method for FR2 Multi-Rx

RAN4 is already discussing the OTA testing enhancements for FR2 [2]. According to this discussion, the test method would be a virtual cable setup, which will include an artificial noise at the TE. Therefore, the SNR at the UE side can be obtained via controlling the artificial noise at the TE side. This allows testing of the demodulation requirements, without having to account for the RF impairments which may happen during a real OTA test.

Observation 2: Real OTA testing may include impairment due to RF, hence demod requirements may not be truly tested in the presence of such impairments.

Proposal 3: RAN4 to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements as discussed in NR FR2 OTA enhancements.

2.3 Receiver Assumptions

There has been extensive discussion during the last meeting as to the receiver assumption for FR2 multi-Rx demodulation requirements. We note there are pros and cons with each option. While 4x4 joint processing may offer better performance, the implementation of this technique may prove to be more complex. On the other hand, 2x2 independent processing may offer lower implementation complexity with a performance trade-off.

Proposal 4: Companies are encouraged to bring results evaluating both independent and joint processing in August meeting.

3. Conclusions

In this paper, we share our views on the remaining issues in regard to general aspects for defining requirements for FR2 multi-Rx DL tests. The following have been proposed.

Observation 1: Inter-module correlation between the UE RX antenna modules can be ignored.

Proposal 1: Consider the correlation model assuming cross-talk between the TRPs and UE Rx antenna modules as presented in eqn. (3).

h16×1=vec(H4x4)=11+γ2diag(1γγ1)⊗RMIMOi,ja                                       Proposal 2: Assume the γ value, i.e., the cross-talk terms to be at least 10dB weaker than the direct channel links (between the TRPs and UE antenna modules).

Observation 2: Real OTA testing may include impairment due to RF, hence demod requirements may not be truly tested in the presence of such impairments.

Proposal 3: RAN4 to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements as discussed in NR FR2 OTA enhancements.

Proposal 4: Companies are encouraged to bring results evaluating both independent and joint processing in August meeting.

4. References

[1] R4-2306000, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #106bis-e, Online, Apr 17 – 26, 2023.

[2] R4-2214357, “WF on NR FR2 OTA testing enhancements,” 3GPP TSG RAN WG4 Meeting #106bis-e, Online, Apr 17 – 26, 2023.