3GPP TSG-RAN WG4 Meeting #107	R4-2309740

Incheon, South Korea, 22 – 26 May 2023

Source:	Skyworks Solutions, Inc.

Title:	LTE Cat-M2 1.4MHz and 3MHz REFSENS in band 71

Agenda item:			5.2.10.2 UE RF requirements

Document for:	Approval

# 1. Introduction

At meeting #106bis-e, it was observed that reference sensitivity (REFSENS) requirements have been tentatively agreed for band 71 at 1.4MHz and 3MHz channel bandwidth (CBW) [1]. In this paper we raise questions on understanding how such requirements could be defined for CBW that are not supported by band B71.

# 2. Discussion

Table 1 and Table 2 below extract the band B71 REFSENS requirements from Table 7.3.1E-8 and Table 7.3.1E-9 for LTE Category M2 QPSK FDD/TDD UE and HD-FDD UE respectively. The requirements for 1.4MHz and 3MHz CBW are highlighted in yellow.

Table 1: LTE band B71 Cat-M2 QSPK REFSENS for FDD /TDD UE

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 71 | [-99.4] | [-95.4] | [-93.4] | [-93.4] | [-93.4] | [-93.4] | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 5: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |

Table 2: LTE band B71 Cat-M2 QSPK REFSENS for HD-FDD UE

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 71 | -100.2 | -96.2 | -94.2 | -94.2 | -94.2 | -94.2 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

In clause 5.5E, the operating frequency bands for LTE category 0, M1, M2 and cat-1bis are the same band definitions than for all other LTE UEs and are specified in table 5.5-1 as highlighted in yellow in the text excerpt of Figure 1.

Figure 1: LTE Cat-0, M1, M2, 1bis operating bands

The supported CBW for band 71 are reproduced in Table 3, with yellow highlights indicating that neither 1.4MHz nor 3MHz CBW are supported for band B71.

Table 3: LTE band B71 Cat-M2 QSPK REFSENS for HD-FDD UE

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 71 |  |  | Yes | Yes1 | Yes1 | Yes1,6 |
| NOTE 1: 1 refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 2: 2 For the 20 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz NOTE 3: 3 refers to the bandwidth for which the uplink transmission bandwidth can be restricted by the network for some channel assignments in FDD/TDD co-existence scenarios in order to meet unwanted emissions requirements (Clause 6.6.3.2). NOTE 4: 4 For the 20 MHz bandwidth, the minimum requirements are restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: 5 For the 15 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 705.5 MHz or 710.5-720.5 MHz NOTE 6: 6 For the 20MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 673-678 MHz or 683-688MHz. NOTE 7: 7 This band is for standalone NB-IoT operation only. None of E-UTRA standard channel bandwidths is applicable. |  |  |  |  |  |  |

Observation 1: For category M2 FDD/TDD UE and category M2 HD-FDD UE, QPSK reference sensitivity levels are either tentatively agreed or agreed for band 71 1.4 MHz and 3 MHz CBW. However, these channel bandwidths are not specified for band 71.

Since 1.4MHz and 3MHz CBW are not specified, it is questionable how RAN5 conformance may be performed since the UE is likely to reject initial cell attach to the band 71 cell. These issues may be critical.

Observation 2: It is not possible to perform Cat-M2 UE device certification for 1.4MHz and 3MHz CBW in band 71.

Due to lack of time, we are not able to retrace the history of these requirements. If the intention was to enable Cat-M2 deployments in CBW smaller than the minimum supported 5MHz CBW, then we may consider at least two options to solve the observed issues:

option 1: introduce support for new 1.4MHz and 3MHz CBW in frequency band 71,

option 2: Capture REFSENS requirements captured in a separate table to clarify the intended deployments.

Proposal: Clarifications from proponents are encouraged to resolve the issues raised in observation 1 and 2. If the intention was to enable Cat-M2 deployments in CBW smaller than the minimum supported 5MHz CBW, then we may consider at least two options to resolve the observed issues:

option 1: Introduce support for new 1.4MHz and 3MHz CBW in frequency band 71,

option 2: Capture REFSENS requirements in a separate table and clarify cell configuration parameters for conformance.

# 3. Conclusion

In this contribution, we observe that reference sensitivity has been tentatively agreed or agreed for LTE Cat-M2 operation in band 71 at 1.4MHz and 3MHz CBW.

Observation 1: For category M2 FDD/TDD UE and category M2 HD-FDD UE, QPSK reference sensitivity levels are either tentatively agreed or agreed for band 71 1.4 MHz and 3 MHz CBW. However, these channel bandwidths are not specified for band 71.

Considering these CBW are not specified for band 71 commercial networks, these requirements may not be verified through conformance testing.

Observation 2: It is not possible to perform Cat-M2 UE device certification for 1.4MHz and 3MHz CBW in band 71.

To address these observations, we make the following proposal.

Proposal: Clarifications from proponents are encouraged to resolve the issues raised in observation 1 and 2. If the intention was to enable Cat-M2 deployments in CBW smaller than the minimum supported 5MHz CBW, then we may consider at least two options to resolve the observed issues:

option 1: Introduce support for new 1.4MHz and 3MHz CBW in frequency band 71,

option 2: Capture REFSENS requirements in a separate table and clarify cell configuration parameters for conformance.

# References

[1]	R4-2305751 1.4MHz and 3MHz REFSENS for 600MHz US LTE band, 3GPP TSG RAN4 Meeting #106bis-e, Electronic meeting, Skyworks Solutions, inc.