3GPP RAN4 Meeting #107	R4-2309792

# Incheon, Korea, May 22nd – 26th, 2023

Title: 	TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements

Agenda Item:	8.20.2.2.4

Source: 	Qualcomm Incorporated

Document for:	Approval

# 1      Introduction

In this document we propose text to capture some of the agreements that have been made in previous meetings and discussions, and merge some of the TP proposals from the referenced documents submitted for this meeting

In the last meeting, the skeleton of RAN4 part for TR 38.858 was agreed as follows:

| No. | Section for TR 38.858 |
| --- | --- |
| 1 | 10.1 Background for analysis |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects |
| 5 | 10.5 Feasibility of FR2 BS aspects |
| 6 | 10.6. FR1 Feasibility of UE aspects |
| 7 | 10.7 FR2 Feasibility of UE aspects |
| 8 | 10.8 Summary |
| 9 | 11.1 Impact on BS RF requirements |
| 10 | 11.2 Impact on UE RF requirements |
| 11 | 12 Adjacent channel co-existence evaluation results |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum |
| 13 | Annex <D>: Adjacent channel co-existence evaluation |

# 2      References

R4-2305969, WF for SBFD feasibility study and requirements impact from UE aspect R4-2305971, TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements R4-2307355 R4-2308267 R4-2308633 R4-2308749

# 3      Text proposal

Begin changed section ******************************************************

10.7 FR2-1 Feasibility of UE aspects

10.7.1    Interference analysis

10.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification.

Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Sub-band filtering for a SBFD-aware UEs – UEs with a new feature

SBFD-aware UEs support half duplex operation while receiving the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study for future releases. In the present study, legacy UEs are the main focus.

For new SBFD aware UEs, we need further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered sufficient for the purpose of system studies for SBFD, therefore AGC effect on self-noise is not modelled. RAN4 decided on a NF of 10dB.

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference . An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The effect of all these distortions is lumped into a single parameter we call selectivity. Based on the discussion and analysis from the meeting, contributions suggested possible the sub-band selectivity values from 20 dB to 34 dB. The receiver performance is simply modelled as being 23 dB below the jammer power level.

The definition of Sub-band/In-channel selectivity is introduced for clarity in the SBFD feasibility study:

- Subband in-channel selectivity is the ratio of the received jammer power in the adjacent uplink subband as measured before FFT operation, to the interference power in the assigned downlink subband as measured after the FFT operation

FFT leakage and selectivity

In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not time-synchronized with the DL sub-band. The analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. Nonetheless, this interference will not be any worse than the selectivity value. For this reason, the 23 dB was agreed upon for modeling the inter-sub-band selectivity.

10.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, did not conclude on the typical values so we are using the formulation from the MPS.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity,

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 10.7.1.1-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 10.7.1.1-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB |  |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

10.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The receiver’s performance is simply represented as 34 dB lower than the jammer power level

10.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. UE ACLR is modeled as 24 dB at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is 34 dB. This model is an approximation of the performance of a typical UE.

10.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

End changed section***********************************************************