3GPP TSG-RAN WG4 Meeting #107	R4-2309825

Incheon, South Korea, 22th May- 26th May, 2023

Title: 	WF for FR2 HST demodulation requirements

Agenda Item:	8.13.5

Source: 	Samsung

Document for:	Approval

# Topic #1: Deployment and Channel Modelling

## Sub-topic 1-1: Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in open space scenario

Issue 1-1-1: Deployment Scenario for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario

Agreement:

- RAN4 will consider both scenario A and scenario B as starting point for initial evaluation with simultaneous multi-Rx reception

  - FFS whether single requirement or separate requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements

Issue 1-1-2: Beam serving coverage for multi-Rx reception in Bi-directional scenario

Agreement:

- Apply the following generic Bi-directional deployment for simultaneous multi-Rx reception scenario with considering symmetrical beam coverage areas both in Scenario-A and Scenario-B for demodulation requirement definition as baseline

- The related beam serving coverage for left panel and right panel of each RRH can be considered as

  - Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1, …

  - Right panel: [k *Ds+ Ds_offset, (k+1) *Ds+ Ds_offset] for the kth RRH, k=0,1, …

  - The following RRH parameters for scenario A and scenario B can be used for initial evaluation

| Parameters | Description | Scenario A | Scenario B-1 | Scenario B-2 |
| --- | --- | --- | --- | --- |
| Ds | Inter-RRH distance | 700m | 700m | 700m |
| Dmin | Distance between rail track and RRH | 10m | 150m | 150m |
| v | Train velocity | 350km/h | 350km/h | 350km/h |
| fd | Maximum Doppler frequency shift | 9722Hz | 9722Hz | 9722Hz |
| fc | Carrier frequency | 30GHz | 30GHz | 30GHz |
| Drx_panel | Distance between two UE Rx panels | 0m | 0m | 0m |
| Ds_offset | Switching transmission point between adjacent RRHs | 10m | 100m | 350m |

Issue 1-1-3: Channel model for PDSCH requirements with Multi-Rx Reception

Agreement:

- No need to model the relative propagation delay from the visible RRH into the channel modelling

  - FFS whether need to relative power for channel modelling

- The following model on doppler shift trajectory can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario as baseline

  - FFS on the starting point used

- Doppler shift fst=fd cosθt (Hz) from the left Tx panel of RRH for PDSCH received at right Rx panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right Tx panel of RRH for PDSCH received at left Rx panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

## Sub-topic 1-2 Deployment and Channel Model in tunnel scenario

Issue 1-2-1: Deployment in Tunnel Scenario

Agreement:

- Prioritized the UE receives signals with single-panel reception assumption with HST-DPS for Uni-directional deployment scenario as a starting point for initial performance evaluation in tunnel scenario deployment

- Bi-directional deployment scenarios are not precluded for initial performance evaluation conditioned on the feasibility study from RRM session

Issue 1-2-2: RRH parameters for Channel Model in tunnel scenario

Agreement:

- Aligned with RRM assumption for tunnel scenario deployment initial study, RAN4 use the following RRH parameters for channel modelling in the tunnel deployment scenario

  - Ds=700m

  - Dmin=1m

  - Take Ds_offset =5m for Uni-directional scenario as baseline

    - Other value of Ds_offset is not precluded

Issue 1-2-3: Channel Model in Tunnel Scenario for UE demodulation

Agreement:

- FFS

  - Option 1: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset

    - Uni-directional scenario

      - Doppler shift fst=fd cosθt (Hz) for PDSCH received at UE is given by

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 350 km/h |
| _max | 9722 Hz |

  - Option 2: Considering alternative scenario with train moving in the same direction with serving beam.

    - Uni-directional scenario

      - Doppler shift fst=fd cosθt (Hz) for PDSCH received at UE is given by

cosθt=-Ds_offset +Ds-vtDmin2+-Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 350 km/h |
|  | 9722 Hz |

  - Other options are not precluded

    - Interested companies can provide the detail modelling

Issue 1-2-4: Channel Model  in Tunnel Scenario for BS demodulation

Agreement:

- FFS

  - Option 1: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset

    - Uni-directional scenario

      - Doppler shift fst=fd cosθt (Hz) for PUSCH received at gNB is given by

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 350 km/h |
|  | 19444 Hz |

  - Option 2: Considering alternative scenario with train moving in the same direction with serving beam.

    - Uni-directional scenario

      - Doppler shift fst=fd cosθt (Hz) for PUSCH received at gNB is given by

cosθt=-Ds_offset +Ds-vtDmin2+-Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 350 km/h |
|  | 19444 Hz |

  - Other options are not precluded

    - Interested companies can provide the detail modelling

# Topic #2: UE demodulation for CA requirement

Issue 2-1-1:  Carrier combination for requirements

Agreement:

- Define Rel-18 FR2 CA PDSCH demodulation requirement with carrier combination as only TDD 120KHz SCS+TDD 120KHz SCS

- Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations

Issue 2-1-2:  Whether existing SNR requirement or 200MHz CBW defined in single carrier can be reused for CA case with 200MHz

Agreement:

- For FR2 HST CA Demod requirements, RAN4 will define requirements for the agreed BW carrier configuration based on collected impairment results provided by companies.

Issue 2-1-3:  RRH and Channel model assumption for CA

Agreement:

- For FR2 HST CA Demod requirements, RAN4 will define requirements for the agreed BW carrier configuration based on collected impairment results provided by companies.

Issue 2-1-4/5:  Additional noise margin for CA requirement based on single carrier

Agreement:

- No additional noise and imperfection model modelling based on single carrier link level simulation for PDSCH CA alignment result is considered.

- The impact of additional noise in CA can be considered into the impairment results up to companies’ implementation

Issue 2-1-6:  Simulation results alignment and requirement derivation

Agreement:

- Companies are encouraged to provide the simulation result in the next meeting for further alignment according to the simulation assumption R4-2305893

- The requirement derivation and whether to add additional margin will be discussed based on the collected simulation results.

Issue 2-1-7: Test applicability rule

Agreement:

- Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST.

- For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority

Issue 2-1-8: Release independent

Way Forward

- FFS on Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

# Topic #3: PDSCH requirement with multi-Rx reception

## Sub-topic 3-1: General for PDSCH requirement with multi-Rx reception

Issue 3-1-1: UE processing assumption for the FFT window

Way Forward

- Proposals

  - Option 1: RAN4 shall consider that the CPE will have independent FFT per panel

  - Option 2:

    - Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added.

  - Option3

    - Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

## Sub-topic 3-2: Test setup for PDSCH requirement with multi-Rx reception

Issue 3-2-1: Capability for PDSCH requirement with simultaneous multi-Rx reception

Agreement:

- Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

  - UE power class 6 (PC6)

  - simultaneousReceptionDiffTypeD-r16

  - maxNumberActiveTCI-PerBWP > 1

- Additional necessary capabilities also need to be considered pending on specific test cases introduced

Issue 3-2-2: Transmission schemes

Agreement:

- FFS on sDCI schemes

Issue 3-2-3: Layer combination

Agreement:

- Companies are encouraged to provide initial results for both layer combination as 1+1 and 2+2 with full-overlapping scheduling based on mDCI transmission scheme in the next meeting

Issue 3-2-4: Test applicability rule of PDSCH with mDCI and sDCI if introduced both

Way forward:

- FFS whether to introduce FR2 HST Demodulation tests for simultaneous reception with sDCI, and related applicability rule to skip sDCI testing if the device supports mDCI

Issue 3-2-5: Simulation Assumption for initial simulation purpose

Way forward:

- Carrier frequency: 30GHz.

- CBW/SCS: 200MHz/120kHz

- TDD pattern: FR2.120-1 (DDDSU)

- MCS: MCS 17 with Rank 2 as baseline

- Number of Active PDSCH TCI states: 2

- Other parameters such as number of HARQ process, K1 values can be reused from Rel-17 FR2 HST

- Rank and Layer combination as a starting point for evaluation

  - Case 1: 1+1 as layer combination with fully over lapping

  - Case 2: 2+2 as layer combination with fully over lapping

- Companies are encouraged to provide simulation results in the next meeting

Table 1. PDSCH Simulation assumptions (For information)

| Parameter |  | Value |
| --- | --- | --- |
| Carrier frequency |  | 30 GHz |
| Duplex mode |  | TDD |
| SCS |  | 120 kHz |
| CBW |  | 200 MHz |
| Antenna configuration |  | 2x2 ULA Low for each Panel |
| TDD pattern |  | DDDSU, S: 10D + 2G + 2U Schedule PDSCH in special slots (Note 1) |
| PDSCH DMRS configuration | DMRS type | Type 1 |
|  | Number of additional DMRS | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 1 |
|  | Length (L) | 13 |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RGB size | Config2 |
| PTRS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (TPT-RS) | 1 |
| MCS and Rank |  | Case 1: MCS 17 for 1+1 Case 2: MCS 17 for 2+2 |
| Number of HARQ processes |  | 8 |
| SSB periodicity |  | 20ms |
| TRS periodicity |  | 10ms or 20ms (Note 1) |
| Propagation conditions |  | The details can be found in issue 1-1-3: Channel model for PDSCH requirements with Multi-Rx Reception |
| Maximum Doppler shift |  | 9722 |
| Test metric |  | SNR at 70% of maximum throughput |
| Note 1: No PDSCH scheduling |  |  |

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 1 and 2 |
|  |  |  |  | 3 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 8 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 5 and 6 |
|  |  |  |  | 3 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 9 and 10 |
|  |  |  |  | 3 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 15 |
|  |  |  |  | l0 = 8 for CSI-RS resource 14 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 13 and 14 |
|  |  |  |  | 3 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 17 and 18 |
|  |  |  |  | 3 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 23 |
|  |  |  |  | l0 = 8 for CSI-RS resource 22 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 21 and 22 |
|  |  |  |  | 3 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 25 and 26 |
|  |  |  |  | 3 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 31 |
|  |  |  |  | l0 = 8 for CSI-RS resource 30 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 2 for CSI-RS resource 29 and 30 |
|  |  |  |  | 3 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |

# Topic #4: Demoduation requirements for tunnel scenario

## Sub-topic 4-1: Test Scope of demodulation requirements of tunnel scenario

Issue 4-1-1: Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST

Way forward:

- Option 1

  - Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario.

    - Uni-directional with DPS scheme 1b

    - Bi-directional with DPS scheme 1a

- Option 2

    - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced.

- Option 3

    - RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases;

- Option 4

    - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Issue 4-1-2: Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST

Way forward:

- Option 1

  - Introduce single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario.

- Option 2

  - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced.

  - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

- Option 3

  - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.