3GPP TSG-RAN WG4 Meeting #107	R4-2309827

Incheon, Korea (Republic of), May 22nd ‒ May 26th, 2023

Title: 	WF for IoT over NTN SAN demodulation requirements

Agenda Item:	9.7.8

Source: 	Ericsson

Document for:	Approval

# Topic #1: Common issue for eMTC and NB-IoT

How to introduce measurement uncertainties and test tolerance for IoT NTN conducted and radiated performance requirements in TS 36.181

Agreement:

- The eMTC and NB-IoT performance relevant measurement uncertainties and test tolerances defined in TS36.141 Table 4.1.2-3 and Table G.3-1 could be reused for IoT NTN. The content of the table can be further checked.

Test applicability of 1 Rx tests and 2 Rx tests in TS 36.181

Way forward:

- FFS how to specify the applicability rule of 1 Rx tests and 2 Rx tests.

  - Option 1: Reuse Rel-17 NR NTN applicability rule.

  - Other options are not precluded.

- This issue does not affect to the completion of performance part.

# Topic #2: SAN demodulation requirement for eMTC

Channel model for PUSCH demodulation requirements

Agreement:

- Both NTN-TDLA100-5 and NTN-TDLC5-5 for PUSCH CE mode A and mode B

Whether a manufactory declaration and an applicability rule are needed for PUSCH CE mode A and B

Agreement:

- Reusing the existing manufacture declaration approach for PUSCH, PUCCH and PRACH requirements in TS 36.104/141.

Preamble parameters for PRACH BF0/1/2/3

Agreement:

- Reuse legacy LTE eMTC PRACH parameters

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

PRACH configuration index (prach-ConfigurationIndex)

Agreement:

- Apply (3, 19, 35, 51) for PRACH format (0, 1, 2, 3) separately. (Same as the existing eMTC PRACH requirements in TS36.104).

# Topic #3: SAN demodulation requirement for NB-IoT

Channel model for NPUSCH format 1 demodulation requirements (3.75kHz single tone and 15kHZ 12 tones)

Agreement:

- NTN_TDLA100-1 and NTN_TDLC5-1

Largest Doppler offset for NPUSCH format 1 (3.75kHz SCS)

Agreement:

- Set 128Hz (to align with PUSCH CE mode B with same transmission duration)

NPRACH preamble test parameters

Agreement:

- Reuse the configuration as the exiting NPRACH test parameters in TS36.141

| Parameters | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |

# Topic #4: SAN demodulation requirements

SAN demodulation requirements in TS 36.108 and TS 36.181

Way forward:

- RAN4 sets the SAN demodulation requirements in TS36.108/TS36.181 according to the simulation result summary R4-2308428 in RAN4#107. In RAN4#108, companies are encouraged to submit the updated simulation results, and RAN4 will discuss how to define the requirements if the span of the results are still large among companies.