3GPP TSG-RAN WG4 Meeting #107 	R4-2309845

Incheon, Korea, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | 0007 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.115-2: Clarifcations for repeater test models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Perf |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2 test models for repeaters included not correct list of respective requirements for which test models should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected list of requirements for test model RUL-FR2-TM1.1 - Editorial correction of RUL-FR2-TM2 - Corrected list of requirements for test model RUL-FR2-TM2a - Editorial correction of RUL-FR2-TM3.1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would include not correct list of requirements for which repeater test models are used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.3.1, 4.9.2.3.2, 4.9.2.3.2a, 4.9.2.3.3, 4.9.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2308616 |  |  |  |  |  |  |  |  |

<Start of changes>

##### 4.9.2.3.1	FR2 test model 1.1 (RUL-FR2-TM1.1)

This model shall be used for tests on:

-	OTA output power (EIRP)

-	OTA Repeater output power (TRP)

-	OTA out of band gain

-	OTA Unwanted emissions

-	OTA ACLR

-	OTA operating band unwanted emissions

-	OTA transmitter spurious emissions

-	OTA input intermodulation

-	OTA ACRR

-	OTA transmit ON/OFF power

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM1.1 are defined in table 4.9.2.3.1-1.

Table 4.9.2.3.1-1: Specific physical channel parameters of RUL-FR2-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PUSCH nRNTI=0 | NRB |
| Modulation PUSCH nRNTI=0 | QPSK |

##### 4.9.2.3.2	FR2 test model 2 (RUL-FR2-TM2)

This model shall be used for tests on:

-	Uplink OTA repeater EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM2 are defined in table 4.9.2.3.2-1 for 64QAM. For 16QAM and QPSK, specific physical channel parameters for RUL-FR2-TM2 are defined in table 4.9.2.3.2-1 with 64QAM PUSCH PRB replaced with selected modulation order PUSCH PRB according to the corresponding test procedure.

Table 4.9.2.3.2-1: Specific physical channel parameters of RUL-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PUSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PUSCH PRBs which are not allocated | NRB-1 |

##### 4.9.2.3.2a	FR2 test model 2a (RUL-FR2-TM2a)

This model shall be used for tests on:

-	Uplink OTA repeater EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM2a are defined in table 4.9.2.3.2-1 with all 64QAM PUSCH PRBs replaced by 256QAM PUSCH PRBs.

##### 4.9.2.3.3	FR2 test model 3.1 (RUL-FR2-TM3.1)

This model shall be used for tests on:

-	Transmitted signal quality

-	OTA frequency stability (at max power)

-	Uplink OTA repeater EVM for modulation (at max power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM3.1 are defined in table 4.9.2.3.1-1 with all QPSK PUSCH PRBs replaced with selected modulation order PUSCH PRBs according to the corresponding test procedure.

##### 4.9.2.3.4	FR2 test model 3.1a (RUL-FR2-TM3.1a)

This model shall be used for tests on:

-	Transmitted signal quality

-	OTA frequency stability (at max power)

-	Uplink OTA repeater EVM for 256QAM modulation (at max power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM3.1a shall be defined in table 4.9.2.3.1-1 with all QPSK PUSCH PRBs replaced by 256QAM.

<End of changes>