3GPP TSG-RAN WG4 Meeting #107	R4-2309892

Incheon, South Korea, 22 – 26 May 2023

Source:	China Telecom

Title:	WF on advanced receiver for MU-MIMO scenario

Agenda Item:	8.19.3

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

  - R4-2305914 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106bis-e

<Topic #1: Assumptions for the advanced receiver for MU-MIMO>

Sub-topic 1-1 Reference receiver

Reference receiver

Candidate options:

  - Option 1: Down select to R-ML as the reference receiver

  - Option 2: Make decision later

  - Option 3: Keep open in case requirements are to be defined for up to 4 total layers and with high modulation orders

Sub-topic 1-2 Discussion on the required information

Sub-topic 1-2-1: Timeline

Timeline for phase I studying

  - Update the WID in the June RAN plenary to add the core part objective, and to add RAN1 and RAN2 as the secondary WG.

Sub-topic 1-2-2: Required information study

The modulation order information of the co-scheduled UE (Only required for R-ML)

On the evaluation assumption of modulation order blind detection

  - It’s encouraged interested companies to further evaluate following case:

  - Also evaluate the following case with more than 1 co-scheduled UEs:

    - Target UE: Full CHBW allocation (52PRBs) with MCS 13 rank 1, 2T2R, TDLC300-100, random precoding

    - Co-UE1: Partial CHBW allocation (0~25 PRBs) with QPSK rank 1

    - Co-UE2: Partial CHBW allocation (26~51 PRBs) with 16QAM rank 1

Additional RRC-based network assistant signaling:

  - Introduce RRC signaling to discriminate MCS table with 256QAM or 1024 QAM enable or not for co-scheduled UEs (optional)

Candidate options on the additional assumptions to the R-ML receiver:

  - Proposal 1: The total number of layers for target and co-scheduled UE are no more than 4

  - Proposal 2: Limit the study to DMRS configurations of dmrs-Type=1 with maxLength=1

Other required information of the co-scheduled UE for both R-ML and E-IRC

| Information | RAN4 Default assumption (If N/A, how could be obtained by the UE) | Signalling if RAN4 default assumption not valid | Way forward on the signalling details if introduced |
| --- | --- | --- | --- |
| The DMRS port information for the co-scheduled UE | N/A (Obtained by UE blind detection) | N/A | FFS whether additional RRC based assistant signalling can be considered. |
| PRB bundling size for the co-scheduled UE Frequency domain resource allocation for the co-UE within each PRG of the target UE | UE assume in each its PRG, the resource allocation and precoding of the potential DMRS sequence aligned co-scheduled UE(s) in other DM-RS ports of different CDM group are aligned with PRG=2 or 4. | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| DMRS power boosting for the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| Time domain resource allocation information of the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| Frequency domain resource allocation for the co-UE across different PRGs of the target UE: | N/A (Obtained by UE blind detection) | N/A | No signalling on frequency domain resource allocation information. |
| CSI-RS location of co-scheduled UE (Only required for R-ML) | UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE | Down-select to one of the below options in the next meeting: Option 1: No RRC signalling is needed Option 2: 1-bit RRC signaling |  |

Sub-topic 1-2-3: UE capability definition

Capability signalling for advanced receiver for MU-MIMO (If introduced)

Candidate options

  - Option 1: Define optional UE capability signaling on MU-MIMO advanced receiver capability:

    - Option 1A: UE supporting R-ML receiver with and without modulation order blind detection

    - Other options are not precluded

<Topic #2: Phase I simulation assumptions and results>

The number of co-scheduled UEs

  - In the phase I study for the advanced receiver with genie aided knowledge of the required information

    - Only consider 1 co-scheduled UE

MCS for the target UE

  - MCS 13 for rank 1 and rank 2

  - MCS 17 for rank 2

  - MCS 4 for rank 1

Modulation order for the co-scheduled UE

  - For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose)

    - For rank 1+1: QPSK

    - For rank 2+2: 64QAM

    - For rank 2+2: QPSK

    - For rank 2+2: 16QAM

PDSCH resource allocation for the target and co-scheduled UE

  - In the phase I study for the advanced receiver with genie aided knowledge of the required information

    - Only consider the scenario with full FDRA for both target and co-scheduled UEs

Phase I Simulation result collection

  - Companies are encouraged to bring phase I simulation results on the agreed cases to the Aug RAN4 meeting

  - The updated simulation result collection template will be shared before the Aug RAN4 meeting