3GPP TSG-RAN WG4 Meeting #107	R4-2310014

Inchon, Korea, May 22 – May 26, 2023

Agenda item:			8.7.4

Source:	Moderator (Xiaomi)

Title:	Topic summary for [107][131]FR2_enh_req_Ph3_part2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

The contributions for the following agenda items are summarised in this document:

8.7.2 UL 256QAM

Topic #1: MPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307043 | Anritsu Limited | Observation 1: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 29.55GHz. Observation 2: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 39GHz. |
| R4-2307931 | RICHTEK KOREA | Observation 1: EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing. Proposal 1: Both Our proposal and Qualcomm’s proposal for the phase noise profiles can be used for 256QAM MPR evaluation. Observation 2: The maximum dynamic range is the difference between the minimum peak EIRP and the minimum EIRP requirements for EVM test. In the last meeting, the MPR limitation issue was discussed because of the limited maximum dynamic range. Therefore, considering the maximum dynamic range issue, we prefer option 1. |
| R4-2308168 | ZTE Corporation | Observation 1: Example 1-based phase noise models with parameters updated can provide comparative lower phase noise. Observation 2: For the 39GHz PN model, the values of the PN model of new 2 and new 3 are very close. These two PN models for 39GHz be merged into one option. Proposal 1: To consider example 1-based phase model with lower phase noise for MPR simulation, which are: - For 29GHz, either new1 or new 2 is feasible - For 39GHz, we slight prefer to new 3. |
| R4-2308223 | LG Electronics France | Observation 1: For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1. Observation 2: For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM. Observation 3: The MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the case 1. Proposal 1: For 29 GHz, We can use the case 1,case 2 and case 3 for MPR simulation. Proposal 2: We need to discuss more how to get the reasonable EVM margin for 39 GHz. Proposal 3: We need to discuss about reasonable MPR value based on the MPR simulation results and EVM test dynamic range analysis. |
| R4-2308226 | Ericsson Limited | Proposal 1: The MPR for UL 256QAM should be in the range of 1dB - 3dB higher than the corresponding value for 64QAM. |
| R4-2308227 | vivo | Proposal 1: Either option 3 or option 4 can be used for MPR evaluation in 29GHz bands. Proposal 3: Focus on MPR evaluation in 29GHz bands first, and further discuss feasible phase noise profile in 39GHz bands. |
| R4-2308805 | Xiaomi | Proposal 1: Both of the phase noise profiles from Qualcomm and MTK for 29GHz can be used in MPR simulation. - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from Qualcomm PSD0; 32 dB. 1; 3e3; 2.37; 1; 3.3. 2; 6e5; 4.7; 8e5; 5.3. 3; 8e5; 2.0; 1.1e6; 2.5. - Parameters from MTK Proposal 2: The phase noise profile from MTK for 39GHz can be used in MPR simulation. - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from MTK |
| R4-2309028 | Sony | Observation 1: the phase noise examples in TR38.803 does not leave enough EVM margin for 256QAM evaluation. Observation 4: It is feasible for implementations to meet the proposed confinement range. Proposal 1: Improved phase noise profile than examples in TR38.803 should be used as the baseline for MPR simulations Proposal 2: The MPR of UL 256 QAM needs to be confined so that the UE can reach reasonable EIRP levels in a real network scenario. Proposal 3: It is proposed that the MPR for UL 256 QAM shall not exceed 3 dB more than 64QAM. |
| R4-2309074 | Apple | Observation 2: For lower order modulations the EVM budget is high so that the allowance is governed by power amplifier non-linearity and Tx chain distortions. To comply with the considerably tighter EVM budget for 256QAM the power amplifier needs to back-off and linearity is increased. With reduced influence of power amplifier on the total budget other factors such as thermal and DAC noise start to play a role. This is not included in the -1dB/dB scaling. Observation 3: Considering advanced UE implementation technologies for MPR confinement has the challenge that those require clarification as they might reach beyond typical RAN4 assumptions. Detailed analysis and measurements would be needed to support and verify simulated performance improvements. |
| R4-2309281 | Qualcomm Incorporated | Proposal 6: The MPR for UL 256 QAM shall not exceed that of 64QAM by more than 3 dB. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 Phase noise profile

Sub-topic description:

Open issues and candidate options before meeting:

Candidate options for modified phase noise profile in last meeting:

For 29GHz:

- Example 1 in TR38.803 for 29 GHz.

- Example 2 in TR38.803.

- New phase noise model using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Parameters from Qualcomm

| PSD0 | 32 dB |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 6e5 | 4.7 | 8e5 | 5.3 |
| 3 | 8e5 | 2.0 | 1.1e6 | 2.5 |

  - Parameters from  MTK

For 39GHz:

- Example 2 in TR38.803.

- Example 1-based using following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Parameters from vivo

  - Parameters from Anritsu

- New phase noise model using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

  - Parameters from MTK

- Adopt min(example1, example2) as the phase noise profile for UL256QAM, where ‘example2’ refers to the example phase noise profiles in TR38.803 and ‘example1’ refers to Example 1-based for 39GHz.

Summary of the simulation results to evaluate the phase noise profiles:

29GHz:

| Companies | Waveform | Phase noise model @29GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -31.1 | -31.5 | 0.4 |
|  |  | UE example 2 |  |  | -26.6 | -28.4 | 1.8 |
|  |  | From Qualcomm |  |  | -34.8 | -36.0 | 1.2 |
|  |  | From MTK |  |  | -35.4 | -36.0 | 0.7 |
|  | DFT-s-OFDM | UE example 1 |  | N_group = 8, N_samp = 4 | -31.2 | -30.1 | -1.0 |
|  |  | UE example 2 |  |  | -26.0 | -30.1 | 4.1 |
|  |  | From Qualcomm |  |  | -34.8 | -34.9 | 0.2 |
|  |  | From MTK |  |  | -35.4 | -34.7 | -0.7 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -28.3288 | -28.9129 | 0.5841 |
|  |  | UE example 2 |  |  | -25.3244 | -25.5883 | 0.2639 |
|  |  | From Qualcomm |  |  | -31.7997 | -33.0798 | 1.2801 |
|  |  | From MTK |  |  | -33.0942 | -33.3608 | 0.2666 |
|  | DFT-s-OFDM | UE example 1 |  | N_group = 8, N_samp = 4 | -28.8567 | -27.1551 | -1.7016 |
|  |  | UE example 2 |  |  | -25.4507 | -28.9614 | 3.5107 |
|  |  | From Qualcomm |  |  | -32.8215 | -29.8556 | -2.9659 |
|  |  | From MTK |  |  | -33.2358 | -30.0408 | -3.195 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.65 |  |  |
|  |  | UE example 2 |  |  | -28.6 |  |  |
|  |  | From Qualcomm |  |  | -32.5 |  |  |
|  |  | From MTK |  |  | -32.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -27.7 | -28.3 | 0.6 |
|  |  | From Qualcomm |  |  | -30.7 | -32.5 | 1.8 |
|  |  | From MTK |  |  | -31.6 | -32.7 | 1.1 |
| LGE R4-2308223 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 |  | -30.75 |  |
|  |  | From Qualcomm |  |  |  | -31.57 |  |
|  |  | From MTK |  |  |  | -31.37 |  |
| Sony R4-2309028 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.3 dB |  |  |
|  |  | From Qualcomm |  |  | -32.9 dB |  |  |
|  |  | From MTK |  |  | -33 dB |  |  |
|  | DFT-s-OFDM | UE example 1 |  |  | -26.9 dB |  |  |
|  |  | From Qualcomm |  |  | -31.4 dB |  |  |
|  |  | From MTK |  |  | -31.4 dB |  |  |

39GHz:

| Companies | Waveform | Phase noise model @39GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 | -27.8 | -28.1 | 0.3 |
|  |  | Example 1from Anritsu |  |  | -28.1 | -28.5 | 0.4 |
|  |  | Example 2 from TR38.803 |  |  | -24.1 | -25.8 | 1.7 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -32.2 | -33.3 | 1.1 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -32.2 | -33.3 | 1.1 |
|  |  | From MTK |  |  | -32.4 | -33.0 | 0.6 |
|  | DFT-s-OFDM | Example 1 from VIVO |  | N_group = 8, N_samp = 4 | -27.9 | -26.7 | -1.2 |
|  |  | Example 1from Anritsu |  |  | -28.2 | -27.0 | -1.2 |
|  |  | Example 2 from TR38.803 |  |  | -23.6 | -27.5 | 4.0 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -32.1 | -32.1 | 0.0 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -32.1 | -32.2 | 0.1 |
|  |  | From MTK |  |  | -32.4 | -31.6 | -0.8 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.1733 | -25.3235 | 0.1502 |
|  |  | UE example 2 |  |  | -22.7246 | -22.9743 | 0.2497 |
|  |  | From Qualcomm |  |  | -29.8719 | -30.2049 | 0.333 |
|  |  | From MTK |  |  | -29.9468 | -30.182 | 0.2352 |
|  | DFT-s-OFDM | UE example 1 |  | N_group = 8, N_samp = 4 | -25.3171 | -23.5189 | -1.7982 |
|  |  | UE example 2 |  |  | -22.8672 | -26.3839 | 3.5167 |
|  |  | From Qualcomm |  |  | -30.0035 | -28.8569 | -1.1466 |
|  |  | From MTK |  |  | -30.0888 | -28.5366 | -1.5522 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -25.2 |  |  |
|  |  | UE example 2 |  |  | -26.2 |  |  |
|  |  | From Qualcomm |  |  | -29.3 |  |  |
|  |  | From MTK |  |  | -29.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.3 | -25.7 | 0.4 |
|  |  | From MTK |  |  | -28.7 | -29.6 | 0.9 |
| LGE R4-2308223 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 |  | -28.56 |  |
|  |  | Example 1 from Anritsu |  |  |  | -28.77 |  |
|  |  | From MTK |  |  |  | -30.96 |  |

Issue 1-1-1: Phase noise profiles evaluation for 29GHz

- Proposals

  - Option 1: Both of new phase noise profiles from Qualcomm and MTK for 29GHz are feasible for MPR simulation. (Anritsu, MTK, Xiaomi, ZTE, vivo, Sony)

    - New phase noise profiles using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

    - Parameters from Qualcomm

| PSD0 | 32 dB |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 6e5 | 4.7 | 8e5 | 5.3 |
| 3 | 8e5 | 2.0 | 1.1e6 | 2.5 |

    - Parameters from  MTK

  - Option 2: Example 1 in TR38.803 for 29GHz and new phase noise profiles from Qualcomm and MTK for 29GHz are feasible for MPR simulation. (LGE)

- Recommended WF

  - Option 1

Issue 1-1-2: Phase noise profiles evaluation for 39GHz

- Proposals

  - Option 1: The new phase noise profile from MTK for 39GHz is feasible for MPR simulation. (Xiaomi, ZTE)

    - New phase noise profile using the pole-zero method based on following function:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

    - Parameters from MTK

  - Option 2：The new phase noise profiles from MTK and adopting min(example1-based, example2) for 39GHz are feasible for MPR simulation, where ‘example1-based’ refers to the example phase noise profiles from vivo and Anritsu, ‘example2’ refers to the example phase noise profile in TR38.803. (Anritsu)

  - Option 3: Further discuss feasible phase noise profile for 39GHz. (Vivo, LGE)

- Recommended WF

  - TBA

Sub-topic 1-2 MPR requirement

Sub-topic description

Open issues and candidate options before meeting:

Summary of the simulation results for MPR from R4-2308223:

Table 6 MPR results for 29 GHz PC2 UL256QAM with margin

| CBW (100 MHz) | Modulation | Phase noise profiles | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | 64QAM (200MHz) |  | ≤ 5.0 | ≤ 5.5 |
|  | 256QAM | Example1 | 9 | 9 |
|  |  | QC | 8.5 | 9 |
|  |  | MDK | 8.5 | 8.5 |
| CP-OFDM | 64QAM (200MHz) |  | ≤ 7.5 | ≤ 7.5 |
|  | 256QAM | Example1 | 11 | 11 |
|  |  | QC | 11 | 11 |
|  |  | MDK | 11 | 11 |

| CBW (400 MHz) | Modulation | Phase noise profiles | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | 64QAM |  | ≤ 6.5 | ≤ 6.5 |
|  | 256QAM | Example1 | 10.5 | 10.5 |
|  |  | QC | 9.5 | 9.5 |
|  |  | MDK | 10 | 9.5 |
| CP-OFDM | 64QAM |  | ≤ 9.0 | ≤ 9.0 |
|  | 256QAM | Example1 | 12.5 | 12.5 |
|  |  | QC | 12 | 12 |
|  |  | MDK | 11.5 | 12 |

Issue 1-2: MRP requirements

- Proposals

  - Option 1: The reasonable MPR value should be defined based on the MPR simulation results and EVM test dynamic range analysis. (LGE)

  - Option 2: The MPR of UL 256 QAM needs to be confined so that the UE can reach reasonable EIRP levels in a real network scenario.

    - The MPR for UL 256 QAM shall not exceed 3 dB (i.e., 1dB~3dB) more than 64QAM. (Sony, Ericsson, Qualcomm)

- Recommended WF

  - TBA

Topic #2: EVM test

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307766 | Huawei, HiSilicon | Proposal 1: the minimum EIRP for UL 256 QAM for EVM test is relaxed by 8.5 dB compared to 64QAM, based on the minimum output power for different PCs Parameter; Unit; PC1; PC2; PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5. |
| R4-2307931 | RICHTEK KOREA | Observation 2: The maximum dynamic range is the difference between the minimum peak EIRP and the minimum EIRP requirements for EVM test. In the last meeting, the MPR limitation issue was discussed because of the limited maximum dynamic range. Therefore, considering the maximum dynamic range issue, we prefer option 1. Proposal 2: RAN4 to consider option 1 as the minimum EIRP requirements for EVM test. |
| R4-2308168 | ZTE Corporation | Proposal 2: To confirm option 1 as the minimum EIRP requirements for EVM test. |
| R4-2308223 | LG Electronics France | Proposal 4: Option 1, which provides more dynamic range margin, is suitable for minimum EIRP requirements. |
| R4-2308226 | Ericsson Limited | Proposal 2: Define the minimum UE EIRP for UL 256QAM as 18dB, 1dB and 8dB for PC1, PC2 and PC5, respectively. Proposal 3: For CP-OFDM, define (K=2, L=1) PTRS configuration for both the EVM test and the MPR requirements evaluations. Observation 1: To agree on removing PTRS partially or completely for DFT-s-OFDM for the EVM test and the MPR requirements evaluations more simulation results and study are needed. |
| R4-2308227 | vivo | Proposal 2: Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration for CP-OFDM, and do not configure PTRS for DFT-s-OFDM. |
| R4-2308805 | Xiaomi | Proposal 3: The minimum EIRP for UL 256 QAM for EVM test could be relaxed by 14 dB based on the difference between the SNR of 256QAM (29.1dB) and the SNR of QPSK(15.1dB): Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 18; 1; 8. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 256 QAM. Proposal 4: If necessary, one UE capability can be introduced to indicate the maximum channel bandwidth to support UL 256QAM. Proposal 5: For CP-OFDM, using a fixed PTRS configuration (K = 2, L = 1) for all 29GHz and 39GHz devices. Proposal 6: For DFT-s-OFDM, don’t specify PTRS configuration for all 29GHz and 39GHz devices. |
| R4-2309028 | Sony | Observation 2: a denser PTRS configuration does not seem to reduce the EVM performance of CP-OFDM but may affect the DFTs-OFDM. Observation 3: with the assumption of 18 dBm minimum EIRP, it is possible that no dynamic range is available if the corresponding MPR is more than 12 dB. Observation 4: It is feasible for implementations to meet the proposed confinement range. |
| R4-2309074 | Apple | Observation 1: Previous analysis on PTRS performance found that the gain is dependent on the device specific phase noise profile. Other results indicate that DFT-s-OFDM can provide negative gain in certain configurations. Mostly narrow allocations seem to benefit from PTRS while larger allocations might see no advantage or even negative effects. Proposal 1: In case use of UE PTRS density recommendation is not agreeable it is proposed to further evaluate the gain of PTRS with certain DFT-s and CP-OFDM configurations. Then specify PTRS several density configurations with respect to DFT-s and CP-OFDM allocations so that the expected gains are positive for the considered PN profiles. EVM budget is quite tight for 256QAM so that this evaluation effort should be justified. Observation 2: For lower order modulations the EVM budget is high so that the allowance is governed by power amplifier non-linearity and Tx chain distortions. To comply with the considerably tighter EVM budget for 256QAM the power amplifier needs to back-off and linearity is increased. With reduced influence of power amplifier on the total budget other factors such as thermal and DAC noise start to play a role. This is not included in the -1dB/dB scaling. Proposal 2: For EVM testing PC1, PC2 and PC5 the minimum EIRP requirements can be defined according to Option 2. It is acceptable as it uses -1dB/dB relation and together with an 1dB correction factor to account for the nearby noise floor. |
| R4-2309281 | Qualcomm Incorporated | Proposal 1: UL PTRS must be justified by UL throughput gains prior to being made mandatory for FR2 UL 256QAM. Observation 1: If the network chooses to ignore a UE’s recommendation for UL PTRS configuration, the burden is the network’s, not the UE’s. Observation 2: An FR2-1 UE is expected to comply with Tx signal quality requirements for BPSK, QPSK, 16QAM and 64QAM without any PTRS configured for UL. Proposal 2: New capability signaling is introduced to convey to the network if the UE needs PTRS assistance to meet signal quality requirements for FR2 UL 256QAM. Proposal 3: A fixed PTRS configuration is adopted for each waveform type (CP- or DFT-s) and applicable only to UEs that indicate via signaling they need PTRS help to meet Tx signal quality requirements. Proposal 4: For FR2-1 UL256QAM using CP-OFDM the following PTRS configuration is established for UEs that signal they need PTRS to meet Tx signal quality requirements: Adopt K=2, L=1 for CP-OFDM. Observation 3: Unlike CP-OFDM, it is not clear how to adjust PTRS parameters when the number of symbols in each OFDM symbol (12*LCRB) starts to become comparable to the product Nsampgroup, * NPT-RSgroup. Proposal 5: For FR2-1 UL with DFT-s-256QAM, the following PTRS configuration is established for UEs that signal they need PTRS to meet Tx signal quality requirements: - Adopt 4 samples/group and 8 groups/OFDM symbol for DFT-s-OFDM for narrow allocations (20 RBs or narrower). - PTRS is not configured for DFT-s-OFDM for allocations wider than 20 RBs. - RAN4 to agree on PTRS configuration strategy for very narrow allocations (<= 8RB) to ensure overhead is kept in check. Proposal 7: The minimum EIRP for EVM compliance for UL 256 QAM shall not exceed that of 64QAM by more than 7 dB. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 Minimum EIRP

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: The minimum EIRP requirements for EVM test

- Proposals

  - Option 1: The minimum EIRP for UL 256 QAM for EVM test is relaxed by 14 dB compared to QPSK (or more than 7dB compared to 64QAM). (Xiaomi, ZTE, MTK, Vivo, Ericsson, LGE, Qualcomm)

| Parameter | Unit | Level for PC1 | Level for PC2 | Level for PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 64 QAM | dBm | 11 | -6 | 1 |
| UE EIRP for UL 256 QAM | dBm | 18 | 1 | 8 |
| Operating conditions |  | Normal Conditions |  |  |

  - Option 2: The minimum EIRP for UL 256 QAM for EVM test is relaxed by 8.5 dB compared to 64QAM. (Apple, Huawei)

| Parameter | Unit | PC1 | PC2 | PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 64 QAM | dBm | 11 | -6 | 1 |
| UE EIRP for UL 256 QAM | dBm | 19.5 | 2.5 | 9.5 |

- Recommended WF

  - TBA

Sub-topic 2-2 PTRS

Sub-topic description

Open issues and candidate options before meeting:

Summary of the simulation results for PTRS configuration with different PTRS correction and RBs allocation:

For CP-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -34.0 | -34.2 | -33.6 | 0.2 | -0.6 |
|  |  | UE example 2 | -26.9 | -28.1 | -27.6 | 1.2 | -0.5 |
|  |  | From Qualcomm | -36.0 | -36.9 | -36.4 | 0.9 | -0.5 |
|  |  | From MTK | -37.9 | -38.4 | -37.8 | 0.5 | -0.6 |
|  | 120kHz, 16RBs | UE example 1 | -31.9 | -32.3 | -32.1 | 0.4 | -0.2 |
|  |  | UE example 2 | -26.7 | -28.3 | -28.0 | 1.6 | -0.3 |
|  |  | From Qualcomm | -35.2 | -36.3 | -36.0 | 1.1 | -0.2 |
|  |  | From MTK | -36.1 | -36.7 | -36.5 | 0.6 | -0.2 |
|  | 120kHz, 64RBs | UE example 1 | -31.1 | -31.5 | -31.5 | 0.4 | -0.1 |
|  |  | UE example 2 | -26.6 | -28.4 | -28.3 | 1.8 | -0.1 |
|  |  | From Qualcomm | -34.8 | -36.0 | -36.0 | 1.2 | -0.1 |
|  |  | From MTK | -35.4 | -36.0 | -35.9 | 0.7 | -0.1 |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -33.9173 | -35.0024 |  | 1.0851 |  |
|  |  | From MTK | -35.6699 | -36.7056 |  | 1.0357 |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3388 | -33.9874 |  | 0.6486 |  |
|  |  | From MTK | -34.4442 | -34.8899 |  | 0.4457 |  |
|  | 120kHz, 16RBs | From Qualcomm | -33.4905 | -33.9333 |  | 0.4428 |  |
|  |  | From MTK | -33.9908 | -34.2379 |  | 0.2471 |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.5213 | -34.0762 |  | 1.5549 |  |
|  |  | From MTK | -33.2888 | -34.1761 |  | 0.8873 |  |
|  | 120kHz, 64RBs | From Qualcomm | -31.7997 | -33.0798 |  | 1.2801 |  |
|  |  | From MTK | -33.0942 | -33.3608 |  | 0.2666 |  |

For DFT-s-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, N_samp = 4 | Net benefit of PTRS N_group = 4, N_samp = 4 | Net benefit of PTRS N_group = 2, N_samp = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -34.1 | -33.6 | -34.4 | -34.3 | -0.4 | 0.3 | 0.2 |
|  |  | UE example 2 | -26.8 | -30.9 | -29.0 | -26.8 | 4.1 | 2.1 | 0.0 |
|  |  | From Qualcomm | -36.0 | -36.6 | -37.0 | -36.6 | 0.6 | 1.0 | 0.6 |
|  |  | From MTK | -37.9 | -38.1 | -38.6 | -38.4 | 0.2 | 0.7 | 0.5 |
|  | 120kHz, 10RBs | UE example 1 | -32.5 | -32.1 | -32.2 | -32.1 | -0.4 | -0.3 | -0.4 |
|  |  | UE example 2 | -26.2 | -29.8 | -27.9 | -26.1 | 3.6 | 1.7 | -0.1 |
|  |  | From Qualcomm | -35.3 | -35.7 | -35.6 | -35.2 | 0.4 | 0.3 | -0.1 |
|  |  | From MTK | -36.5 | -36.6 | -36.6 | -36.4 | 0.1 | 0.1 | -0.2 |
|  | 120kHz, 16RBs | UE example 1 | -32.0 | -31.4 | -31.4 | -31.3 | -0.6 | -0.6 | -0.7 |
|  |  | UE example 2 | -26.2 | -29.9 | -27.9 | -26.0 | 3.7 | 1.7 | -0.2 |
|  |  | From Qualcomm | -35.1 | -35.4 | -35.2 | -34.8 | 0.3 | 0.1 | -0.3 |
|  |  | From MTK | -36.1 | -35.9 | -35.9 | -35.7 | -0.2 | -0.2 | -0.4 |
|  | 120kHz, 64RBs | UE example 1 | -31.2 | -30.1 | -30.0 | -29.9 | -1.1 | -1.2 | -1.3 |
|  |  | UE example 2 | -26.0 | -30.1 | -27.8 | -26.0 | 4.1 | 1.8 | 0.0 |
|  |  | From Qualcomm | -34.8 | -34.9 | -34.6 | -34.1 | 0.1 | -0.2 | -0.7 |
|  |  | From MTK | -35.4 | -34.7 | -34.5 | -34.3 | -0.7 | -0.9 | -1.1 |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -34.7217 | -34.3119 |  |  | -0.4098 |  |  |
|  |  | From MTK | -36.4809 | -35.7798 |  |  | -0.7011 |  |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3038 | -32.9937 | -33.946 | -32.8345 | -0.3101 | 0.6422 | -0.4693 |
|  |  | From MTK | -34.5513 | -34.0163 | -34.7963 | -34.022 | -0.535 | 0.245 | -0.5293 |
|  | 120kHz, 16RBs | From Qualcomm | -35.5055 | -33.3672 |  |  | -2.1383 |  |  |
|  |  | From MTK | -34.1009 | -32.5789 |  |  | -1.522 |  |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.6484 | -30.7554 |  |  | -1.893 |  |  |
|  |  | From MTK | -33.4813 | -31.4505 |  |  | -2.0308 |  |  |
|  | 120kHz, 64RBs | From Qualcomm | -32.8215 | -29.8556 |  |  | -2.9659 |  |  |
|  |  | From MTK | -33.2358 | -30.0408 |  |  | -3.195 |  |  |

Issue 2-2-1: PTRS configuration for EVM test and MPR requirement for CP-OFDM

- Proposals

  - Option 1: Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration for CP-OFDM. (Ericsson, vivo, Xiaomi, Qualcomm )

- Recommended WF

  - TBA

Issue 2-2-2: PTRS configuration for EVM test and MPR requirement for DFT-S-OFDM

- Proposals

  - Option 1: PTRS is not configured for all RBs allocation.(vivo, Xiaomi)

  - Option 2: The following PTRS configuration is established for UEs that signal they need PTRS to meet Tx signal quality requirements:

    - Adopt 4 samples/group and 8 groups/OFDM symbol for DFT-s-OFDM for narrow allocations (20 RBs or narrower).

    - PTRS is not configured for DFT-s-OFDM for allocations wider than 20 RBs.

    - RAN4 to agree on PTRS configuration strategy for very narrow allocations (<= 8RB) to ensure overhead is kept in check.

- Recommended WF

  - TBA

Issue 2-2-3: Whether the PTRS is configured mandatorily for UL EVM and MPR for FR2-1 UL 256QAM?

- Proposals

  - Option 1: Yes.

  - Option 2: No, and don’t need new capability.

  - Option 3: No, and one capability signaling need be introduced to convey to the network if the UE needs PTRS assistance to meet Tx signal quality requirements for FR2-1 UL 256QAM.

    - Whether needed PTRS assistance or not can be justified by UL throughput gains.

    - A fixed PTRS configuration is adopted for each waveform type (CP- or DFT-s).

- Recommended WF

  - TBA

Topic #3: TP

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307231 | Nokia, Nokia Shanghai Bell | TP provides a text proposal to clarify the recorded results and record some additional results with the agreed target SNR at BS side of 30 dB at 39 GHz in TR |
| R4-2308804 | Xiaomi | TP is to capture the simulation assumptions for phase noise profiles evaluation and MPR simulation results from different companies into TR 38.891 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 TP

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1-1: Approved TP in R4-2307231

- Proposals

  - Option 1: Yes

  - Option 2: Modification is needed

- Recommended WF

  - TBA

Issue 3-1-2: Approved TP in R4-2308804

- Proposals

  - Option 1: Yes

  - Option 2: Modification is needed

- Recommended WF

  - TBA