3GPP TSG-RAN WG4 Meeting # 107												R4-2310043

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.32.3

Source:	Moderator (Ericsson)

Title:	WF on R18 RedCap RRM requirements

Document for:	Approval

### Defining of requirements for eDRX cycle > 10.24 sec in FR1 and FR2

How to specify the requirements when configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements

The requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304.

Measurement period when configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements

  - The detection/measurement/evaluation delay requirement are specified in the step of:

    - RAN configured DRX cycle, if eDRX_Inactive  20.48sec

When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements

  - UE shall perform measurements/evaluation of neighbor cell within single PTW irrespective of IDLE or INACTIVE PTW being used.

When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s and when the PTWs are partially overlapping for serving and neighbour cell measurements

  - UE performs measurements during:

    - Option 1a: Max (RAN PTW, CN PTW)

      - Prioritize measurement time during the PTW at the cost of increase UE complexity.

    - Option 1b: Min (RAN PTW, CN PTW)

      - Prioritize UE complexity at the cost of reduced measurement time.

When to measure when IDLE and INACTIVE eDRX PTW do not coincide

  - Option 1: RAN4 to agree that UE follows the INACTIVE eDRX when IDLE and INACTIVE eDRX PTWs do not coincide

  - Option 2: No need to define requirements for the case when PTWs do not coinside.

Transition requirements

  - Option 1: RAN4 to define requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec).

  - Option 2: No need to define new transition requirements for transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec).

Issue 1-8: CG-SDT requirements with PTW

  - Do not define specific CG-SDT requirements with PTW

  - Reuse the existing CG-SDT requirements

  - It is RAN4 understanding that UE is allowed to make CG-SDT transmission either inside or outside PTW based on CG-SDT network configuration

### Baseband BW reduction impact

Whether to define CGI reading requirements for Rel-18 RedCap UE

No CGI reading related requirements are specified for R18 RedCap when SIB1 is scheduled with BW > 5 MHz.