3GPP TSG-RAN WG4 Meeting # 107														R4-2310083

Incheon, KR, May 22nd – May 26th , 2023

Title:	LS on FR2 SCell activation enhancements

Response to:

Release:	Rel-18

Work Item:	NR_RRM_enh3

Source:	RAN WG 4

To:	RAN WG 2

Cc:	RAN WG 1

Contact person:

Name:	Jie Cui

E-mail Address:	jie_cui@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

In RAN4#107 meeting, RAN4 had some more conclusions on unknown FR2 SCell activation enhancement, including both network triggering reporting during FR2 unknown SCell activation and beam sweeping factor reduction for FR2 unknown SCell activation.

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

- Agreement: - Descope the L3 reporting based FR2 unknown SCell activation enhancement for direct SCell activation in R18. - The enhancement under discussion except the L3 reporting based enhancement can also apply to direct SCell activation. - The enhancement under discussion can also apply to other single PUCCH SCell activation - Agreement: - Descope the SCell activation enhancement in FR2-2 in R18. - Agreement: - Do not consider additional solution for report L3 measurement results for unknown FR2 SCell activation enhancement in R18 eFeRRM WI. - Agreement: - The report of L3 measurement result after SCell activation command needs to be valid. - The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) and accuracy requirements in TS 38.133 Clause 10. - Agreement: - UE does not need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

Enhancement 2: Beam sweeping factor reduction in FR2 unknown SCell activation

Agreement in RAN4#106-bis: - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Note: above enhancement only applies for FR2 unknown SCell activation enhancement - Note: How to capture in spec can be discussed in CR stage - The candidate values for X1/X2 are FFS Agreement in RAN4#107: - Candidates of X1 are {1,2,4,6} - Candidates of X2 are {0,1,2,3,4,5,6,7} - If X1 is absent, beam sweeping factor for cell detection part is 8 - If X2 is absent, beam sweeping factor for SSB-based L1 measurement is 8

# Actions

To RAN WG2 group.

ACTION: 	RAN4 sincerely asks RAN2 to take into account the above conclusions for their work and design the corresponding signaling to support the above enhancements solutions.

# Date of Next TSG-RAN WG4 Meetings

TSG-RAN4 Meeting #108	August 21 – August 25	 Toulouse, France

TSG-RAN4 Meeting #108bis	October 9 – October 13	 Xiamen, China