3GPP TSG-RAN WG4 Meeting # 107		R4-2310099

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			9.8.5

Source:	Moderator (MediaTek inc.)

Title:	WF on IoT NTN enhancements RRM requirements

Document for:	Information

# Introduction

This document is to capture all agreements in email thread [107][237] IoT_NTN_enh.

Online Agreements

# Topic #1: RRM core requirements (AI 9.8.4)

#### Issue 1-1: Measurement capabilities on number of NGSO satellites

- Agreements

  - Clarify that the sets of neighbor satellites for inter-frequency measurements that the UE shall be capable to measure in each frequency layer in NGSO scenarios are not necessary the same

#### Issue 1-2: Scaling factor for multiple NGSO satellites

- Agreements

  - Introduce scaling factor for multiple NGSO satellites for NB intra-frequency neighbour cell measurement in connected mode. FFS for inter-frequency measurements.

#### Issue 2-2: Requirements appliacability for time-based neighbour cell measurements

- Agreements

  - UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, and the relaxed neighbour cell measurement is only allowed when the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled and the time span to before t-Servic is longer than Ttrigger

#### Issue 3: Location-based cell reselection

- For location triggered cell reselection measurement,

  - For eMTC and fixed cell, re-use the requirements from NR NTN as a starting point

  - For eMTC and moving cell, wait for further progress in Rel-18 NR NTN WI

  - FFS whether the requirements are also applicable to NB-IoT pending on further RAN2 agreement

#### Issue 4-1: Neighbour cell measurements in connected mode, time-based

Continue discussion on the following proposals:

- Proposal 1: For for t-service triggered neighbor cell measurement before RLF for NB-IoT:

  - Option 1a: Do not define requirements for t-service triggered neighbour cell measurement. (Huawei)

  - Option 1b: The UE shall be able to measure only intra-frequency neighbour cell before t-service provided that the time span from the SI broadcasting t-service to t-service is longer than Tidentify_intra, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation.

- Proposal 2a: Support neighbor cell measurements initiated before t-service before RLF and wait for RAN2 definition on the indication of the “start time” of neighbor satellite to decide the point in time where the measurements are initiated.

- Proposal 2b: For NB-IOT NTN of quasi-earth fixed cell and earth-moving cell, RAN4 should introduce time-based connected mode measurement initiation in core spec according to RAN2’s design. The exact time to start measurements in connected mode before t-Service can be left to UE implementation.

- Proposal 3: The NB-IoT UE shall start the intra/inter-frequency measurements at least before time T1 before start of t-Service, where T1 is the time required to perform one measurement.

#### Issue 4-2: Neighbour cell measurements in connected mode, location-based

Continue discussion on the following proposals:

- Proposal 1: For location-based triggering, add the corresponding criteria (reference location and distance threshold) and the same requirements as NRSRP based triggering can apply, and the details should wait for more progress in RAN2. (Huawei)

- Proposal 2: For eMTC NTN of quasi-earth fixed cell and earth-moving cell, RAN4 should introduce location-based IDLE mode measurement initiation condition in core spec according to RAN2’s design. The measurement initiation mechanism for NR-NTN could be reused. (CMCC)

- Proposal 3: For earth-moving cell, RAN4 to discuss how to define the requirements for the propagation of the referenceLocationh based on the satellite movement. (Nokia)

Recommended WF: Discuss proposals.

#### Issue 5: For eMTC, CHO requirements

No agreement

#### Issue 6-1: GNSS re-acquisition, impact on RLM

- Agreements

  - Capture in specification that when a UE is measuring the GNSS in a GNSS-MG RLM monitoring is suspended. FFS on the impact on RLM requirements.