3GPP TSG-RAN WG4 Meeting #107	R4-2310140

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF on R17 NR MIMO RRM requirements maintenance

Agenda Item:	5.4

Source: 	Samsung

Document for:	Approval

- Topic #1 Core part maintenance

Issue 1-1-1: Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list

Agreement

Add applicability in spec for active uplink TCI state switching delay for unified TCI:

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.

Issue 1-1-2 Definition of maintained PL-RS

Agreement:

PL-RS is maintained provided:

-	the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS activated as PL-RS per serving cell among all active UL TCI states (UL or joint TCI state) for PUSCH/PUCCH/SRS transmissions

-    The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period.

-	SNR of the associated SSB ≥-3dB

Issue 1-1-3 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Agreements

  - No requirements are defined for the case when SSB is indicated as PL-RS in UL TCI state in FR2 and PL-RS is not maintained

Issue 1-2-1 Whether need clarification or update in RAN4 spec when PDCCH/PDSCH and SSB associated different PCI are overlapped on the same RE

- Proposals:

  - Proposal 1: (vivo)

    - Introduce scheduling restrictions for the cases when UE simultaneously receive SSB and PDSCH/PDCCH, while SSB is associated to a PCI different from the PCI to which the active TCI of PDSCH/PDCCH is associated. RRM requirements do not apply for these cases.

  - Proposal 3: (Ericsson)

    - Do not agree to introduce scheduling restriction “if the TCI state of the PDSCH /PDCCH is associated to the SSB of the cell with different PCI, UE is not expected to receive PDSCH/PDCCH on the symbols corresponding to the SSB indexes configured for L1-RSRP measurement on the serving cell”