3GPP TSG-RAN WG4 Meeting # 107												R4-2310167

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF on NR RRM enhancements – FR1-FR1 NR DC

Agenda Item:	8.9.4

Source: 	OPPO

Document for:	Approval

Topic #1: SCG activation/deactivation requirements for FR1-FR1 NR-DC

Issue 1-1-1: UE behaviour when UE detected either BFD or RLF on the deactivated PSCell for RACH-less SCG activation

- Agreement:

No requirement shall be defined for this case when UE detected either BFD or RLF on the deactivated PSCell during RACH-less SCG activation.

Issue 1-1-2：UE report on the beam failure if Beam failure has been declared or TCI become unknown during SCG activation procedure

- Agreement in AdHoc (May 23rd):

  - FFS

    - Option 1: RAN4 assumes whether to report beam failure to the network can be up to UE implementations if beam failure has been detected during SCG activation procedure.

    - Option 2: RAN4 assumes UE should not indicate beam failure to the network if beam failure has been detected during SCG activation procedure.

  - FFS whether LS is needed to check with RAN2 on the indication for above case.

- Agreement in RRM session (May 24th):

  - Update ad-hoc agreement as follows:

    - Do not specify UE behavior in RAN4 specifications

Issue 1-1-3：Tsearch for RACH-less PSCell activation

<Way forward>

Continue discussion on whether to update the side conditions for Tsearch.

  - known target cell

  - known TCI state

  - UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD

Issue 1-1-4：Tsearch for RACH-based case

Proposal

- For RACH based PSCell activation, if the target cell is an unknown FR2 PSCell configured with BFD-and-RLM with value true and no RLM has occurred, then Tsearch = [12]* Trs ms, if Es/Iot ≥ -2 dB, otherwise Tsearch = 24* Trs ms

<Way forward>

  - Continue discussion