3GPP TSG-RAN WG4 Meeting #107	R4- 2310243

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon, Apple

Title:	TP for TR 38.896 on the A-MPR requirements for band n28

Agenda Item:	7.22.2

Document for:	Approval

# Background

According to the WID on R18 HPUE FDD [1], PC2 A-MPR for band n28 in [2][3][4]. Based on these contributions, the PC2 A-MPR requirements for NS_18 are proposed. Together with the simulation results, it’s proposed to capture the contributions into the TR 38.896 as shown in the text below.

# References

- RP-230352 Revised WID High power UE (power class 2) for NR FR1 FDD single band, China Unicom, RAN#99

- R4-2300717 PC2 A-MPR for band n28, Huawei, HiSilicon, RAN4#106

- R4-2304519 PC2 A-MPR for band n28 NS_18, Huawei, HiSilicon, RAN4#106bis-e

- R4-2309060 On PC2 A-MPR Proposal for NS_18, Apple, RAN4#107

# Text Proposal

##### ---Start of changes---

## 5.x	NR band n28

### 5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n28 | 26 | [FFS] |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

### 5.x.2	A-MPR requirements

### 5.x.2.1 NS_18

### 5.x.2.1.1 A-MPR simulation results from Huawei

The simulation results are obtained for NS_18 for both PC2 1Tx and PC3 1Tx, which are shown in Table 5.x.2.1.1-1 below. The figures in the 2nd column show the differences in the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR. It can be seen that similar A-MPR value distributions are shown for BW=15/20/25/30MHz.

Table 5.x.2.1.1-1: A-MPR simulation results for NS_18 with 1Tx, SCS=15kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 5MHz |  |  |
| 10MHz |  |  |
| 15MHz |  |  |
| 20MHz |  |  |
| 25MHz |  |  |
| 30MHz |  |  |

### 5.x.2.1.2 A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 5MHz, 10MHz, 15MHz, 20MHz, 25MHz and 30MHz. The A-MPR regions proposed in [3] were used as starting point. Selected simulation results can be found in Table 5.x.2.1.2-1 where allocations for DFT-s-OFDM and CP-OFDM for QPSK are provided.

Table 5.x.2.1.2-1: A-MPR simulation results for NS_43

### 5.x.2.1.3 A-MPR requirements

Based on the simulation results from previous sections, the PC2 A-MPR requirements for NS_18 are proposed below.

The existing A-MPR regions for PC3 as defined in TS 38.101-1 are adapted for PC2, which are shown as in the tables below. The new tables are applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 for PC3; Table 6.2.3.13-2, A1 for PC2 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 for PC3; Table 6.2.3.13-2, A2 for PC2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 for PC3; Table 6.2.3.13-2, A3, A4, A5, A6 for PC2 |

Table 6.2.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

The A-MPR values for 1Tx PC2 are proposed in the table below.

Table 6.2.3.13-2: A-MPR for NS_18 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [3.5] | N/A | ≤ [8] | [3.5] | [10] | [4.5] | [2] |
|  | QPSK | ≤ [3.5] |  | ≤ [8] | [3.5] | [10] | [4.5] | [2] |
|  | 16 QAM | ≤ [4] |  | ≤ [9] | [3.5] | [11] | [5] | [2] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [10] | [3.5] | [11] | [5.5] | [2.5] |
|  | 256 QAM | ≤ [6.5] |  | ≤ [11] | [3.5] | [11] | [5.5] |  |
| CP-OFDM | QPSK | ≤ [5.5] |  | ≤ [9.5] | [5] | [11.5] | [6] | [4] |
|  | 16 QAM | ≤ [5.5] |  | ≤ [10] | [5] | [11.5] | [6] | [4] |
|  | 64 QAM | ≤ [6] |  | ≤ [11.5] | [5] | [11.5] | [6] | [4] |
|  | 256 QAM | ≤ [9] |  | ≤ [11.5] | [5] | [11.5] | [7.5] |  |

Note that BW=25MHz is missing for NS_18 in Table 6.5.3.3-1 of TS38.101-1 and needs to be added.

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30 |  |
| 692-698 | -26.2 | 6 MHz |

### 5.x.2.2 NS_17

FFS

##### ---End of changes---